3GPP TSG-SA5 Meeting #145-e 	S5-225231

# e-meeting, 15 - 24th August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0376 | rev |  | Current version: | 17.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update measurement for EES |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eECM |  |  |  |  |  | Date: |  |  | 2022-8-03 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As indicated in A.100 in TS 28.552, EAS discovery request related measurement is important for EES preformance, “The discovery procedure may fail due to the unavailability of the relavant EAS. If the failure rate increases beyond a defined threshold, corrective actions (e.g instantiating the required EAS) can be taken by the OAM systems.” According to TS 23.558, the EAS discorvery request may fail due to multiple reasons, e.g., EEC is not registered with EES, UE location is outside the Geographical or Topological Service Area of an EAS. So it would be useful to collect the measurement for failed discovery request per reject casue. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add new measurement for EES related to number of failed discovery request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurement for EES performance assurance is not completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.1 and A.100 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.15.1	EAS Discovery procedure related measurements

#### 5.15.1.1	Number of discovery requests

a)	This measurement provides the number of EAS discovery requests (see clause 8.5.2 of TS 23.558 [52]) received by the EES.

b)	CC

c)	On receipt by the EES from the EEC of EAS Discovery Request. Each discovery request is added.

d)	A single integer value

e)	DIS.EasDisReq

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

#### 5.15.1.2	Number of successful discovery request

a)	This measurement provides the number of successful EAS discovery request at the EES.

b)	CC

c)	On transmission of EAS Discovery Response (see clause 8.5.2 of TS 23.558 [52]) by the EES to the EEC that sent the registration request. Each accepted request is added.

d)	A single integer value

e)	DIS.EasDisSucc

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

5.15.1.3	Number of failed discovery request

a)	This measurement provides the number of failed EAS discovery request to be responsed by the EES.

b)	CC

c)	On transmission of EAS Discovery Response (see clause 8.5.2 of TS 23.558 [52]) by the EES to the EEC that indicates a rejected registration request. Each rejected EAS discovery request is added to the relevant subcounter per rejection cause.

d)	Each subcounter is an integer value

e)	DIS.EasDisFail.cause

Where cause indicates the cause of EAS discovery request failure

f)	EESFunction

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this performance measurements is for EES performance assurance.

2nd Change

# A.100	Use case of measurements for EES.

EES related measurements are used to measure the performance of an EES on each of the supported functionality.

Discovery procedures enable entities in an edge deployment to obtain information about EAS and their available services, based on specified criteria of interest. It is useful to analyse the EAS discovery success rate in order to assess EES performance. The discovery procedure may fail due to the unavailability of the relavant EAS. If the failure rate increases beyond a defined threshold, corrective actions (e.g instantiating the required EAS) can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EAS discovery procedures.

An EEC performs registration with an EES in order to provide information that can be used by the EES in Edge Computing services. It is useful to analyse the EEC registration success rate in order to assess EES performance. If the failure rate increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EEC Registration procedures.

The EAS Registration procedure allows an EAS to provide its information to an EES in order to enable its discovery. It is useful to analyse the EAS registration success rate in order to assess EES performance. If the failure rate increases beyond a defined threshold, corrective actions can be taken by the OAM systems. Hence, it is necessary to collect measurement related with EAS Registration procedures.

End of Change