3GPP TSG-SA5 Meeting #145-e 	S5-225296

# e-meeting, 15 - 24 August 2022

# Source:	Ericsson

# Title:	Adding solution for size of charging information

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.826.

# 2	References

[1]		3GPP TR 28.826: " Study on Nchf charging services phase 2 improvements and optimizations"

# 3	Rationale

Adding solution for size of charging information using CHF controlled request size.

# 4	Detailed proposal

First change

#### 5.5.5.x	Solution #1.a: CHF control of request size

A possible solution for key issues 1d covering requirements REQ-CH_INFO-02, control of charging request size.

The CHF can today set the following limits for a request to trigger reporting of charging information:

-	time: limits the duration the CTF can wait before reporting.

-	volume: limits the volume measured by the CTF before reporting.

-	event: limits the number of events measured by the CTF before reporting.

-	Number Of CCC: limits the number of charging condition changes performed CTF before reporting.

This solution would add two new limits the size of payload (before and after any compression) of the Charging data request:

-	payload: limits the uncompressed (can be size before compression if compression is used) size of the message that CTF can use for reporting.

-	gzip payload: limits the size of the message after gzip compression that CTF can use for reporting.

Example of how the definition of trigger could look in the TS 32.291 [6] table 6.1.6.2.1.7-1:

Table 5.5.5.x-1: Definition of type Trigger

| Attribute name |  | Data type |  | P |  | Cardinality |  | Description |  | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| triggerType |  | TriggerType |  | Oc |  | 0..1 |  | the events whose occurrence lead to charging event is issued towards the CHF |  |  |  |
| triggerCategory |  | TriggerCategory |  | M |  | 1 |  | This field indicates whether the charging data generated by the NF consumer for the trigger lead to a Charging Event towards the CHF immediately or not. |  |  |  |
| timeLimit |  | DurationSec |  | OC |  | 0..1 |  | Time limit if trigger type is "Expiry of data time limit" |  |  |  |
| volumeLimit |  | Uint32 |  | OC |  | 0..1 |  | Volume limit if trigger type is "Expiry of data volume limit". This attribute is not valid from Nchf_ ConvergedCharging API version v2.0.0 |  |  |  |
| volumeLimit64 |  | Uint64 |  | OC |  | 0..1 |  | Volume limit if trigger type is "Expiry of data volume limit". This attribute replaces the volumeLimit attribute from Nchf_ ConvergedCharging API v2.0.0 |  |  |  |
| eventLimit |  | Uint32 |  | OC |  | 0..1 |  | Event limit if trigger type is "Expiry of data event limit". |  |  |  |
| maxNumberOfccc |  | Uint32 |  | OC |  | 0..1 |  | Maximum number if trigger type is "Max nb of number of charging condition changes" |  |  |  |
| maxPayload |  | Uint64 |  | OC |  | 0..1 |  | Maximum payload size without any compression, if trigger type is "Max payload”. |  |  |  |
| maxGzipPayload |  | Uint64 |  | OC |  | 0..1 |  | Maximum payload after gzip compression, if trigger type is "Max gzip payload”. |  |  |  |
| tariffTimeChange |  | DateTime |  | OC |  | 0..1 |  | This field contains UTC time indicating the switch time when the tariff will be changed. |  |  |  |

End of changes