3GPP TSG-SA5 Meeting #145-e 	S5-225389

# e-meeting, 15 - 24 August 2022

# Source: 	Huawei, Deutsche Telekom

# Title:	pCR TR 28.908 Add AIML capabilities deployment scenarios

# Document for:	Approval

Agenda Item:	6.7.5.1

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

References

[1]		SP-211443 New Study on AI/ ML management

[2] 		3GPP TR 37.817 Study on enhancement for Data Collection for NR and EN-DC.

[3] 		3GPP TR 28.813 Study on new aspects of Energy Efficiency (EE) for 5G.

[4] 		3GPP TR 28.908 Study on Artificial Intelligence / Machine Learning (AI/ML) management

3	Rationale

The approved new SI [1] proposed to study the AI/ML management capabilities and management services to support/coordinate AI/ML in 5GS (3GPP management system, 5GC and NG-RAN). The AI/ML management workflow will helpful to illustrate the AI/ML management related capabilites and potential relationship.

In addition, one of the objective in SI [1] is investigation of deployment scenarios where the solutions are needed for AI/ML model training and each of the AI/ML model management capability. As discussed in RAN TR 38.817[2], OAM may need to be involved to support AI/ML capability in RAN. And as discussed in TR 28.813 [3], to provide centralized ES for RAN domain area, 3GPP management system need AI technology, so as to provide more efficient ES for RAN domain area while keeping basic KPIs stable for SLA assurance. Therefore, this contribution proposes to add AI/ML capabilites deployment scenarios for AI-ML management.

This contribution proposes to add AI/ML capabilites deployment scenarios.

4	Detailed proposal

It is proposed to add the following chapter in TR 28.908 [2].

1st modified section

4	Concepts and overview

4.X	AI/ML deployment overview

AI/ML related capability can be deployed at domain level or cross domain level. A domain AI/ML provides domain specific training and inference, e.g. AI/ML training capability and AI/MLinference capability in a RAN management domain, CN management domain. A cross domain AI/ML can provide AI/ML management capability, e.g. to control training and monitoring the performance in 3GPP cross management domain.

4.Y	AI/ML deployment scenarios with management

This clause provides the deployment scenarios for AI/ML management, which addresses the AI/ML related capabilities including AI/ML training capability, AI/ML inference capability and AI/ML management capability.

Deployment scenario1: In this deployment scenario which provides intelligence in the 3GPP domain management layer, the AI/ML Management capability is located in 3GPP cross domain layer to provide management capability. The AI/ML domain training capability and AI/ML domain inference capability are located in 3GPP domain layer.

Figure X.Z-2: Example Deployment scenario1

Deployment scenario2: In this deployment scenario which supports the intelligence in RAN, the AI/ML Management capability is located in 3GPP cross domain layer to provide management capability. The AI/ML domain training capability is located in 3GPP domain layer to provide the AI/ML training capability to network function layer. The AI/ML RAN inference capability is located in Network function layer to provide inference capability.

Figure X.Z-3: Example Deployment scenario2

Notes: The AI/ML RAN inference Capability is not within the standard discussion scope of SA5, the split between the training capability and inference capability within the domain is up to the implementer.

End of modified section