3GPP TSG-SA5 Meeting #145-e 	S5- 225412

# e-meeting, 15 - 24 August 2022

# Source:	China Unicom

# Title:	Add potential solution for PLMN-related attribute requirement for operator specific IOC

# Document for:	Approval

Agenda Item:	6.8.6

1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

2	References

[1]	3GPP TR 28.835 v0.3.0: “Management Aspects of 5G MOCN Network Sharing Phase2”

3	Rationale

It was approved to add potential solution for PLMN-related attribute requirement for opetrator specific IOC. This contribution proposes to add pLMNInfoList attribute for some operator-related IOCs.

4	Detailed proposal

This contribution proposes to make the following changes in [1].

1st Change

5.X	Issue #X:  PLMN-related attribute requirement for operator specific IOC

5.X.2	Potential solution

1. The OperatorDU IOC defined in TS 28.541[2] shall add PLMN-related attributes.

The pLMNInfoList attribute shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | M | T | T | F | T |

- The GNBDUFunction IOC defined in TS 28.541[2] shall add PLMN-related attributes.

  - The pLMNInfoList attribute shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | CM | T | T | F | T |

  - The attribute constraint of pLMNInfoList shall be added as follows:

| Name | Definition |
| --- | --- |
| pLMNInfoList S | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature is supported. |

- The Configurable5QISet IOC defined in TS 28.541[2] shall create and configure one instance for each POP to support different POP’s pre-configured 5QIs in MOCN networking sharing scenario.

- TheConfigurable5QISet IOC shall add PLMN-related attributes.

  - The pLMNInfoList attribute shall be added as follows:

| Attribute name | S | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| pLMNInfoList | CM | T | T | F | T |

  - The attribute constraint of pLMNInfoList shall be added as follows:

| Name | Definition |
| --- | --- |
| pLMNInfoList S | Condition: The NG-RAN Multi-Operator Core Network (NG-RAN MOCN) network sharing with multiple Cell Identity broadcast feature is supported. |

End of Change