3GPP TSG-SA5 Meeting #144-e	S5-225469

# e-meeting, 15th Aug 2022 - 24th Aug 2022

# Source:	Nokia, Nokia Shanghai Bell

# Title:	pCR TR 28.831 Add key issue Definition of getMOIAttributes

# Document for:	Approval

Agenda Item:		6.8.2.3 - FS_eSBMAe_WoP#3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]		3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

# 3	Rationale

The definition of createMOI misses many important aspects and needs to be improved.

# 4	Detailed proposal

The following changes are proposed for TR 28.831 [1].

Begin of modifications

## 4.X	Key issue #X: Definition of getMOIAttributes

### 4.X.1	Issue description

The operation "getMOIAttributes" in TS 28.532 is underspecified in many aspects and needs to be improved.

### 4.X.2	Analysis

The following issues have been identified with the current definition of "getMOIAttributes".

- The "scope" parameter is mandatory. This does not allow for simple deployments where it is sufficient to retrieve the attributes of only one object instance with a request.

- Handling of structured attributes is not specified. For example, it is not clear if only complete attributes can be retrieved or also attribute fields and attribute elements.

- The "filter" works only on a set of managed objects that was previously scoped with the "scope" parameter. According to the current definition it is not possible to use the "filter" only for selecting managed objects whose attributes shall be retrieved.

- The current definition of the response gives the impression that the response is always a list of attribute name/value pairs. This is not the case though. In current solution sets it is always a list of complete attribute representations in flat or hierarchical format.

End of modifications