3GPP TSG-SA5 Meeting #145-e 	S5-225480

# e-meeting, 15 - 24 August 2022

# Source:	Huawei

# Title:	pCR 28.865 Add solution of service deployment related to DCSA

# Document for:	Approval

Agenda Item:	6.9.5.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	 SP-211442: "New SID on deterministic communication service assurance"

[2]	S5-224421: "TR 28.865 Study on deterministic communication service assurance"; v0.3.0

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3"; v18.0.0

3	Rationale

This tdoc addresses the solution of service deployment related to DCSA. In [3], some management aspects have been specified related to deterministic communication service.

3.1	Service requirements and QoS attributes related to DCSA

3.1.1	Service requirements related to deterministic communication

In clause “6.3.3 ServiceProfile <<dataType>>” and “6.3.4 SliceProfile <<dataType>>” in [3], there are some network slice related service requirement attributes which are related or specific to (highlighted) deterministic communications. The following attributes are extracted from ServiceProfile for example.

| Attribute name | S |
| --- | --- |
| dLLatency | O |
| uLLatency | O |
| availability | O |
| delayTolerance | O |
| dLDeterministicComm | O |
| uLDeterministicComm | O |
| dLThptPerSlice | O |
| dLThptPerUE | O |
| uLThptPerSlice | O |
| uLThptPerUE | O |
| dLMaxPktSize | O |
| uLMaxPktSize | O |
| maxNumberofPDUSessions | O |
| termDensity | O |
| jitter | O |
| survivalTime | O |
| radioSpectrum | O |
| reliability | O |
| maxDLDataVolume | O |
| maxULDataVolume | O |
| synchronicity | O |
| positioning | O |

The survivalTime attribute specifies the time (millisecond) that an application consuming a communication service may continue without an anticipated message.

The synchronicity attribute specifies whether synchronicity of communication devices is supported, it is important for deterministic communication service.

The dLDeterministicComm attribute and uLDeterministicComm attribute specifies the properties of the deterministic communication in downlink and uplink respectively for periodic user traffic. Whose attributes are available in clause “6.3.8 DeterminComm <<dataType>>” in [3], as listed in the table below.

| Attribute name | S |
| --- | --- |
| servAttrCom | CM |
| availability | M |
| periodicityList | M |

The servAttrCom attribute is mandatory only when requirements are being defined on deterministic communication for periodic user traffic per network slice (GSMA attribute). Otherwise, the attribute is optional.

The periodicityList attribute specifies a list of periodicities supported by the network slice for deterministic communication.

The “6.3.9 XLThpt<<dataType>>” defines the throughput properties for attributes dLThptPerSlice, dLThptPerUE, uLThptPerSlice, uLThptPerUE, in which the attributes guaThpt and maxThpt (highlighted) are related to GFBR QoS flow type for deterministic communications.

| Attribute name | S |
| --- | --- |
| servAttrCom | CM |
| guaThpt | O |
| maxThpt | O |

The guaThpt attribute describes the guaranteed data rate.

The maxThpt attribute describes the maximum data rate.

Some of the above attributes above are also available in “6.3.4 SliceProfile <<dataType>>”, which contains CNSliceSubnetProfile, RANSliceSubnetProfile, TopSliceSubnetProfile.

3.1.2	QoS attributes related to deterministic communication

In clause “5.3.76	FiveQICharacteristics” in [3], some attributes related or specific to (highlighted) deterministic communication service are defined.

| Attribute name | S |
| --- | --- |
| fiveQIValue | M |
| resourceType | M |
| priorityLevel | O |
| packetDelayBudget | O |
| packetErrorRate | O |
| averagingWindow | O |
| maximumDataBurstVolume | O |

The averagingWindow attribute indicates the Averaging Window (in unit of ms) of a 5QI. The maximumDataBurstVolume attribute indicates the Maximum Data Burst Volume (in unit of Byte) of a 5QI. These two parameters are used for deterministic communication service.

In clause “5.3.84	QoSData <<dataType>>” in [3], some attributes for GBR QoS flow type closely related to (highlighted) determinintic communications are extracted as follows:

Attribute name

qosId

fiveQIValue

maxbrUl

maxbrDl

gbrUl

gbrDl

maxPacketLossRateDl

maxPacketLossRateUl

extMaxDataBurstVol

The gbrUl attribute and gbrDl attribute represents the guaranteed uplink and downlink bandwidth respectively. The extMaxDataBurstVol attribute denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB.

In clause “5.3.81	PccRule <<dataType>>” in [3], some properties specific to GFBR QoS flow type (highlighted) for determinintic communications are extracted as follows:

| Attribute name | S |
| --- | --- |
| pccRuleId | M |
| qosData | M |
| tscaiInputUl | O |
| tscaiInputDl | O |

The tscaiInputDl attribute contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction).

The tscaiInputUl attribute contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction).

TSCAI describes TSC traffic characteristics for use in the 5G System. TSCAI may be used by the 5G-AN, if provided by SMF. The knowledge of TSC traffic pattern is useful for 5G-AN as it allows more efficiently scheduling of QoS Flows that have a periodic, deterministic traffic characteristics either via Configured Grants, Semi-Persistent Scheduling or with Dynamic Grants.

In clause “5.3.93	TscaiInputContainer <<dataType>> in [3], this data type is defined.

| Attribute name | S |
| --- | --- |
| periodicity | O |
| burstArrivalTime | O |

The Periodicity attribute identifies the time period between the start of two bursts in reference to the TSN GM. The burstArrivalTime attribute Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM.

3.2	The role of DCSA MnS producer in service deployment

There are mainly two aspects for the above contents in existing specification, the service requirements and QoS attributes related to DCSA.

For the service deployment of deterministic communications, the generic procedure of network slice provisioning could be utilized, including decomposition of ServiceProfile in the cross domain management into SliceProfile which contains CNSliceSubnetProfile, RANSliceSubnetProfile, TopSliceSubnetProfile in different domain management, feasibility check etc. It is considered the configuration of network resources, QoS policy for the support of deterministic communications etc are the responisibility of the domain management.

DCSA MnS producer in the cross domain management can provide support of SLS requirement analysis, SLS decomposition and feasibility check for ServiceProfile processing. It is handled by the module “service requirement modelling” in DCSA MnS producer. The inputs are the attributes in the ServiceProfile. The outputs are the attributes in the SliceProfile.

DCSA MnS producer in the domain management can provide support of network slice subnet requirement analysis, feasibility check in the network slice subnet level, network resource, network optimization policy e.g. SON policy and QoS policy configuration processing etc. It is handled by the module “Network preparation” and module “Service and Network analysis” in DCSA MnS producer.

It is proposed to add service requirements, QoS attributes and role of DCSA MnS producer in service and network requirements processing for deterministic communication service from management perspective. An example of procedure is also provided.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

Editor's note: this clause will contain the issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related issues.

5.X.2	Potential solutions

5.X.2.a	Potential solution #x:service deployment

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for issue#1 at a high-level.

For service deployment of deterministic communication in a certain network slice, the modelling and analysing of SLS requirements, feasibility check for network slice and network slice subnet, network resource management policy and QoS policy configuration specific to deterministic communications etc should be supported.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for issue#1.

1.	Service requirements related to deterministic communication

In clause “6.3.3 ServiceProfile <<dataType>>” and “6.3.4 SliceProfile <<dataType>>” in [3], there are some network slice related service requirement attributes which are related or specific to deterministic communications. The following attributes are extracted from ServiceProfile for example.

| Attribute name | S |
| --- | --- |
| dLLatency | O |
| uLLatency | O |
| availability | O |
| delayTolerance | O |
| dLDeterministicComm | O |
| uLDeterministicComm | O |
| dLThptPerSlice | O |
| dLThptPerUE | O |
| uLThptPerSlice | O |
| uLThptPerUE | O |
| dLMaxPktSize | O |
| uLMaxPktSize | O |
| maxNumberofPDUSessions | O |
| termDensity | O |
| jitter | O |
| survivalTime | O |
| radioSpectrum | O |
| reliability | O |
| maxDLDataVolume | O |
| maxULDataVolume | O |
| synchronicity | O |
| positioning | O |

The survivalTime attribute specifies the time (millisecond) that an application consuming a communication service may continue without an anticipated message.

The synchronicity attribute specifies whether synchronicity of communication devices is supported, it is important for deterministic communication service.

The dLDeterministicComm attribute and uLDeterministicComm attribute specifies the properties of the deterministic communication in downlink and uplink respectively for periodic user traffic. Whose attributes are available in clause “6.3.8 DeterminComm <<dataType>>” in [3], as listed in the table below.

| Attribute name | S |
| --- | --- |
| servAttrCom | CM |
| availability | M |
| periodicityList | M |

The servAttrCom attribute is mandatory only when requirements are being defined on deterministic communication for periodic user traffic per network slice (GSMA attribute). Otherwise, the attribute is optional.

The periodicityList attribute specifies a list of periodicities supported by the network slice for deterministic communication.

The “6.3.9 XLThpt<<dataType>>” defines the throughput properties for attributes dLThptPerSlice, dLThptPerUE, uLThptPerSlice, uLThptPerUE, in which the attributes guaThpt and maxThpt are related to GFBR QoS flow type for deterministic communications.

| Attribute name | S |
| --- | --- |
| servAttrCom | CM |
| guaThpt | O |
| maxThpt | O |

The guaThpt attribute describes the guaranteed data rate.

The maxThpt attribute describes the maximum data rate.

Some of the above attributes above are also available in “6.3.4 SliceProfile <<dataType>>”, which contains CNSliceSubnetProfile, RANSliceSubnetProfile, TopSliceSubnetProfile.

2.	QoS attributes related to deterministic communication

In clause “5.3.76	FiveQICharacteristics” in [3], some attributes related or specific to  deterministic communication service are defined.

| Attribute name | S |
| --- | --- |
| fiveQIValue | M |
| resourceType | M |
| priorityLevel | O |
| packetDelayBudget | O |
| packetErrorRate | O |
| averagingWindow | O |
| maximumDataBurstVolume | O |

The averagingWindow attribute indicates the Averaging Window (in unit of ms) of a 5QI. The maximumDataBurstVolume attribute indicates the Maximum Data Burst Volume (in unit of Byte) of a 5QI. These two parameters are used for deterministic communication service.

In clause “5.3.84	QoSData <<dataType>>” in [3], some attributes for GBR QoS flow type closely related to determinintic communications are extracted as follows:

Attribute name

qosId

fiveQIValue

maxbrUl

maxbrDl

gbrUl

gbrDl

maxPacketLossRateDl

maxPacketLossRateUl

extMaxDataBurstVol

The gbrUl attribute and gbrDl attribute represents the guaranteed uplink and downlink bandwidth respectively. The extMaxDataBurstVol attribute denotes the largest amount of data that is required to be transferred within a period of 5G-AN PDB.

In clause “5.3.81	PccRule <<dataType>>” in [3], some properties specific to GFBR QoS flow type (highlighted) for determinintic communications are extracted as follows:

| Attribute name | S |
| --- | --- |
| pccRuleId | M |
| qosData | M |
| tscaiInputUl | O |
| tscaiInputDl | O |

The tscaiInputDl attribute contains transports TSCAI input parameters for TSC traffic at the ingress of the NW-TT (downlink flow direction).

The tscaiInputUl attribute contains transports TSCAI input parameters for TSC traffic at the ingress interface of the DS-TT/UE (uplink flow direction).

TSCAI describes TSC traffic characteristics for use in the 5G System. TSCAI may be used by the 5G-AN, if provided by SMF. The knowledge of TSC traffic pattern is useful for 5G-AN as it allows more efficiently scheduling of QoS Flows that have a periodic, deterministic traffic characteristics either via Configured Grants, Semi-Persistent Scheduling or with Dynamic Grants.

In clause “5.3.93	TscaiInputContainer <<dataType>> in [3], this data type is defined.

| Attribute name | S |
| --- | --- |
| periodicity | O |
| burstArrivalTime | O |

The Periodicity attribute identifies the time period between the start of two bursts in reference to the TSN GM. The burstArrivalTime attribute Indicates the arrival time (in date-time format) of the data burst in reference to the TSN GM.

3.	The role of DCSA MnS producer in service deployment

For the service deployment of deterministic communications, the generic procedure of network slice provisioning could be utilized, including decomposition of ServiceProfile in the cross domain management into SliceProfile which contains CNSliceSubnetProfile, RANSliceSubnetProfile, TopSliceSubnetProfile in different domain management, feasibility check etc. It is considered the configuration of network resources, QoS policy for the support of deterministic communications etc are the responisibility of the domain management.

DCSA MnS producer in the cross domain management can provide support of SLS requirement analysis, SLS decomposition and feasibility check for ServiceProfile processing. It is handled by the module “service requirement modelling” in DCSA MnS producer. The inputs are the attributes in the ServiceProfile. The outputs are the attributes in the SliceProfile.

DCSA MnS producer in the domain management can provide support of network slice subnet requirement analysis, feasibility check in the network slice subnet level, network resource, network optimization policy e.g. SON policy and QoS policy configuration processing etc. It is handled by the module “Network preparation” and module “Service and Network analysis” in DCSA MnS producer.

Figure 5.X.2.a.2-1 Example of DCSA MnS service procedure in service deployment

1. The NSMS MnS consumer transmits CreateMOI (including ServiceProfile) to the NSMS MnS producer to request for creation of a network slice containing deterministic communication services;

2. The NSMS MnS producer transmits DCSA service Request (including ServiceProfile) to the DCSA MnS producer in the cross domain management for service requirement and feasibility analysis for the deterministic communication services;

3. The DCSA MnS producer in the cross domain management performs service requirement and feasibility analysis, e.g. utilizing the three-layer model to analyse whether the SLS requirements for deterministic communication service could be met, and the recommended result of sliceProfile;

4. The DCSA MnS producer in the cross domain management transmits DCSA service response (result of ServiceProfile analysis) to the NSMS MnS producer;

5. The NSMS MnS producer performs SLS requirements decomposition, generats the SliceProfile;

6. The NSMS MnS producer transmits CreateMOI (including the corresponding xxxSliceSubnetProfile) to each NSSMS MnS producer to request for creation of network slice subnet supporting of deterministic communication service for each domain management;

7. The NSSMS MnS producers in each domain transmits DCSA service Request (including xxxSliceSubnetProfile) to the DCSA MnS producers in the corresponding domain management;

8. The DCSA MnS producers in the corresponding domain management performs network requirements and feasibility analysis for the deterministic communication services, e.g. analyses and prepares network capabilities to ensure the network slice subnet requirements, and provides the corresponding network deployment solution such as network resource management policy and QoS management policy;

9. The DCSA MnS producers in the corresponding domain management transmit DCSA service Response (result of xxxSliceSubnetProfile analysis) to the NSSMS MnS producers;

10. The NSSMS MnS producers performs resource managemen policy and QoS policy configuration in support of the deterministic communication services;

11. The NSSMS MnS producers transmit CreateMOI Response (xxxSliceSubnetId) to the NSMS MnS producer to inform the creation of each xxxSliceSubnet in support the requested deterministic communication services;

12. The NSMS MnS producer transmits CreateMOI Response (including S-NSSAI) to the NSMS MnS consumer to inform the creation of a network slice in support the requested deterministic communication services.

End of change