# 3GPP TSG-SA5 Meeting #145e	S5-225488

# 15 Aug to 24 Aug 2022, E-meeting

# Source:	China Telecom

# Title:	Update key issue for the performance measurement of the NWDAF on the interaction aspect

# Document for:	Approval

# Agenda Item:	6.7.6.2

1	Decision/action requested

The group is asked to discuss and agree on the proposal.

2	References

[1]	3GPP TR 28.864-030: "Study on Enhancement of the management aspects related to NWDAF".

3	Rationale

In potential solution #1 and #2 [1], the Editor’s notes are left about providing performance measurement regarding the type of services provided by the NWDAF based on the logical decomposition, i.e. the analytic related services and the ML Model related services provided by the NWDAF. The potential solution #1 and #2 is updated to focus on the performance measurement related to the NWDAF supporting analytics services, such as NWDAF containg Analytics Logical Function (AnLF). Therefore, the editor notes are removed.

4	Detailed proposal

Start of 1st Change

## 4.3	Key Issue #3: Performance Measurement of the NWDAF on the Interaction Aspect

### 4.3.1	Description

The NWDAF was firstly introduced in Rel-15 and it follows the SBA concept. The interaction between the producer and consumer of the services provided by NWDAF are defined in TS 23.288 [2].

As the requirement on network automation kept increasing, the NWDAF is expected to provide a variety of services to the other NFs and play a very important role in the future. Moreover, in the scenario where multiple NWDAFs are deployed, the NWDAF is also able to act as an Aggregator NWDAF to interact with one to multiple other NWDAFs.

From the perspective of management, it is necessary for the operator to monitor the performance of NWDAF based on the interaction between the producer and consumer of the services provided by the NWDAF. The interaction may include the requests, subscriptions, responses and notifications received and/or generated by NWDAF.

However, in the management domain, the performance measurement of NWDAF on this aspect is missing.

In this key issue, the potential solution(s) is provided on the metrics and KPIs for performance measurement of the NWDAF based on the interaction between the producer and consumer of the analytics services provided by the NWDAF.

Start of 2nd Change

### 4.3.2	Potential solutions

#### 4.3.2.1	Potential solution #1: number of subscriptions and/or requests

##### 4.3.2.1.1	Introduction

This solution is proposed on how to provide the performance measurements of the NWDAF based on the subscriptions and/or requests received by NWDAF. The proposed performance measurements are defined as the number of subscriptions and/or requests received by NWDAF.

The proposed solution can be a potential solution for the performance measurement of the NWDAF supporting analytics services, such as NWDAF containg Analytics Logical Function (AnLF), based on the interaction aspect.

##### 4.3.2.1.2	Description

This proposed performance measurements of the NWDAF are defined as the number of subscriptions and/or requests received by NWDAF.

Moreover, since different type of analytics, network slices and 5G VN group may consume different resource (e.g., virtual CPU resource, virtual memory resource or virtual disk resource) of NWDAF, it is necessary to define the performance measurements of NWDAF based on the granularity of analytics, slice or 5G VN group.

Therefore, this proposed performance measurements are also defined as the number of subscriptions and/or requests received by NWDAF based on the analytics ID, S-NSSAI or 5G VN group ID. The analytics ID identifies the requested analytics and the S-NSSAI identifies the network slice. The 5G VN group ID use External Group ID and Internal Group ID to identify the 5G VN group. The analytics ID, S-NSSAI and Internal or External Group ID can be obtained from the NWDAF service consumer (see TS 23.288 [2]).

The number of subscriptions received by NWDAF can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsSubscription_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted subscription is added to the relevant counter.

The number of requests received by NWDAF can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsInfo_Request service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted request is added to the relevant counter.

The number of subscriptions received by NWDAF based on the analytics ID(s) can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsSubscription_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted subscription is added to the relevant subcounter per analytics ID.

The number of subscriptions received by NWDAF based on the S-NSSAI can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsSubscription_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted subscription is added to the relevant subcounter per S-NSSAI.

The number of subscriptions received by NWDAF based on the 5G VN group can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsSubscription_Subscribe service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted subscription is added to the relevant subcounter per Internal or External Group ID.

The number of requests received by NWDAF based on the analytics ID(s) can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsInfo_Request service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted request is added to the relevant subcounter per analytics ID.

The number of requests received by NWDAF based on the S-NSSAI can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsInfo_Request service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted request is added to the relevant subcounter per S-NSSAI.

The number of requests received by NWDAF based on the 5G VN group can be measured as follows:

- When the NWDAF receives the Nnwdaf_AnalyticsInfo_Request service operation (See TS 23.288 [2]) from the NWDAF service consumer, each accepted request is added to the relevant subcounter per Internal or External Group ID.

Start of 3rd Change

#### 4.3.2.2	Potential solution #2:  number of notifications and/or responses

##### 4.3.2.2.1	Introduction

This solution is proposed on how to provide the performance measurements of the NWDAF based on the notifications and/or responses generated by NWDAF. The proposed performance measurements are defined as the number of notifications and/or responses generated by NWDAF.

The proposed solution can be a potential solution for the performance measurement of the NWDAF supporting analytics services, such as NWDAF containg Analytics Logical Function (AnLF), based on the interaction aspect.

##### 4.3.2.2.2	Description

This proposed performance measurements of the NWDAF are defined as the number of notifications and/or responses generated by NWDAF.

Moreover, since different type of analytics, network slices and 5G VN group may consume different resource (e.g., virtual CPU resource, virtual memory resource or virtual disk resource) of NWDAF, it is necessary to define the performance measurements of NWDAF based on the granularity of analytics, slice or 5G VN group.

Therefore, this proposed performance measurements are also defined as the number of notifications and/or responses generated by NWDAF based on the analytics ID(s), S-NSSAI or 5G VN group ID. The analytics ID identifies the requested analytics and the S-NSSAI identifies the network slice. The 5G VN group ID use External Group ID and Internal Group ID to identify the 5G VN group. The analytics ID(s), S-NSSAI and Internal or External Group ID can be obtained from the NWDAF service consumer (see TS 23.288 [2]).

The number of notifications generated by NWDAF can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsSubscription_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated notification is added to the relevant counter.

The number of responses generated by NWDAF can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsInfo_Request Response service operation to NWDAF service consumer (See TS 23.288 [2]), each generated response is added to the relevant counter.

The number of notifications generated by NWDAF based on the analytics ID(s) can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsSubscription_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated notification is added to the relevant subcounter per analytics ID.

The number of notifications generated by NWDAF based on the S-NSSAI can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsSubscription_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated notification is added to the relevant subcounter per S-NSSAI.

The number of notifications generated by NWDAF based on the 5G VN group can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsSubscription_Notify service operation to NWDAF service consumer (See TS 23.288 [2]), each generated notification is added to the relevant subcounter per Internal or External Group ID.

The number of responses generated by NWDAF based on the analytics ID(s) can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsInfo_Request Response service operation to NWDAF service consumer (See TS 23.288 [2]), each generated response is added to the relevant subcounter per analytics ID.

The number of responses generated by NWDAF based on the S-NSSAI can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsInfo_Request Response service operation to NWDAF service consumer (See TS 23.288 [2]), each generated response is added to the relevant subcounter per S-NSSAI.

The number of responses generated by NWDAF based on the 5G VN group can be measured as follows:

- When the NWDAF generates the Nnwdaf_AnalyticsInfo_Request Response service operation to NWDAF service consumer (See TS 23.288 [2]), each generated response is added to the relevant subcounter per Internal or External Group ID.

End of Modified Sections