3GPP TSG-SA5 Meeting #145e	S5-225517

# 15 - 24 August 2022, E-meeting

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0015 | rev | - | Current version: | 17.0.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MDA request and reporting workflow |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel, NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS |  |  |  |  |  | Date: |  |  | 2022-04-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The MDA request and reporting workflow has a couple of errors: 1) The MDA reporting supports 3 methods - file based reporting, streaming based reporting and NRM notification-based reporting, however the workflow provides only one option; 2) The MDAReport IOC is defined but not used in the workflow; 3) The operation and notification names are not correctly given in the workflow. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the errors of the workflow as mentioned above. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mistake in the workflow will lead to wrong implementation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

## 11.1	MDA request and reporting workflow

Figure 11.1-1: Generic MDA request and reporting workflow

1.	MDAS Producer receives a createMOI (see createMOI operation defined in TS 28.532 [11]) request from an authorized MDAS Consumer to create an MDARequest MOI (see clause 9).

2.	The MDAS Producer creates the MOI for the MDARequest IOC per the createMOI request.

3.	The MDAS Producer sends the createMOI response  to the MDAS Consumer with DN of the MOI.

4.	The MDAS producer subscribes to the relevant notifications or setup the streaming connections, per the selected reporting method (identified by reportingMethod attribute in the MDARequest MOI):

-	If the reportingMethod designated in the MDARequest MOI is “File”:

4a.	if subscription for the reporting target (specified by the reportingTarget attribute in the MDARequest MOI) do not exist, the MDAS producer subscribes to the file data reporting related notifications (see TS 28.532 [11]) for the reporting target;

-	If the reportingMethod designated in the MDARequest MOI is “Streaming”:

4b/4c.	 if the streaming connection with the reporting target does not exist, the MDAS producer invokes the establishStreamingConnection operation (see TS 28.532 [11]) to setup the streaming connection with the streaming target;

4d/4e.	the MDAS producer invokes the addStream operation (see TS 28.532 [11]) to add the stream for the expected MDA reports.

4f/4g.	if the newly added stream is to replace an existing one, the MDAS producer invokes the deleteStream operation (see TS 28.532 [11]) to delete the stream.

- If the reportingMethod designated in the MDARequest MOI is “Notification”:

4h.	if subscription for the reporting target do not exist, the MDAS producer subscribes to the provisioning related notifications (see TS 28.532 [11]) for the reporting target.

5.	While the MDARequest is active, the MDAS Producer keeps performing MDA,  and making the MDA report (see the MDAReport IOC defined in clause 9) according to the MDARequest MOI.

5a.	the MDAS producer makes the MDA report ready and sends the MDA report to the reporting target per the selected reporting method (identified by reportingMethod attribute in the MDARequest MOI):

-	If the reportingMethod designated in the MDARequest MOI is “File”:

5b.	the MDAS producer makes the MDA report into a file;

5c.	the MDAS producer emits the notifyFileReady notification (see TS 28.531 [11]) to the reporting target for the MDA report.

-	If the reportingMethod designated in the MDARequest MOI is “Streaming”:

5d.	the MDAS producers makes the MDA report into a stream date unit;

5e.	invokes the reportStreamData operation (see TS 28.531 [11]) to the reporting target for the MDA report.

- If the reportingMethod designated in the MDARequest MOI is “Notification”:

5f.	the MDAS producer creates and MDAReport MOI (see clause 9) for the MDA report;

5g.	if notifyMOICreation is used, the MDAS producer emits the notifyMOICreation notification (see TS 28.531 [11]) to the reporting target for the MDA report.

5h.	if notifyMOIChanges is used, the MDAS producer emits the notifyMOIChanges notification (see TS 28.531 [11]) to the reporting target for the MDA report.

Next modified section

Annex X (informative):
PlantUML source code

# X.1	PlantUML code for MDA workflow

This annex contains the PlantUML source code for the MDA workflow specified in clause 11 of the present document.

## X.1.1	PlantUML code for MDA requesting and reporting workflow

@startuml

skinparam shadowing false

skinparam monochrome true

hide footbox

participant "MDAS consumer" as MC

participant "MDAS producer" as MP

participant "Reporting target" as RT

MC -> MP: 1. createMOI request \n (for MDARequest IOC)

MP -> MP: 2. create the MOI for MDARequest IOC

MP --> MC: 3. createMOI response \n (for MDARequest IOC)

Alt reportingMethod is "File"

opt if the file data reporting related \nnotifications are not subscribed yet

MP -> MP: 4a. subscribe to the file data reporting related notifications \nfor the reporting target

end

else reportingMethod is "Streaming"

opt if streaming(WebSocket) connection does not exist

MP -> RT: 4b establishStreamingConnection request

RT --> MP: 4c establishStreamingConnection response

end

MP -> RT: 4d. addStream

RT --> MP: 4e addStream response

opt if an existing stream is replaced

MP -> RT: 4f. deleteStream

RT --> MP: 4g deleteStream response

end

else reportingMethod is "Notification"

opt if the provisioning related notifications are not \nsubscribed for the reporting target yet

MP -> MP: 4h. subscribe to the provisioning related notifications \n for the reporting target

end

end

loop while the MDARequest is active

MP -> MP: 5a. perform MDA and make the MDA report ready \n (according to the MDAReport IOC)

Alt reportingMethod is "File"

MP -> MP: 5b. make the MDA report into a file

MP -> RT: 5c. notifyFileReady (for the MDA report)

else reportingMethod is "Streaming"

MP -> MP: 5d. make the MDA report into a stream data unit

MP -> RT: 5e. reportStreamData (for the MDA report)

else reportingMethod is "Notification"

MP -> MP: 5f. create an MDAReport MOI for the MDA report

Alt notifyMOICreation is used \nto notify the creation of MDAReport MOI

MP -> RT: 5g. notifyMOICreation \n (for MDAReport IOC)

else notifyMOIChanges is used \nto notify the creation of MDAReport MOI

MP -> RT: 5h. notifyMOIChanges \n (for MDAReport IOC)

end

end

end

@enduml

End of modified sections