3GPP TSG-SA5 Meeting #145-e 	S5-225525

# e-meeting, 15 August – 24 August 2022

# Source:	China Mobile

# Title:	pCR TR 28.833 Add potential solutions for topic 3

# Document for:	Approval

Agenda Item:	6.8.4.3

- Decision/action requested

The group is asked to discuss and endorse the proposal in section 4

2	References

[1]	SP-220324 " New Study on Management Aspects of 5GLAN "

[2]             3GPP TS 23.552: "Management and orchestration;5G performance measurements".

[3]             3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2"

3	Rationale

This contribution proposes to add potential solutions for topic 3.

4	Detailed proposal

It proposes to make the following changes.

1st Change

5	Topics

5.3	Topic 3: for 5G VN group performance measurement management

5.3.1	Use case

The performance of 5G LAN-type services need to be monitored in 5G VN group level by the operator since it is relevant to whether the end users can use the service of 5G LAN and scale up/down a 5G VN based on capacity for efficient consumption of network resources.

The performance measurement of 5G LAN-type services may include measurement of 5G VN group status, the number of subscribers in 5G VN groups, the duration of 5G VN group communication which information can be gathered from NEF/UDM/SMF/UPF/PCF supporting 5G LAN-type services.

Therefore, the 3GPP management system needs to support the request for 5G VN group performance measurement management as it impacts the end user experience, and the optimization may be required according to the performance.

5.3.2	Potential requirements

REQ-LAN-PMM-01 The 3GPP management system shall have the capability to measure 5G LAN-type services performance (e.g. UE throughput, Performance measurements for NEF/UDM/SMF/UPF/PCF) to be assured in 5G VN group level.

5.3.3	Key Issues

5.3.3.1	Description

TS 28.552 describes the performance measurements for AMF, SMF, UPF, UDM, PCF and NEF.

Some performance measurements can be meanfule to be provided to a 5G VN group communication, for example:

- Performance measurement for SMF: Number of successful PDU session creations,

- Performance measurement for UPF: Number of incoming GTP data packets on the N3 interface, from (R)AN to UPF,

- Performance measurement for NEF: Number of application trigger requests accepted for delivery.

According to the use case described in clause 5.3.1, the following issue should be investigated in this study.

Key issue#Y: Which performance measurements are feasible to be provided by 3GPP management system for a 5G VN group communication?

5.3.4	Solutions

Performance indicators at 5G VN group level can be derived from the performance measurements via the corresponding performance management service consumed by a 5G VN group. 5G VN group can be associated with External Group ID and Internal Group ID. The performance indicators are split into subcounters per External Group ID or Internal Group ID for 5G VN group.

In case a performance measurement is defined for more than one sub-counter, it is convenient to use Filter to define the performance measurement of interest  (see TS 23.552[2]). Therefore,  a new Filter value External Group ID or Internal Group ID is added.

For example, number of successful PDU session creations is the number of PDU sessions that are associated with a 5G VN group and successfully created by the SMF. On transmission by the SMF to AMF of Nsmf_PDUSession_CreateSMContext Response that indicates a successful PDU session creation (see TS 23.502 [3]). Each PDU session successfully created is added to the relevant subcounter per External Group ID or Internal Group ID.

Number of incoming GTP data packets on the N3 interface, from (R)AN to UPF is the number of GTP data PDUs associated with a 5G VN group on the N3 interface which have been accepted and processed by the GTP-U protocol entity in UPF on the N3 interface. The measurement can be split into subcounters per External Group ID or Internal Group ID.

For the number of application trigger requests  accepted for delivery, transmission of Nnef_Trigger_Delivery response by the NEF to AF indicating the application trigger request associated with a 5G VN group has been accepted for delivery to the UE (see TS 23.502 [7]). Each response increments the relevant subcounter per External Group ID or Internal Group ID by1.

Editor’s note: Whether the measurement method is given for each performance indicator is FFS.

5.3.5	Evalution

5.3.6	Conclusion

.

End of Change