3GPP TSG-SA5 Meeting #145-e 	S5-225547

# e-meeting, 15 - 24 August 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0128 | rev | - | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add asynchronous network slicing provisioning procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Deutsche Telekom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2022-08-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current procedures for network slicing are based on synchronous operations. In reality the procedures may take longer and the consumer has no information about the progress and status of the operation. A solution needs to be defined based on an asynchronous pattern. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The procedures in clause 7 for allocation, deallocation and modification on both slice and slice subnet level are updated to use a common asynchronous pattern based on Job IOCs managed by generic provisioning operations from TS 28.532. The main steps of each procedure (see ‘Background process’ block in each diagram) are not meant to be changed by this CR and only have some minor editorial updates. Due to change to asynchronous pattern, the slicing-specific operations in clause 6 and corresponding stage 3 solutions in clause 9 are no longer used by any procedure and are therefore removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No asynchronous option is available for the network slicing provisioning procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1, 6.2 , 6.5.1, 6.5.2, 6.5.3, 6.5.4, 7.2, 7.3, 7.4, 7.5, 7.6, 7.7, 9,1, 9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First Change

## 6.1	Management services for network slice provisioning

The management services for network slice provisioning are listed in table 6.1-1.

Table 6.1-1: Management services for network slice provisioning

| MnS Name | MnS Component Type A (operations and notifications) | MnS Component Type B (information model) | Note |
| --- | --- | --- | --- |
| Network Slice Provisioning | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - deleteMOI operation - getMOIAttributes operation - modifyMOIAttributes operation | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to request allocating, deallocating, or modifying an NSI. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators. |
| Network Slice Provisioning data reporting | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NSI information model defined in clause 6.3 of TS 28.541 [6] | This management service enables its consumer to obtain notifications about NSI Information model data. The typical scenario is "Network Slices as NOP internals" model where this MnS is consumed by operators |

## 6.2	Management services for network slice subnet provisioning

The management services for network slice subnet provisioning are listed in table 6.2-1.

Table 6.2-1: Management services for NSS provisioning

| MnS Name | MnS Component of type A (Operations and notifications) | MnS Component of type B (information model) |
| --- | --- | --- |
| Network Slice Subnet Provisioning | Operations defined in clause 5 of TS 28.532 [8]: - createMOI operation - modifyMOIAttributes operation - getMOIAttributes operation - deleteMOI operation | NSSI information model defined in clause 6.3 of TS 28.541 [6] |
| Network Slice Subnet Provisioning data reporting | Operations defined in clause 5 of TS 28.532 [8]: - subscribe operation - unSubscribe operation Notifications defined in clause 5 of TS 28.532 [8]: - notifyMOICreation notification - notifyMOIDeletion notification - notifyMOIAttributeValueChanges notification | NSSI information model defined in clause 6.3 of TS 28.541 [6] |

Second Change

## 6.5.	Operations of provisioning

### 6.5.1	Void

### 6.5.2	Void

### 6.5.3	Void

### 6.5.4	Void

### 6.5.5	AllocateNetwork operation

#### 6.5.5.1	Description

This operation is invoked by allocateNetwork operation service consumer to request the provider to satisfy the network related requirements.

#### 6.5.5.2	Input parameters

| - Parameter Name | - Support Qualifier | - Information Type / Legal Values | - Comment |
| --- | --- | --- | --- |
| - attributeListIn | - M | - LIST OF SEQUENCE< attribute name, attribute value> | - This parameter specifies the network related requirements defined in ServiceProfile in Clause 6.3.3 in TS 28.541 [6]. |

#### 6.5.5.3	Output parameters

| - Parameter name | - Support Qualifier | - Matching Information / Legal Values | - Comment |
| --- | --- | --- | --- |
| - serviceProfileId | - M | - String | - A unique identifier of the network related requirements which have been supported by the allocated network. |
| - status | - M | - ENUM (OperationSucceeded, OperationFailed) | - An operation may fail because of a specified or unspecified reason. |

Third Change

# 7	Provisioning procedures of networks and network slicing

## 7.1	General

The procedures of provisioning of 5G networks are listed in the following subclauses.

## 7.2	Procedure of Network Slice Instance Allocation

The Figure 7.2-1 illustrates the procedure of creating a new NSI or using an existing NSI to satisfy the required network slice related requirements.

Figure 7.2-1: Network Slice Instance Allocation Request procedure

1)	The Network Slice Management Service Provider (NSMS_Provider) receives an AllocateJob creation request (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]) for an instance of AllocateJob IOC (see clause 6.3.xx in TS 28.541 [6]) from the Network Slice Management Service Consumer (NSMS_Consumer) with network slice related requirements (the network slice related requirements are defined as the attributes in the ServiceProfile see clause 6.3.3 in TS 28.541 [6]).

2)	The NSMS_Provider creates and configures an AllocateJob instance and initiates execution of the NSI allocation process.

3a)	The NSMS_Provider sends an AllocateJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the AllocateJob instance (see clause 6.3.xx in TS 28.541 [6]).

4)	Based on the network slice related requirements and the knowledge of the capabilities of existing deployed network slices, the NSMS_Provider compare/match the provided requirements against all the candidate NetworkSlice instances, and then decides whether to use an existing NSI or create a new NSI. If the network slice related requirements allow the requested NSI to be shared and if an existing suitable NSI can be reused, the NSMS_Provider may decide to use the existing NSI.

5a)	If using an existing NSI and the existing NSI needs to be modified to satisfy the network slice related requirements, the NSMS_Provider invokes the procedure to modify the existing NSI as described in clause 7.6.

5b-1)	If creating a new NSI, the NSMS_Provider derives the network slice subnet related requirements from the received network slice related requirements. Before NSMS_Provider derives the network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.

5b-2)	The NSMS_Provider invokes the NSSI allocation procedure as described in clause 7.3.

5b-3)	The NSMS_Provider creates the MOI for NetworkSlice and configures the MOI with the DN of MOI for the NetworkSliceSubnet, other configuration information may be configured for the created MOI.

Note: The detailed configuration information is described in network slice NRM (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6]).

6a)	If the NSMS_Consumer has subscribed to MOI change notifications, the NSMS_Provider sends the NSI allocation result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSMS_Consumer. The notification will include updated AllocateJob attribute values that cover process status and if successful also additional outputs:

- DN of an existing or  newly created  NSI that has been selected to satisfy the network slice related requirements (see NetworkSlice IOC defined in clause 6.3.1 in TS 28.541 [6])

- Any values that have been assigned by the MnS producer for the ServiceProfile.

Otherwise, the result may include the reason of failure, for example, the required latency or user number cannot be satisfied, or the physical resource is not enough.

Note: The NSMS_Producer may also update certain attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured. Additionally, since attribute value changes are only reported once, it is possible that the final notification will not include all of the MOI changes if some changes had already been reported by an earlier notification.

6b)	The NSMS_Provider sends the final result of the Allocate job (DN) with the new or updated NSI instance information to the NSMS_Consumer.

6c-1)	If the NSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

6c-2)	The NSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

7)	When the NSMS_Consumer decides to delete the AllocateJob instance (e.g., after obtaining the allocation result), the NSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSMS_Provider for the specific AllocateJob instance.

8)	The NSMS_Provider deletes the specified AllocateJob instance.

)	The NSMS_Provider sends an AllocateJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

## 7.3	Procedure of Network Slice Subnet Instance Allocation

The Figure 7.3-1 illustrates the procedure of creating a new network slice subnet instance or using an existing network slice subnet instance to satisfy the required network slice subnet related requirements.

Figure 7.3-1: Network Slice Subnet Instance Allocation Request procedure

1)	The Network Slice Subnet Management Service Provider (NSSMS_P) receives an AllocateJob creation request (see createMOI operation defined in clause 11.1.1.1   in TS 28.532 [8]) for an instance of AllocateJob IOC (see clause 6.3.xx in TS 28.541 [6]) from the Network Slice Subnet Management Service Consumer (NSSMS_C) with network slice subnet related requirements (network slice subnet related requirements defined as the attributes in the SliceProfile see clause 6.3.4 in TS 28.541 [6]).

2)	The NSSMS_Provider creates an AllocateJob instance and initiates execution of a NSSI allocation process.

3a)	The NSSMS_Provider sends an AllocateJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the AllocateJob instance (see clause 6.3.xx in TS 28.541 [6]).

4)	The NSSMS_Provider checks the feasibility of network slice subnet related requirements. If the network slice subnet related requirements can be satisfied, the steps 5) and either the Modify NSSI procedure or step 6  is needed, otherwise step 8 or step 9.

5)	Based on the network slice subnet related requirements and the existing NSSI capabilities, the NSSMS_Provider decides whether to use an existing NSSI or create a new NSSI. If the network slice subnet related requirements allow the requested NSSI to be shared and if an existing suitable NSSI can be reused, the NSSMS_Provider decides to use the existing NSSI.

6a) 		If using an existing NSSI and the existing NSSI needs to be modified to satisfy the network slice subnet related requirements, the NSSMS_Provider invokes the procedure to modify the existing NSSI as described in clause 7.7.

b-1)	If creating a new NSSI, the NSSMS_Provider creates the NetworkSliceSubnet MOI. The NSSMS_Provider derives the corresponding network slice subnet constituent (i.e. NF, constituent NSS) related requirements and transport network related requirements (e.g. 3GPP endpoint information, latency requirements, bandwidth requirements and isolation requirements) from the received network slice subnet related requirements. Part of these requirements may be referenced by attribute "epTransportRef" as defined in clause 6.3.2.2 in TS 28.541[6]. Before NSSMS_Provider derives the constituent network slice subnet related requirements, NSMS_Provider may invoke corresponding network slice subnet capability information querying procedure as described in clause 7.8.2.

6b-2)	If the NSSI to be created contains virtualisation part (i.e. VNF or VL), the NSSMS_Provider derives the NS instance instantiation information (the NS instance instantiation information is described in clause 7.3.2.2 and clause 7.3.3.2 [3]) based on network slice subnet related requirements. The NSSMS_Provider determines new VNF instance(s) that need to be deployed and the existing VNF instance(s) that need to be reused according to the necessary network function(s) and then derives the profile of virtual link(s) according to the connection requirements between the network functions. The NSSMS_Provider chooses a proper NSD deployment flavour and creates data concerning the SAPs of the NS instance. The NSSMS_Provider invokes the NS instantiation procedures to create a NS instance. The NSSMS_Provider configures the NSS MOI with the NS instance identifier.

Note: NS instantiation procedure is described in TS 28.526 [7].

For each required NSSI constituent, the following steps 6b-3a), 6b-3b) and 6b-4) are needed:

6b-3a)	If the required NSSI constituent is constituent NSSI, the NSSMS_Provider invokes NSSI Allocation Procedure.

6b-3b)	If the required NSSI constituent is NF instance, the NSSMS_Provider invokes NF Creation Procedure as described in clause 7.10 or NF Modification Procedure as described in clause 7.11.

6b-4)	The NSSMS_Provider configures the NetworkSliceSubnet MOI with the DN of the MOI for NSSI constituent (i.e. ManagedFunction MOI, NetworkSliceSubnet MOI).

6b-5)	For each required transport network related requirements, the NSSMS_Provider invokes corresponding procedure of coordination with relevant TN Manager to handle the TN part as described in clause 7.9.

7a)	If the NSSMS_Consumer has subscribed to MOI change notifications, the NSSMS_Provider sends the NSSI allocation result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSSMS_Consumer. The notification will include updated AllocateJob attribute values that cover process status and if successful also additional outputs:

- DN of an existing or newly created NSSI that has been selected to satisfy the network slice subnet related requirements (see NetworkSliceSubnet IOC defined in clause 6.3.2 in TS 28.541 [6]).

- Any values that have been assigned by the MnS producer for the SliceProfile.

Otherwise, the result may include the reason of failure, for example, the required latency or user Number cannot be satisfied, or the physical resouce is not enough.

Note: The NSSMS_Producer may also update certain attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured. Additionally, since attribute value changes are only reported once, it is possible that the final notification will not include all of the MOI changes if some changes had already been reported by an earlier notification.

7b)	The NSMS_Provider sends the final result of the Allocate job (DN) with the new or updated NSI instance information to the NSMS_Consumer.

7c-1)	If the NSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

7c-2)	The NSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

8)	When the NSMS_Consumer decides to delete the AllocateJob instance (e.g., after obtaining the allocation result), the NSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSMS_Provider for the specific AllocateJob instance.

9)	The NSMS_Provider deletes the specified AllocateJob instance.

10)	The NSMS_Provider sends an AllocateJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

## 7.4	Procedure of Network Slice Instance Deallocation

Figure 7.4-1 depicts the procedure of deallocating a network slice instance by the network slice management service provider to satisfy the NSI deallocation request received from an authorized consumer.

Figure 7.4-1: Network slice instance deallocation procedure

1)	The Network Slice Management Service Provider (NSMS_Provider) receives a DeallocateJob creation request (see createMOI operation defined in clause 11.1.1.1  in TS 28.532 [8] ) for an instance of DeallocateJob IOC (see clause 6.3.xx in TS 28.541 [6]) from the Network Slice Management Service Consumer (NSMS_Consumer) indicating that the NetworkSlice MOI is no longer needed for the given requirements i.e ServiceProfile.

2)	The NSMS_Provider creates a DeallocateJob instance and initiates execution of a NSI deallocation process.

3a)	The NSMS_Provider sends a DeallocateJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the DeallocateJob instance (see clause 6.3.xx in TS 28.541 [6]).

4a)	The NSMS_Provider may decide to terminate the NSI, and invokes the NSSI deallocation procedure as described in clause 7.5.

4b)	The NSMS_Provider may decide not to terminate but modify the NSI, and invokes the NSI modification procedure as described in clause 7.6.

5a)	If the NSMS_Consumer has subscribed to MOI change notifications, the NSMS_Provider sends the NSI deallocation result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSMS_Consumer.

Note: The NSMS_Provider may also update certain status attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured.

5b)	The NSMS_Provider sends the final result of the Deallocate job (DN) with the new or updated NSI instance information to the NSMS_Consumer.

5c-1)	If the NSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

5c-2)	The NSMS_Providerr will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

6)	When the NSMS_Consumer decides to delete the DeallocateJob instance (e.g., after obtaining the deallocation result), the NSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSMS_Provider for the specific DeallocateJob instance.

7)	The NSMS_Provider deletes the specified DeallocateJob instance.

8)	The NSMS_Provider sends a DeallocateJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

## 7.5	Procedure of network slice subnet instance deallocation

Figure 7.5-1 depicts the procedure of deallocating a network slice subnet instance by the network slice subnet management service provider to satisfy the NSSI deallocation request received from an authorized consumer.

Figure 7.5-1: Network slice subnet instance deallocation procedure

1)	The Network Slice Subnet Management Service Provider (NSSMS_Provider) receives a DeallocationJob creation request (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]) for an instance of DeallocateJob IOC (see clause 6.3.xx in TS 28.541 [6])  from the Network Slice Subnet Mnagement Service Consumer (NSSMS_Consumer) indicating that the NetworkSliceSubnet MOI is no longer needed for the given requirements i.e SliceProfile.

2)	The NSSMS_Provider creates a DeallocateJob instance and initiates execution of a NSSI deallocation process.

3a)	The NSSMS_Provider sends a DeallocateJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the DeallocateJob instance (see clause 6.3.xx in TS 28.541 [6]).

4a-1)	The NSSMS_Provider may decide to terminate the NSSI, it invokes (constituent) NSSI deallocation procedure as described in clause 7.5 if the NSSI consists of constituent NSSI.

4a-2)	The NSSMS_Provider invokes NF deletion procedure as described in clause 7.12 only if the NF is dedicated for this NSSI and not being used by any other NSSI in the network, otherwise, the NSSMS_Provider invokes NF modification procedure as described in clause 7.11.

4a-3)	The NSSMS_Provider invokes TN related coordination procedure with responsible manager as described in clause 7.9 if the NSSI consists of TN part.

4a-4)	The NSSMS_Provider invokes NS termination procedure if the NSSI contains virtualized part.

Note: NS termination procedure is described in TS 28.526 [7].

4b)	The NSSMS_Provider may decide not to terminate the NSSI, it invokes NSSI modification procedure as described in clause 7.7.

5a)	If the NSSMS_Consumer has subscribed to MOI change notifications, the NSSMS_Provider sends the NSSI deallocation result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSSMS_Consumer.

Note: The NSSMS_Producer may also update certain status attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured.

5b)	The NSSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

5c-1)	If the NSSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

5c-2)	The NSSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

6)	When the NSSMS_Consumer decides to delete the DeallocateJob instance (e.g., after obtaining the deallocation result), the NSSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSSMS_Provider for the specific DeallocateJob instance.

7)	The NSSMS_Provider deletes the specified DeallocateJob instance.

8)	The NSSMS_Provider sends a DeallocateJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

## 7.6	Procedure of Network Slice Instance Modification

The Figure 7.6-1 illustrates the procedure of modifying an existing NSI.

Figure 7.6-1: Network Slice Instance Modification Request procedure

1)	The Network Slice Management Service Provider (NSMS_Provider) receives a ModifyJob creation request (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]) for an instance of ModifyJob IOC (see clause 6.3.xx in TS 28.541 [6]) from the Network Slice Management Service Consumer (NSMS_Consumer) with the DN of NetworkSlice MOI and the new network slice related requirements (see ServiceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	The NSMS_Provider creates a ModifyJob instance and initiates execution of a NSI modification procedure.

3a)	The NSMS_Provider sends a ModifyJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the ModifyJob instance (see clause 6.3.xx in TS 28.541 [6]).

4)	Based on the new network slice related requirements, the NSMS_Provider invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 5), else go to step 7).

5)	The NSMS_Provider decomposes the NetworkSlice MOI modification request into NSSI modification request(s), i.e., generating the new network slice subnet related requirements for each NSSI if needed.

6)	The NSMS_Provider, as the role of Network Slice Subnet Management Service Consumer (NSSMS_Consumer), invokes the NSSI modification procedure.

7a)	If the NSMS_Consumer has subscribed to MOI change notifications, the NSMS_Provider sends the NSI modification result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSMS_Consumer.

Note: The NSMS_Producer may also update certain status attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured.

7b)	The NSMS_Provider will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

7c-1)	If the NSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

7c-2)	The NSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

8)	When the NSMS_Consumer decides to delete the ModifyJob instance (e.g., after obtaining the modification result), the NSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSMS_Provider for the specific ModifyJob instance.

9)	The NSMS_Provider deletes the specified ModifyJob instance.

10)	The NSMS_Provider sends a ModifyJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

## 7.7	Procedure of Network Slice Subnet Instance Modification

The Figure 7.7-1 illustrates the procedure of modifying an existing NSSI.

Figure 7.7-1: Network Slice Subnet Instance Modification Request procedure

1)	The Network Slice Subnet Management Service Provider (NSSMS_Provider) receives a ModifyJob creation request (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]) for an instance of ModifyJob IOC (see clause 6.3.xx in TS 28.541 [6]) from the Network Slice Subnet Management Service Consumer (NSSMS_Consumer) with the DN of NetworkSliceSubnet MOI and the new network slice subnet related requirements (see SliceProfile defined in clause 6.3.3 in TS 28.541[6]).

2)	The NSSMS_Provider creates a ModifyJob instance and initiates execution of a NSSI modification process.

3)	The NSSMS_Provider sends a ModifyJob creation response (see createMOI operation defined in clause 11.1.1.1 in TS 28.532 [8]).

3b)	The NSMS_Consumer subscribes to change notifications from the AllocateJob instance (see clause 6.3.xx in TS 28.541 [6]).

4)	Based on the new network slice subnet related requirements, the NSSMS_Provider invokes the feasibility check procedure. If the modification requirements can be satisfied, go to step 5), else go to step 10).

5)	The NSSMS_Provider decomposes the NetworkSliceSubnet MOI modification request into modification requests for each NSSI constituent.

6)	If the requested NSSI constituent is constituent NSSI, the NSSMS_Provider invokes NSSI modification procedure as described in clause 7.7.

7)	If the requested NSSI constituent is NF instance, the NSSMS_Provider invokes NF creation procedure as described in clause 7.10 or NF modification procedure as described in clause 7.11.

8)	If the NSSI contains the virtualized part, the NSSMS_Provider invokes the NS instance scaling and/or NS instance updating and/or NS instance instantiation procedure as described in TS 28.526 [7].

9)	If the NSSI contains the TN part, the NSSMS_Provider invokes the TN related coordination procedure as described in clause 7.9.

10a)	If the NSSMS_Consumer has subscribed to MOI change notifications, the NSSMS_Provider sends the NSSI modification result via notification (see notifyMOIChanges defined in clause 11.1.1.11 in TS 28.532 [8]) to the NSSMS_Consumer.

Note: The NSSMS_Producer may also update certain status attributes while the procedure is still in progress. Such updates would also trigger MOI change notifications to be sent in case a matching subscription has been configured.

10b)	The NSSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

10c-1)	If the NSSMS_Consumer has not subscribed to MOI change notifications, it may instead poll at regular intervals the status of the procedure as well as other MOI attributes of interest using a getMOIAttributes request (see clause 11.1.1.2 in TS 28.532 [8]).

10c-2)	The NSSMS_Producer will send a getMOIAttributes response including current values of the requested attributes (see clause 11.1.1.2 in TS 28.532 [8]).

11)	When the NSSMS_Consumer decides to delete the ModifyJob instance (e.g. after obtaining the modification result), the NSSMS_Consumer sends a job deletion request (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]) to the NSSMS_Provider for the specific ModifyJob instance.

12)	The NSSMS_Provider deletes the specified ModifyJob instance.

13)	The NSSMS_Provider sends a ModifyJob deletion response (see deleteMOI operation defined in clause 11.1.1.4 in TS 28.532 [8]).

Third Change

# 9	RESTful HTTP-based solution set of provisioning

## 9.1	Void

## 9.2	Void

End of Changes