3GPP TSG-SA5 Meeting #145-e 	S5-225551

# e-meeting, 15 - 24 August 2022

# Source:	Huawei

# Title:	Potential solution No.2, conclusion and recommendation for KI #1

# Document for:	Approval

Agenda Item:	6.9.2.1

# 1	Decision/action requested

Include the proposed changes in TR 28.913

# 2	References

[1]		3GPP TR 28.913: "Study on new aspects of EE for 5G networks phase 2"

[2]	SP-211440: "New Study on new aspects of EE for 5G networks Phase 2"

# 3	Rationale

During SA5#142e, S5-222024 introduced a new key issue ‘Considering additional virtual resources usage metrics in EC estimation’, which proposed to consider vDisk usage, in addition to vCPU usage, to estimate the energy consumption of VNF/VNFCs. The key issue and potential solution #1 are captured in clause 4.1 of [1].

During SA5#144e, S5-224093 proposed to consider the input/output traffic volume of virtual resources, in addition to vCPU and vDisk usage, to estimate the energy consumption of VNF/VNFCs. This proposal has been rejected, and it was mentioned that incoming/outgoing traffic is already impacting vCPU or vDisk processing, meaning that, whatever processing vCPUs and vDisks are doing, it is for handling traffic volumes. Hence considering traffic volume again in the equation as separate influencing factor has been deemed irrelevant, as it would make traffic volume impact to be considered twice. During offline discussions, it was also said that the vMemory usage of virtual compute resources has also no significant impact on their energy consumption.

Therefore, this pCR proposes to capture this (see Potential solution No. 2) and add a conclusion and a recommendation for this key issue.

# 4	Detailed proposal

First change

## 4.1	Key Issue #1: Considering additional virtual resources usage to estimate VNF energy consumption

### 4.1.1	Description

In Release 17 (see [2] clause 6.7.3.1), the Energy Consumption (EC) of VNFs is obtained by summing up the estimated energy consumption of its constituent Virtualized Network Function Components (VNFC), where the estimated energy consumption of a VNFC is obtained by taking the estimated energy consumption of the virtual compute resource instance on which the VNFC runs. The energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource instance X runs. This proportion is obtained by dividing the vCPU mean usage of the virtual compute resource instance X, by the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node as X, during the same observation period.

This key issue investigates how additional performance measurements of virtual compute resources, also provided by NFV MANO, can be considered in the estimation of the energy consumption of VNFCs, and consequently of VNFs.

### 4.1.2	Potential solutions

#### 4.1.2.1	Potential solution #1: Consideration of mean disk usage

##### 4.1.2.1.1	Introduction

In this potential solution #1, it is proposed to consider the mean virtual disk usage of the virtualised compute resource instance, in addition to the mean vCPU usage, to estimate the energy consumed by the virtual compute resource instance. Thus, the definition of the estimated energy consumption of a virtual compute resource instance combines both virtual CPU mean usage and virtual disk mean usage.

##### 4.1.2.1.2	Description

In this potential solution #1, the energy consumption of a virtual compute resource instance X is estimated as a proportion of the energy consumption of the NFVI node on which the virtual compute resource runs. This proportion is obtained by adding weighted relative mean virtual CPU usage and virtual disk usage of the virtual compute resource instance X. The relative mean virtual CPU usage of the virtual compute resource instance X is obtained by dividing the vCPU mean usage of the virtual compute resource instance X, by the sum of the vCPU mean usage of all virtual compute resource instances running on the same NFVI Node as X, The relative mean virtual disk usage of the virtual compute resource instance X is obtained by dividing the vDisk mean usage of the virtual compute resource instance X, by the sum of the vDisk mean usage of all virtual compute resource instances running on the same NFVI Node as X. This is defined by the equation below:

, where:

- VCpuUsageMean is the mean virtual CPU usage of the virtual compute resource instance during the observation period, provided by NFV MANO,

-  is the sum of the mean virtual CPU usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.1.2 of [3],

- VDiskUsageMean is the mean virtual disk usage of the virtual compute resource instance during the observation period, provided by NFV MANO,

- is the sum of the mean virtual disk usage of all virtual compute resource instances running on the same NFVI Node during the same observation period, all separately provided by NFV MANO (see clause 7.1.6 of [3],

- ECNFVINode,measured is the measured energy consumption of the NFVI node on which the virtual compute resource instance runs, during the same observation period, as per ETSI ES 202 336-12 [4],

- w1 and w2 are the weights assigned by the Operator to VCpuUsageMean and VDiskUsageMean respectively. How the Operator assigns values to w1 and w2 is not subject to standardization. However, it should be noted that:

, or, stated differently:

#### 4.1.2.i	Potential solution #2: Consideration of other performance measurements related to virtual compute resources

##### 4.1.2.i.1	Introduction

ETSI GS NFV-IFA 027 [3] clause 7 defines, in addition to virtual compute resource CPU usage (clauses 7.1.2 and 7.1.3) and disk usage (clauses 7.1.5 and 7.1.6) performance measurements, additional virtual compute resource performance measurements produced by VIM such as:

# vMemory usage (clauses 7.1.4 and 7.1.5), and

# incoming/outgoing traffic (clauses 7.1.8 and 7.1.9).

##### 4.1.2.i.2	Description

In this potential solution #2:

# regarding vMemory, it is proposed to consider that the vMemory usage of virtual compute resources has no significant impact on their energy consumption;

# regarding incoming / outgoing traffic, it is proposed to consider that the incoming/outgoing traffic is anyway impacting vCPU or vDisk processing, meaning that, whatever processing vCPU and vDisk is doing, it is for handling traffic volumes. Hence considering traffic volume again in the equation as separate influencing factor is deemed irrelevant, as it would make traffic volume impact to be considered twice.

### 4.1.3	Conclusion

The potential solution #1, which proposes to consider the weighted vDisk usage of the virtual compute resources, in addition to the weighted vCPU usage (see TS 28.554 [2] clause 6.7.3.1.4), provides additional flexibility to the network operator for the estimation of the energy consumed by virtual compute resources/VNF/VNFCs.

Other virtual compute resource related performance measurements such as vMemory usage and incoming/outgoing traffic are considered as respectively having non-significant impact on the energy consumption of virtual compute resources/VNFC/VNFs and already impacting the vCPU / vDisk usage of virtual compute resources. For this reason, there is no justification to include them in the definition of the energy consumption of virtual compute resources/VNFC/VNFs.

### 4.1.4	Recommendation

It is proposed to introduce the potential solution #1 into TS 28.554 – clause 6.7.3.1.

End of changes