3GPP TSG-SA5 Meeting #145-e	S5-225590

# Online, , 15th Aug 2022 - 24th Aug 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0384 | rev | - | Current version: | 17.7.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-17 CR for TS28.552 Correct Mean and Max Time of requested conditional handover executions |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | ePM_KPI_5G |  |  |  |  |  | Date: |  |  | 2022-08-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The measurements are broken. The sending of a "N2 Path Switch Request" message is from Target NG-RAN to AMF, not from Source NG-RAN to the Target NG-RAN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct start-time for measurements of Mean and Max Time of requested conditional handover executions |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The measurements are broken, cannot be measured correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.6.8.11, 5.1.1.6.8.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of changes

5.1.1.6.6.11	Mean Time of requested conditional handover executions

a)	This measurement provides the mean time of inter-gNB conditional handover executions during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by accumulating the time interval for every successful Inter-gNB handover executions procedure per S-NSSAI between the receipt by the Source NG-RAN from the Target NG-RAN of a "Release Resource" and the receipt by the source NG-RAN from the target NG-RAN of a “HANDOVER SUCCESS”over a granularity period using DER, for conditional handovers. The end value of this time will then be divided by the number of inter-gNB conditional handovers observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value (in milliseconds.)

e)	MM.ChoExeInterReq.TimeMean.SNSSAI

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of Inter-gNB handovers during the granularity period.

The second change

5.1.1.6.6.12	Max Time of requested conditional handover executions

a)	This measurement provides the max time of inter-gNB conditional handover executions during each granularity period. The measurement is split into subcounters per S-NSSAI.

b)	DER(n=1)

c)	This measurement is obtained by measuring the time interval for every successful Inter-gNB handover executions procedure per S-NSSAI between the receipt by the source NG-RAN from the target NG-RAN of a "Release Resource" and the receipt by the source NG-RAN from the target NG-RAN of a “HANDOVER SUCCESS”over a granularity period using DER, for conditional handovers. The high tide mark of this time will be stored in a gauge, the gauge shall be reinitialised at the beginning of each granularity period.

d)	Each measurement is an integer value (in milliseconds.)

e)	MM.ChoExeInterReq.TimeMax.SNSSAI

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

i)	One usage of this measurement is for monitoring the mean time of Inter-gNB handovers during the granularity period.

End of changes