3GPP TSG-SA5 Meeting #146	S5-226088

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.156 | CR | 0044 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify attributes and IOCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-10-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is confusion about the terminology regarding the modeling of attributes. It is not specified in TS 28.622 and TS 28.541 whether an IOC is optional or mandatory to support. It should be. Some references are poorly specified. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify the definition of attributes and optionality of IOCs. Improve references. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misunderstanding leads to interoperability problems for attributes and IOCs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.1, 5.2.1.1, 5.3.2, 5.3.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

### 5.2.1	Attribute

#### 5.2.1.1	Description

An attribute is a typed element representing a property of a class. See Unified Modelling Language (OMG UML), Infrastructure [1] clause 10.2.5.

An attribute is either supported by an implementation or not, following the rules of SupportQualifier for that attribute. Non-supported attributes have no meaning, cannot be created, deleted, changed or read and never have a value as if the attribute was never specified. Supported attributes have zero or more values. New values can be added and existing values can be changed or deleted.

The terminology 'creating' or 'deleting' an attribute is also used. Deleting an attribute means removing all its values;  creating an attribute means adding one or more values for an attribute that had no values previously.

An element that is typed implies that the element can only refer to a constrained set of values.

See Unified Modelling Language (OMG UML), Infrastructure [1] clause 10.1.4 for more information on attribute type.

See clauses 5.3.4 and 5.4.3 for predefined data types and user-defined data types that can apply type information to an element.

The following table captures the properties of this modelled element. Attribute properties for a specific attribute are valid and the same for all instances of that IOC.

Table 5.2.1.1-1: Attribute properties

| Property name | Description | Legal values |
| --- | --- | --- |
| documentation | Contains a textual description of the attribute. Should refer (to enable traceability) to the specific requirement. | Any |
| isOrdered | For a multi-valued multiplicity; this specifies if the values of this attribute instance are sequentially ordered. See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True, False (default) |
| isUnique | For a multi-valued multiplicity, this specifies if the values of this attribute instance are unique (i.e., no duplicate attribute values). See subclause 7.3.44 and its Table 7.1 of [2]. If the property is present for attributes with a multiplicity of greater than “1”, it shall be set to either “True” or “False”. It shall not be set to “N/A”. | True (default), False |
| isReadable | Specifies that this attribute can be read by the manager. | True , False (default), (see NOTE 3) |
| isWritable | Specifies that this attribute can be written by the manager under the conditions specified in Annex B. | True, False (default) (see NOTE 4) |
| type | Refers to a predefined (see subclause 5.4.3) or user defined data type (see section 5.3.4). See also subclause 7.3.44 of [2], inherited from StructuralFeature. | NA |
| isInvariant | Attribute value is set at object creation time and cannot be changed under the conditions specified in Annex B. | True, False (default) |
| allowedValues | Identifies the values the attribute can have. When specific values or restrictions are listed, this means that other values are not allowed, except if the attribute description explicitly states that other values are allowed. If other values shall be allowed the description should include the text: "isExtensible=True" | Dependent on type |
| isNotifyable | Identifies if a notification shall be sent in case of a value change (see Note 1, Note 2). | True (default), False |
| defaultValue | Identifies a value at specification time that is used at object creation time under conditions defined in Annex B. If there is no defined default value, the property shall be omitted from the attribute description or specified as ‘defaultValue: None.’. | No value (default) or a value that is dependent on allowedValues |
| multiplicity | Defines the number of values the attribute can simultaneously have. See subclause 7.3.44 of [2]; inherited from StructuralFeature. | See 5.2.8 Default is 1 |
| isNullable | Identifies if an attribute can carry no information. The implied meaning of carrying “no information” is context sensitive and is not defined in this Model Repertoire. | True, False (default) |
| supportQualifier | Identifies the required support of the attribute. See also subclause 6. | M, O (default), CM, CO, C |
| passedById | See Table 5.2.9.1-1: passedById property | True, False (default) |

NOTE 1: Whether a client/manager can receive the notification depends on a) if the client/manager has subscribed or registered for reception of such notification and b) if a notification mechanism is supported.

NOTE 2: If the attribute is a role-attribute and its property passedById is ‘False’, then changes in the navigable association target end instance do not trigger a notification.

NOTE 3: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to read.

NOTE 4: The value True is conditional on the presence of a mechanism for the MnS Producer to authenticate the MnS Consumer and verify whether the authenticated MnS Consumer is authorized to write.

Next change

### 5.3.2	<<InformationObjectClass>>

#### 5.3.2.1	Description

The <<InformationObjectClass>> is identical to UML class except that it does not include/define methods or operations.

A UML class represents a capability or concept within the system being modelled. Classes have data structure and behaviour and relationships to other elements.

This class can inherit from zero, one or multiple classes (multiple inheritances).

See more on UML class in 10.2.1 of [1].

An <<InformationObjectClass>> is always optional to support unless specified otherwise. An <<InformationObjectClass>> may be supported or not-supported differently in different name containment relationships (see clause 5.3.3).

Name containment will dictate support for the source IOC. E.g. if an IOC-A (source) name contains IOC-B (target) and IOC-B is supported IOC-A shall also be supported.

End of  changes