3GPP TSG-SA5 Meeting #146 	S5-226468

# Toulouse, France, 14 -18 November 2022											Revision of S5-20xxxx

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.254 | CR | 0027 | rev | - | Current version: | 16.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the notifyURI for NEF Charging |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16 |  |  |  |  |  | Date: |  |  | 2022-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | For the NEF converged charging, only the IEC, PEC and ECUR are supported. The notifyURI is not appliable. The notifyURI and Triggers in the detailed message format for converged charging should be “-”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the notifyURI in the charging data request. Correct the Triggers in the charging data response. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The detailed message format for converged charging is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2a.1.2.1,6.2a.1.2.2,6.3.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.2a.1.2.1	Charging Data Request message

Table 6.2a.1.2.1.1 illustrates the basic structure of a Charging Data Request message as used for NEF converged charging.

Table 6.2a.1.2.1.1: Charging Data Request message contents

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57], and holds the identifier of the AF |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | This field indicates, if included, that this is a one-time event and that there will be no update or termination. |
| One-time Event Type | OC | Described in TS 32.290 [57]. |
| Notify URI | - | This field is not applicable |
| Triggers | OC | This field is described in TS 32.290 [57] and holds the NEF specific triggers described in clause 5.x |
| Multiple Unit Usage | OC | This field contains the parameters for the quota management request and/or usage reporting. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| Used Unit Container | OC | Described in TS 32.290 [57] |
| NEF API Charging Information | OM | This field holds the NEF API specific information described in clause 6.3.1.4 |

Next change

6.2a.1.2.2	Charging Data Response message

Table 6.2a.1.2.2.1 illustrates the basic structure of a Charging Data Response message as used for NEF converged charging.

Table 6.2a.1.2.2.1: Charging Data Response message content

| Information Element | Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | Described in TS 32.290 [57] |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | OC | This field holds the parameters for the quota management information. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |

Next change

6.3.4	Detailed message format for converged charging

The Operation types are listed in the following order: I [Initial] / U (Update)/T [Termination]/E [event]. Therefore, when all Operation types are possible it is marked as IUTE. If only some Operation types are allowed for a node, only the appropriate letters are used (e.g. IUT or E) as indicated in the table heading. The omission of an Operation type for a particular field is marked with "-" (e.g. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.3.4.1 illustrates the basic structure of the supported fields in the Charging Data Request for exposure function API online charging.

Table 6.3.4.1: Supported fields in Charging Data Request message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/T/U/E |
| Session Identifier |  | ITE |
| Subscriber Identifier |  | ITE |
| NF Consumer Identification |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Sequence Number |  | ITE |
| Retransmission Indicator |  | --- |
| One-time Event |  | --E |
| One-time Event Type |  | --E |
| Notify URI |  | --- |
| Triggers |  | ITE |
| Multiple Unit Usage |  | ITE |
| NEF API Charging Information |  |  |
| External Individual Identifier |  | ITE |
| External Group Identifier |  | ITE |
| Internal Group Identifier |  | ITE |
| API Direction |  | ITE |
| API Target Network Function |  | ITE |
| API Result Code |  | ITE |
| API Name |  | ITE |
| API Reference |  | ITE |
| API Content |  | ITE |

Table 6.3.4.2 illustrates the basic structure of the supported fields in the Charging Data Response for exposure function API converged charging.

Table 6.3.4.2: Supported fields in Charging Data Response Message

| Information Element | Node Type | NEF |
| --- | --- | --- |
|  | Supported Operation Types | I/U/T/E |
| Session Identifier |  | ITE |
| Invocation Timestamp |  | ITE |
| Invocation Result |  | ITE |
| Invocation Sequence Number |  | ITE |
| Session Failover |  | ITE |
| Triggers |  | --- |
| Multiple Unit Information |  | I-E |

End of change