| 3GPP TR 32.847 V1.6.0 (2023-01) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects Study on Charging Aspects for Network Slicing Phase 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Intpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	10

2	References	10

3	Definitions of terms, symbols and abbreviations	11

3.1	Terms	11

3.2	Symbols	11

3.3	Abbreviations	11

4	General background	12

4.1	Background from GSMA	12

4.2	Background from 3GPP network architecture	13

4.3	Network Slice Management Charging	13

4.4	Network Slice Performance and Analytics Charging	14

4.5	Different types of maximum	14

5	Potential charging requirements	14

6	Key Issues	15

6.1	Key Issue #1: Converged Charging for simultaneous number of UEs/max. number of UEs per network slice	15

6.1.0	General Description	15

6.1.1	Solution#1.1 New NetworkSliceConvergedCharging service to NSACF – quota request beyond "max Nb of Reg UEs"	16

6.1.1.1	Procedure Description	16

6.1.1.2	Flow description	17

6.1.2	Solution#1.2 NSACF (CTF) - Event based charging	19

6.1.2.1	General	19

6.1.2.2	Charging information	19

6.1.2.3	Trigger mechanism	19

6.1.2.4	Architecture description	19

6.1.2.5	Flow description	19

6.1.3	Solution#1.3 NSACF (CTF) - Session based charging	20

6.1.3.1	General	20

6.1.3.2	Charging information	21

6.1.3.3	Trigger mechanism	21

6.1.3.4	Architecture description	21

6.1.3.5	Flow description	21

6.1.3.6	Charging scenarios	23

6.1.4	Solution#1.4 NSACF and CEF - Event based charging	24

6.1.4.1	General	24

6.1.4.2	Architecture description	25

6.1.4.3	Flow description	25

6.1.5	Solution#1.5 NSACF and CEF - Session based charging	26

6.1.5.1	General	26

6.1.5.2	Architecture description	26

6.1.5.3	Flow description	26

6.1.6	Solution#1.6 New NSACF (CTF) – NS_quota management by CCS	27

6.1.6.1	General Description	27

6.1.6.2	Architecture Description	27

6.1.6.3	Flow description	27

6.1.7	Evaluation	29

6.1.8	Conclusion	30

6.2	Key Issue #2: Converged Charging for simultaneous number of PDU sessions/max. number of PDU sessions per network slice	31

6.2.0	General Description	31

6.2.1	Solution#2.1 NSACF (CTF) – Event based charging	31

6.2.1.1	General	31

6.2.2	Solution#2.2 NSACF (CTF) - Session based charging	32

6.2.2.1	General	32

6.2.3 	Solution#2.3 NSACF and CEF - Event based charging	32

6.2.3.1	General	32

6.2.4 	Solution#2.4 NSACF and CEF - Session based charging	32

6.2.5	Solution#2.5 NSACF (CTF) – NS quota management	32

6.2.6	Evaluation	32

6.2.7	Conclusion	33

6.3	Key Issue #3: Volume based Converged Charging per network slice	33

6.3.1	General Description	33

6.3.2	Solution#3.1 Volume aggregation by NS Tenant CCS	33

6.3.2.1	General description	33

6.3.2.2	Architecture description	34

6.3.2.3	Flow description (Tenant specific S-NSSAI)	35

6.3.2.4	Flow description (Shared S-NSSAI)	37

6.3.3	Solution#3.2 Volume control by CCS based on performance collection	37

6.3.3.1	General description	37

6.3.3.2	Architecture description	38

6.3.3.3	Flow description	38

6.3.4	Evaluation	39

6.3.5	Conclusions	40

6.4	void	40

6.5	void	40

6.6	Key Issue #6: UE Converged Charging based on network slice charging	40

6.6.1	General Description	40

6.6.2	Solution#6.1 NS Tenant CCS separated from UE CCS	41

6.6.2.1	General description	41

6.6.2.2	Architecture description	42

6.6.2.3	Flow description	42

6.6.3	Solution#6.2  NWDAF based Network slice charging	45

6.6.3.1	General description	45

6.6.3.2	Architecture description	46

6.6.3.3	Flow description	46

6.6.4	Solution#6.3  NSACF based Network slice charging – CEF based	46

6.6.4.1	General description	46

6.6.4.2	Architecture description	46

6.6.4.3	Flow description	47

6.6.5	Solution#6.4  NSACF based Network slice charging – CTF based	51

6.6.5.1	General description	51

6.6.5.2	Architecture description	51

6.6.5.3	Flow description	52

6.6.6	Evaluation	53

6.6.7	Conclusion	53

6.7	Key Issue #7: Converged Charging for actual duration per network slice	54

6.7.0	General Description	54

6.7.1	Solution#7.1 Duration determination by NS Tenant CCS	54

6.7.1.1	General description	54

6.7.1.2	Architecture description	54

6.7.1.3	Flow description	54

6.7.2	Solution#7.2  Network slice actual duration charging from the SMF	56

6.7.2.1	General description	56

6.7.2.2	Architecture description	56

6.7.2.3	Flow description	56

6.7.3	Solution#7.3  Network slice actual duration charging from the NSACF	57

6.7.3.1	General description	57

6.7.3.2	Architecture description	57

6.7.3.3	Flow description	57

6.7.4	Solution#7.4  Network slice actual duration charging from the NWDAF	59

6.7.4.1	General description	59

6.7.4.2	Architecture description	59

6.7.4.3	Flow description	59

6.7.5	Evaluation	60

6.7.6	Conclusions	60

6.8	Key Issue #8: Converged Charging for NSSAA	60

6.8.1	General Description	60

6.8.2	Solution#8.1 Converged Charging for NSSAA	61

6.8.2.1	General description	61

6.8.2.2	Architecture description	61

6.8.2.3	Flow description	61

6.8.3	Solution#8.2 Converged Charging for NSSAA from AAA-P	63

6.8.3.1	General description	63

6.8.3.2	Architecture description	63

6.8.3.3	Flow description	63

6.8.4	Evaluation	65

6.8.5	Conclusion	65

6.9	Key Issue #9: Service based interface between UE CCS and Tenant CCS	65

6.9.1	General Description	65

6.9.2	Solution #9.1 Reusing Nchf_ConvergedCharging service between UE CHF and Tenant CHF	65

6.9.2.1	General description	65

6.9.2.2	Architecture description	66

6.9.2.3	Flow description	66

6.9.3	Solution #9.2 New charging service API between UE CHF and Tenant CHF	66

6.9.3.1	General description	66

6.9.3.2	Architecture description	66

6.9.3.3	Flow description	66

6.9.4	Evaluation	67

6.9.5	Conclusion	67

7	Conclusions and recommendations	67

Annex A (informative): Change history	68

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, certain modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" shall not to be used as substitutes for "may" and "need Not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document continues the study on charging aspects of network slicing covered by 3GPP TR 32.845 [2] but no longer limited to charging data collection functionalities without quota management as specified for Network Slice management charging in 3GPP TS 28.202 [3] and for Network slice performance and analytics charging in 3GPP TS 28.201 [4].

The following is studied:

-	Network slice converged charging for Network Slices enabled over 3GPP 5GS;

-	Correlation aspects between Network Slice charging and the end user charging accessing the Network Slice;

-	Inter-Provider (B2B) charging aspects by considering the different Organizations (e.g. MNO, Verticals).

The Converged Charging service aspects associated to the 3GPP TR 23.700-40 [5] conclusion on network slice information is also considered.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 32.845: "Charging management; Study on charging aspects of network slicing”.

[3]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2".

[4]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); Stage 2".

[5]	3GPP TR 23.700-40: "Study on enhancement of network slicing; Phase 2”.

[6]	GSMA 5GJA NG.116: "Generic Network Slice Template".

[7]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[8]	3GPP TS 28.530: "Management and orchestration; Concepts, use cases and requirements".

[9]	3GPP TS 32.255: "Telecommunication management; Charging management; 5G data connectivity domain charging; Stage 2".

[10]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[13]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[14]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI) Generic management services".

[15]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[16]		3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[17]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM);Stage 2 and stage 3".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GJA		5G Joint Activity (Initiative of NG)

AMF	Access and Mobility Management Function

CEF	Charging Enablement Function

CHF	Charging Function

CSC	Communication Service Customer

CSP	Communication Service Provider

EPS	Evolved Packet System

GSM	Global System for Mobile Communications

GSMA	GSM Association

GST	Generic Network Slice Template

MnS	Management Service

NEST	Network Slice Types

NG	Networks Group (GSMA WG)

NOP	Network Operator

NS	Network Slice

NSC	Network Slice Customer

NSI	Network Slice Instance

NSP	Network Slice Provider

NSSAA	Network Slice-specific Authentication and Authorization

NSSAAF 	Network Slice-Specific Authentication and Authorization Function

NWDAF	Network Data Analytics Function

PDU	Protocol Data Unit

SMF	Session Management Function

S-NSSAI	Single Network Slice Selection Assistance Information

# 4	General background

## 4.1	Background from GSMA

GSMA 5GJA has specified in NG.116 [6] the concept of Generic Network Slice Template (GST) from which several Network Slice Types (NESTs) can be derived by assigning values to applicable attributes defined in the GST.

As per 3GPP TS 23.501 [7] is the network slice a logical network that provides specific network capabilities and network characteristics, and GSMA tailored the network slice based on the specific requirements adhered to a Service Level Agreement (SLA) agreed between Network Slice Customer (NSC) and Network Slice Provider (NSP).

Multiple roles related to network slicing are defined by GSMA and displayed in figure 4.1-1, based on some roles specified in 3GPP TS 28.530 [8] and described in 3GPP TR 32.845 [2].

The Charging model will fullfill the following used roles:

-	Communication Service Customer (CSC): Uses communication services, e.g. end user, tenant, vertical.

-	Communication Service Provider (CSP): Provides communication services. Designs, builds and operates its communication services. The CSP provided communication service can be built with or without network slice.

-	Network Operator (NOP): Designs, builds and operates networks and provides related services, including network services and network slices.

-	Network Slice Customer (NSC): The CSP or CSC who uses Network Slice as a Service.

-	Network Slice Provider (NSP): The CSP or NOP who provides Network Slice as a Service.

Depending on the scenarios an organisation can play one or several roles simultaneously, and apply charging models based on corresponding business relationship, e.g.:

-	Charging models for CSP to charge CSC for using communication services

-	Charging models for NOP to charge CSP for using their Networks and related Network services

-	Charging models for NOP playing NSP Role to charge CSP playing NSC Role for using their Networks and related network slices

Figure 4.1-1: Model roles in network slicing

The GST is a set of generic attributes that can characterise a type of network slice/service and is not tied to any specific network deployment.

The NEST is a GST filled with values. The attributes and their values are assigned to fulfil a given set of requirements derived from a network slice customer use case. The NEST is an input to the network slice preparation performed by NSP. NEST-related attributes can be differentiated by a Service Category.  For instance, when an attribute of a NEST includes a Service Category parameter and the UE subscription information includes a parameter value associated with that Service Category, then the UE will be subject to the differentiated service associated with the Service Category in the network slice.

Figure 4.1-2 places the GST and NEST in the context of the network slice lifecycle. As first step, the NSC provides the service requirements for the use case that it would like to fulfil, to the NSP. The NSP maps the service requirements into the attributes of the GST with the appropriate values generating a NEST.

Figure 4.1-2: GST and NEST in context of the network slice lifecycle

In GSMA 5GJA NG.116 [6], there are two attributes the maximum number of UEs and the maximum number of PDU sessions, which characterising the network slice (one S-NSSAI) and defined as the maximum number of UEs and the maximum number of PDU sessions that can use the network slice simultaneously.

The Network Slice Instance (NSI) concept is as used in 3GPP TS 28.202 [3].

## 4.2	Background from 3GPP network architecture

The architectural requirements per 3GPP TR 23.700-40 [5] are copied below to derive potential charging requirements:

-	Solutions have to build on the 5G System architectural principles as in 3GPP TS 23.501 [7], including flexibility and modularity for newly introduced functionalities.

-	Subject to operator policy and national/regional regulations, solutions have to be able to exempt Emergency services, Multimedia Priority Service, and Mission Critical Service from count limits.

NOTE:	Handling of UE/PDU Session count exemption and any pre-emption to other non-mission critical services is expected to be controlled by operator policy.

-	While the support of network slicing applies only to 5GS, solutions supports subscriptions that allows EPS/5GS interworking. Therefore EPS/5GS interworking needs to be considered for the enforcement of counts for maximum number of UEs per network slice and for maximum number of PDU Sessions per network slice."

## 4.3	Network Slice Management Charging

Network Slice Management Charging specified by 3GPP TS 28.202 [3], enables the Converged Charging System (CCS) to capture for each Network Slice instance per 3GPP TS 28.530 [8] definition, the supported list of service profiles representing the Service Level Specification (SLS) between Network Slice Customer (NSC) and Network Slice Provider (NSP).

For each S-NSSAI, associated attributes and their values representing the Service Level Specification (SLS) are available to the Converged Charging system, via service profiles captured with the NetworkSlice instance.

These information are captured in NSM charging CHF record generated by CHF upon NetworkSlice instance creation, modification and termination.

## 4.4	Network Slice Performance and Analytics Charging

Network Slice Performance and Analytics Charging specified by 3GPP TS 28.201 [4], enables the Converged Charging System (CCS) via CEF (Charging Enablement Function) to obtain the performance and analytics information from MnS procedure specified in 3GPP TS 28.532 [13] and/or NWDAF described in 3GPP TS 23.288 [12].

The charging information (e.g. Latency, Number of PDU sessions and Number of registered Subscribers) is collected per Single Network Slice Selection Assistance Information (S-NSSAI). NSPA charging CHF record generated by CHF for each network slice (i.e. S-NSSAI).

The definition of KPI measurement is present as the following:

- The number of PDU sessions: PDUSesMaxNbr KPI describes the maximum number of PDU sessions that are successfully established in a network slice, as specified in 3GPP TS 28.554 [14] clause 6.4.5. The measurement of PDUSesMaxNbr is obtained by sampling at a pre-defined interval, the number of PDU sessions established by SMF (SM.SessionNbrMax.SNSSAI), and then selecting the maximum value, as specified in the 3GPP TS 28.552 [15] clause 5.3.1.2.

- The number of registered Subscribers: AMFMaxRegNbr KPI describe the maximum number of subscribers that are registered to a network slice, as specified in 3GPP TS 28.554 [14] clause 6.2.6.The measurement of AMFMaxRegNbr is obtained by sampling at a pre-defined interval the number of registered subscribers in an AMF (RM.RegisteredSubNbrMax.SNSSAI) and then taking the maximum, as specified in the 3GPP TS 28.552 [15] clause 5.2.1.2.

## 4.5	Different types of maximum

There are today three definitions of the maximum:

- In the GSMA GST, GSMA 5GJA NG.116 [6], it is possible to specify the maximum number UEs and PDU sessions allowed to of simultaneous use a network slice, also aligned with the OAM, 3GPP TS 28.541[17].

- The NSACF, 3GPP TS 23.501 [7] clause 5.15.11, allows for monitoring and controlling the number of registered UEs per network slice and the number of PDU Sessions per network slice.

- The OAM, 3GPP TS 28.554 [14] clauses 6.2.6 and 6.4.5, measures the number of UEs or PDU sessions on a network slice at specific interval and may give the maximum value of these.

From this it can be noticed that the GSMA and NSACF are aligned in the definition as it may control the maximum as well as monitoring it, the OAM provided 5G end to end Key Performance Indicators (KPI) in TS 28.554 [14] is more of a measurement where samples are taken at certain interval and after that there may be a maximum or median value calculated for a period. This means in the OAM case that the values may have been higher than the reported one if the sample was taken before and after a peak. OAM Service profile in TS 28.541[17] is the configuration of the network slice which can be obtained by NSACF and CHF.

When the concept of "maximum" is used in this specification, the GSMA/NSACF definition and service profile received by OAM is assumed.

# 5	Potential charging requirements

REQ-NSCH-01	The 5G charging framework should support converged charging per S-NSSAI.

REQ-NSCH-02	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs".

REQ-NSCH-03	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions".

REQ-NSCH-04	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", exempting Emergency Registered UEs

REQ-NSCH-05	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", exempting Emergency services, Multimedia Priority Service, and Mission Critical Service.

REQ-NSCH-06	The 5G charging framework should support converged charging per S-NSSAI, based on "number of UEs", including UEs connected in the EPS.

REQ-NSCH-07	The 5G charging framework should support converged charging per S-NSSAI, based on "number of PDU sessions", including PDN connections in the EPS.

REQ-NSCH-08	The 5G charging framework should support converged charging per tenant based on assigned group of S-NSSAI(s).

REQ-NSCH-09	The 5G charging framework should support converged charging per S-NSSAI, based on "aggregated PDU sessions volume".

REQ-NSCH-10	void

REQ-NSCH-11 The 5G charging framework should support individual UE converged charging based on Network Slice usage charging criteria.

REQ-NSCH-12	The 5G charging framework should support converged charging per S-NSSAI for the time an S-NSSAI is used.

REQ-NSCH-13 The 5G charging framework should support converged charging for UE Network Slice-specific Authentication and Authorization (NSSAA) invocation towards a 3rd party for network slice access.

# 6	Key Issues

## 6.1	Key Issue #1: Converged Charging for simultaneous number of UEs/max. number of UEs per network slice

### 6.1.0	General Description

This key issue is for investigating how to support network slice access based converged charging service on simultaneous/max. number of UEs, considering REQ-NSCH-01, REQ-NSCH-02, REQ-NSCH-04, REQ-NSCH-06.

The maximum number of UEs is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as the maximum number of UEs that can use the network slice simultaneously. The max. number of UEs that can use the network slice simultaneously is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging. Since the maximum number of simultaneous UEs can part of the offering and in the extension part of the price for the offering, then also the number of UEs that use the network slice simultaneously can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of simultaneous UEs one price and if it does go above another price.

This investigation covers the following:

-	determination of "number of UEs"/max. number of UEs per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to "number of UEs"/max. number of UEs;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on "number of UEs"/max. number of UEs;

- 	determination of all needed charging information for online and offline charging methods.

For the sake of simplification, "number of UEs" refers to "number of UEs that can use the network slice simultaneously".

### 6.1.1	Solution#1.1 New NetworkSliceConvergedCharging service to NSACF – quota request beyond "max Nb of Reg UEs"

#### 6.1.1.1	Procedure Description

This solution addresses the Key Issue #1 by considering the new 5GC NF which has been defined as Network Slice Admission Control Function (NSACF).  While NSACF counts all UE registrations in S-NSSAI as received from the AMF(s), the CHF/5G Converged Charging System functionalities provide the charging service and quota management capabilities to this NSACF.

The CHFreceives and stores the "Maximum Number of UEs"  via the ConvergedCharging service, as specified in 3GPP TS 28.202 [3] clause 5.1.3 "Network Slice Management charging information" and 6.2.1.3 Definition of Service profile charging information. The "max. Nb of Reg UEs" is used in the following clause.

As described in TS 23.501 [7] clause 5.15.11.0, , the NSACF is configured with "max Nb of Reg UEs".

A new NetworkSliceConvergedCharging service is introduced for this Network Slice quota management between the CHF and the NSACF and its use is described in following clause. In this context, the Network Slice quota (NS_quota) handled by the CHF relates to quota of "Nb of Reg UEs", with a CCS defined maximum referred to as "NS Reg UEs Quota limit".

In this solution, the "NS Reg UEs Quota limit" may be above the "max Nb of Reg UEs".

During the UEs registration, the NSACF provides the network slice admission control for all AMF(s) by counting the number of registered UEs against the "max Nb of Reg UEs", and once this maximum is reached NSACF will trigger a request for more quota under NetworkSliceConvergedCharging service to CHF.

The following figure 6.1.1.1-1 represents the high-level entities involved in the solution.

Figure 6.1.1.1-1 High level functional solution overview

#### 6.1.1.2	Flow description

The figure 6.1.1.2-1 describes the high-level charging procedure for Network slice converged charging based on NS_quota handling by CHF.

Figure 6.1.1.2-1: NetworkSlice quota management - registration

The steps are with the following additions:

0ch. CHF gets per S-NSSAI the "max Nb of Reg UEs" obtained during NSI creation and determines the "NS Reg UEs Quota limit".

0. The NSACF counts the “Nb of Reg UEs” and check against the configured "max Nb of Reg UEs" value.

1. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Xch-a: At least one S-NSSAI has reached the configured "max Nb of Reg UEs" value.

Xch-b: the NSACF invokes the Nchf_NSConvergedCharging service for requesting more NS_quota of Nb of Reg UEs for the S-NSSAI.

Xch-c. Account and rating control for the S-NSSAI. The S-NSSAI for which the configured value of  “max Nb of Reg UEs” is reached will be either granted with more NS_quota or the request for more NS_quota is indicated as rejected by the CCS.

Xch-d. CHF provides response to NSACF with “NS_Quota” for S-NSSAI(s) based on the status of step Xch-c.

2. NS admission control for UEs registrations per 3GPP TS 23.502 [11].

Steps Xch-a to Xch-d can be repeated until the "NS Reg UEs Quota limit" is reached.

### 6.1.2	Solution#1.2 NSACF (CTF) - Event based charging

#### 6.1.2.1	General

Event based charging where the thresholds values for simultaneous number of UEs are set separately for going below (downwards) or above (upwards). This means that when a UE is added to the network slice and it at that point passes a upwards threshold it will trigger charging, and when a UE is removed from a network slice and it at this point passes a downwards threshold it will trigger charging. There may be several upwards triggers but only one downwards.

#### 6.1.2.2	Charging information

The minimum information needed to be able to charge for the number of simultaneous UE connected to a network slice can be found in table 6.1.2.2-1.

Table 6.1.2.2-1: Minimum charging information of simultaneous number of UEs

| Information Element | Description |
| --- | --- |
| Identifier of Network Slice | This field identifies Network Slice and can be the Network Slice Instance or the S-NSSAI. |
| Trigger | This field identifies the passing of the threshold upwards or downwards for simultaneous number of UEs |
| Trigger Timestamp | This field holds the time when the threshold was passed |
| Number of simultaneous UEs | This field holds the number of simulations UEs after the passing of the threshold |

#### 6.1.2.3	Trigger mechanism

Thresholds may be set at provisioning of the network slice together with other network slice profiled information and updated by configuration changes.

#### 6.1.2.4	Architecture description

Figure 6.1.2.4-1: NSACF (CTF) converged charging architecture

#### 6.1.2.5	Flow description

The figure 6.1.2.5-1 describes the high-level charging procedure for Network slice converged charging based on NSACF (CTF) using Nchf_NSConvergedCharging service.

Figure 6.1.2.5-1: NSACF (CTF) and Nchf_NSConvergedCharging service event based

0ch. CHF obtains per S-NSSAI the "max number of UEs" during NSI creation, corresponding to maximum value of NS_quota for "Number of UEs" unit.

1. The NSACF is configured per S-NSSAI, with "max number of UEs" and a set of "Number of UEs" intermediate thresholds (downwards and upwards triggers) under the "max number of UEs".

2. NS admission control triggered from AMF per 3GPP TS 23.502 [11] clause 4.2.11 Number of UEs per network slice availability check and update procedure.

2ch-a: For a particular S-NSSAI one "Number of UEs" threshold is crossed or "max number of UEs" is reached.

2ch-b: The NSACF invokes the Nchf_NSConvergedCharging service for reporting used units "Number of UEs".

2ch-c. Account and rating control based on "Number of UEs" used units for the S-NSSAI.

2ch-d. CHF provides response to NSACF.

### 6.1.3	Solution#1.3 NSACF (CTF) - Session based charging

#### 6.1.3.1	General

Session based charging where the session is started and kept as long as the simultaneous number of UEs is above a certain value. This means that when a UE is added to the network slice and it at that point passes the threshold it will initiate charging, and when a UE is removed from a network slice and it at this point it again passes the threshold it will terminate charging. In between these the CHF may set new triggers.

#### 6.1.3.2	Charging information

The minimum information needed to be able to charge for the number of simultaneous UE connected to a network slice can be found in table 6.1.3.2-1.

Table 6.1.3.2-1: Minimum charging information of simultaneous number of UEs

| Information Element | Description |
| --- | --- |
| Identifier of Network Slice | This field identifies Network Slice and can be the Network Slice Instance or the S-NSSAI. |
| Trigger | This field identifies the passing of the threshold for simultaneous number of UEs |
| Trigger Timestamp | This field holds the time when the threshold was passed |
| Number of simultaneous UEs | This field holds the number of simulations UEs after the passing of the threshold |

#### 6.1.3.3	Trigger mechanism

The threshold may be set at provisioning of the network slice together with other network slice profiled information and updated by configuration changes. The CHF may provide new triggers during the charging session.

The following table 6.1.3.3-1 describes the set of thresholds and triggering conditions:

Table 6.1.3.3-1: trigger thresholds

| Thresholds | Trigger conditions | Allowed to be updated by CHF |
| --- | --- | --- |
| N1 | Number of simultaneous UEs for triggering [initial] when crossing upwards | No |
| N2 | Number of simultaneous UEs for triggering [update] when crossing upwards and downwards | yes |
| N3 | Number of simultaneous UEs for triggering [termination] when crossing downwards | yes |

#### 6.1.3.4	Architecture description

Same architecture as clause 6.1.2.4.

#### 6.1.3.5	Flow description

The figure 6.1.3.5-1 describes the high-level charging procedure for Network slice converged charging based on NSACF (CTF) using Nchf_NSConvergedCharging service.

Figure 6.1.3.5-1: NSACF (CTF) and Nchf_NSConvergedCharging service session based

Steps 0ch to 1: same as flow in clause 6.1.2.5

2. NS admission control triggered from AMF per 3GPP TS 23.502 [11] per clause 4.2.11Number of UEs per network slice availability check and update procedure.

2ch-a: For a particular S-NSSAI one trigger occurs: one "Number of UEs" threshold is crossed for initial.

2ch-b: The NSACF invokes the Nchf_NSConvergedCharging service initial and reporting of used units " Number of UEs".

2ch-d. CHF provides response to NSACF, with possible new triggers.

3. NS admission control triggered from AMF per 3GPP TS 23.502 [11] Number of UEs per network slice availability check and update procedure.

3ch-a: For a particular S-NSSAI one trigger occurs: one " Number of UEs" threshold is crossed for update or termination.

3ch-b: The NSACF invokes the Nchf_NSConvergedCharging service update or release and reporting of used units " Number of UEs".

3ch-c. Account and rating control.

3ch-d. CHF provides response to NSACF, with possible new triggers for the next step.

#### 6.1.3.6	Charging scenarios

Determination of passing a threshold for a network slice using Session based charging (SCUR).

Figure 6.1.3.6-1: SCUR – UE addition or removal to network slice

0) Threshold configured as initial trigger: N1.

1)	UE to be added: A UE is to be added to the network slice.

2)	Trigger evaluated: the NSACF (CTF) determines that the addition of the UE would cause the N1 threshold to be passed.

3)	Charging Data Request [Initial]: the NSACF (CTF) starts the charging session by sending a request to the CHF for the passing of the threshold.

4)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

5)	 Create CDR: the CHF creates a CDR related to the service.

6)	Charging Data Response [Initial]: the CHF informs the NSACF (CTF) on the result of the request, the CHF may provide new thresholds indicated with corresponding operation "update or termination": e.g. N2 (update), N3 (termination).

7)	 UE added: The UE is added to the network slice.

8)	UE to be added or removed: a UE is to be added to or removed from the network slice.

9)	Trigger evaluated: the NSACF (CTF) determines that the change in number of UE would cause a CHF provided (e.g. N2) to be passed.

10)	Charging Data Request [Update]: the NSACF (CTF) updates the charging session by sending a request to the CHF for the passing of the threshold.

11)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

12)	 Update CDR: the CHF updates the CDR related to the service.

13)	Charging Data Response [Update]: the CHF informs the NSACF (CTF) on the result of the request. The CHF may provide new thresholds indicated with corresponding operation "update or termination" overriding step 6 triggers.

14)	 UE added: if the trigger was addition of UE then the UE is added to the network slice.

15)	UE removed: a UE is to be removed from the network slice.

16)	Trigger evaluated: the NSACF (CTF) determines that the removal of UE would cause the termination threshold trigger (e.g. N3) to be passed.

17)	Charging Data Request [Termination]: the NSACF (CTF) terminates the charging session by sending a request to the CHF for the passing of the threshold.

18)	Account, Rating Control: the CHF may use the information to rate and make deductions from customer's account balance.

19)	 Close CDR: the CHF closes the CDR related to the service.

20)	Charging Data Response [Termination]: the CHF informs the NSACF (CTF) on the result of the request.

### 6.1.4	Solution#1.4 NSACF and CEF - Event based charging

#### 6.1.4.1	General

The general description, charging information and triggers for this solution are the same as for solution#1.2. Use of Nchf_NSConvergedCharging service from CEF.

#### 6.1.4.2	Architecture description

Figure 6.1.4.2-1: NSACF converged charging architecture - CEF

#### 6.1.4.3	Flow description

The figure 6.1.4.3-1 describes the high-level charging procedure for Network slice converged charging based on CEF using Nchf_NSConvergedCharging service and NSACF service.

Figure 6.1.4.3-1: CEF and use of NSACF service

0ch and 1: same as clause 6.1.2.5 (with CEF instead of NSACF)

2. Use of Nnsacf_SliceEventExposure_Subscribe/Unsubscribe service exposed by NSACF to subscribe to be notified on " Number of UEs" intermediate thresholds or "max number of UEs" per 3GPP TS 23.502 [11] clause 4.15.3.2.10 flows and NSACF services.

3. Use of Nnsacf_SliceEventExposure_Notify received by CEF based on subscribed thresholds.

3ch-a to 3ch-d: same as steps 2ch-a to 2ch-d as clause 6.1.2.5 (with CEF instead of NSACF).

### 6.1.5	Solution#1.5 NSACF and CEF - Session based charging

#### 6.1.5.1	General

The general description, charging information and triggers for this solution are the same as for solution#1.3. Use of Nchf_NSConvergedCharging service from CEF

#### 6.1.5.2	Architecture description

Same architecture as clause 6.1.4.2.

#### 6.1.5.3	Flow description

The figure 6.1.5.3-1 describes the high-level charging procedure for Network slice converged charging based on CEF using Nchf_NSConvergedCharging service and NSACF service.

Figure 6.1.5.3-1: CEF and use of NSACF service – session based

0ch and 3: same as clause 6.1.4.3.

3ch-a to 3ch-d: same as 2ch-a to 2ch-d clause 6.1.3.5 with CEF instead of NSACF.

4. Nnsacf_SliceEventExposure_Notify received by CEF based on subscribed thresholds.

4ch-a to 4ch-d: same as 3ch-a to 3ch-d clause 6.1.3.5 with CEF instead of NSACF.

### 6.1.6	Solution#1.6 New NSACF (CTF) – NS_quota management by CCS

#### 6.1.6.1	General Description

This solution combines solution#1.3 and parts of solution#1.1 based on Network Slice quota management between the CHF and the NSACF, session based charging and intermediate thresholds.

The general description is the same as clause 6.1.1.1, with the following differences:

- the "CHF max Nb of Reg UEs" correspond to the "max Nb Reg UEs" stored in the CCS;

- the "NSACF max Nb of Reg UEs" correspond to the "max Nb Reg UEs" stored in the NSAC;

- the "NS Reg UEs Quota limit" is always below the "NSACF max Nb of Reg UEs";

The NSACF is configured at provisioning of the network slice, with "NSACF max Nb of Reg UEs" and at least one intermediate "Nb of Reg UEs" threshold below the "max Nb of Reg UEs" for triggering the CHF.

The CHF is configured at provisioning of the network slice, with "CHF max Nb Reg UEs" which is below or equal to the "NSACF max number of PDU sessions". When the "CHF max Nb Reg UEs" is met the CHF will reject any request from NSACF increasing the Nb Reg UEs.

During the charging session, the CHF may provide new intermediate thresholds which override existing ones, as described in solution #1.3, associated with trigger mechanism in clause 6.1.3.3.

In this solution #1.6, the NSACF may request for NS_quota of "Nb of Reg UEs" to be granted for the S-NSSAI:

- upon crossed threshold triggering initial;

- upon exceeding NS_quota previously supplied by CHF;

NS_quota supplied by CHF may be set to the "NS Reg UEs Quota limit", or to a lower value.

#### 6.1.6.2	Architecture Description

Same architecture as clause 6.1.2.4.

#### 6.1.6.3	Flow description

The flow and steps are similar as in clause 6.1.3.5 apply with CCS control of NS-quota and "NS Reg UEs Quota limit" as described in figure 6.1.6.3-1.

Figure 6.1.6.3-1: NSACF (CTF) – CCS control of NS_quota

Step 0ch: "NS Reg UEs Quota limit" under the "max Nb of Reg UEs" for S-NSSAI, determined by CCS,.

Step 1: NSACF configured with "max Nb of Reg UEs" and a set of intermediate thresholds per S-NSSAI.

Steps 2 to 2ch-a: same as figure 6.1.3.5-1

2ch-b: The NSACF invokes the Nchf_NSConvergedCharging service initial and request for NS_quota of "Nb of Reg UEs" to be granted for the S-NSSAI.

2ch-c. CHF Account and rating control, CHF determines NS_quota to be granted, based on "NS Reg UEs Quota limit".

2ch-d. CHF provides response to NSACF, with possible NS_quota.

2ch-e. NS_quota as new limit for NSACF.

Steps 3 to 3ch-d: same as figure 6.1.3.5-1

4. NS admission control procedure per 3GPP TS 23.502 [11].

4ch-a. NS_quota of "Nb of Reg UEs" will be exceeded with addition of a new UE.

4ch-b: The NSACF invokes the Nchf_NSConvergedCharging service update and request for more NS_quota.

4ch-c. CHF Account and rating control, additional NS_quota for 'Nb of Reg UEs" may be granted  or not by CHF.

4ch-d. CHF provides response to NSACF, with possible new NS_quota, or rejection of additional UE.

4ch-e. if supplied the new NS_quota applies, otherwise (i.e. rejection of additional UE) the previous NS_quota applies.

5. NS admission control procedure per 3GPP TS 23.502 [11].

### 6.1.7	Evaluation

This clause evaluates the solutions for Key issue#1, they are all based on the Nchf_ConvergedCharging service.

Solution #1.1 partially solves Key issue#1:

Pro:

- Use of CCS capability of Network Slice quota management to dynamically grant additional NS_quota of "Nb of Reg UEs", to go beyond the NSACF configured "max Nb of Reg UEs" (with CCS determined "NS Reg UEs Quota limit"), based on charging criteria.

Cons:

- Not possible to apply specific rate and costs for different ranges of "Nb of Reg UEs" (before the NSACF configured "NSACF max Nb of Reg UEs" is reached).

- Not possible to control NS_quota of "Nb of Reg UEs" below the configured "NSACF max Nb of Reg UEs".

- Allowing to go beyond "NSACF max Nb of Reg UEs", could lead to network performance issues due to unsufficient deployed capacity.

Solutions #1.2 and #1.4 partially solves Key issue#1:

Pro:

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs" (before the NSACF configured "NSACF max Nb of Reg UEs" is reached) using pre-configured static thresholds.

- Event-based to CHF (No charging session): simple implementation and low resource usage in the CHF and CTF.

Cons:

- Not possible to go beyond the NSACF configured "NSACF max Nb of Reg UEs" for the Network Slice.

- Not possible to control NS_quota of "Nb of Reg UEs" within the configured "NSACF max Nb of Reg UEs".

- Not possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

Solutions #1.3 and #1.5 partially solve Key issue#1:

Pro:

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs", under the NSACF configured "NSACF max Nb of Reg UEs".

- Possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

Cons:

- Not possible to go beyond the NSACF configured "NSACF max Nb of Reg UEs" for the Network Slice.

- Not possible to control NS_quota of "Nb of Reg UEs" within the configured "NSACF max Nb of Reg UEs".

- Charging session to CHF: in case of long NS lifetime, the corresponding charging session will be maintained for a long time if thresholds for termination are not crossed.

Solutions #1.5 (CEF) compared to solution #1.3 does not allow the CHF (see 6.1.3.6	Charging scenarios) to act before a UE is added or removed.

Solutions #1.6 incorporate solution#1.3 and fully solves Key issue#1:

Pro:

- Use of CCS capability of Network Slice quota management to dynamically grant additional NS_quota of "Nb of Reg UEs" within the "CHF max Nb of Reg UEs", based on charging criteria.

- Possible to apply specific rate and costs for different ranges of "Nb of Reg UEs", under the "CHF max Nb of Reg UEs".

- Possible to dynamically tune the ranges of "Nb of Reg UEs" for specific rate and costs.

- Avoid performance issues by remaining under "NSACF max Nb of Reg UEs".

Cons:

- Charging session to CHF: in case of long NS lifetime, the corresponding charging session will be maintained for a long time if thresholds for termination are not crossed.

For all solutions, the particular REQ-NSCH-06 is covered under the NSACF behavior which counts UEs connected in EPS when EPS counting is required. In this case, the network description of the solution relates to interworking with EPC procedures instead of 5GC procedures involving the AMF.

For all solutions, the particular REQ-NSCH-04 cannot be fulfilled: based on clause 5.15.11.0 of TS 23.501 [11], counts in the NSACF also include emergency registered UEs undifferentiated from registered UEs.

### 6.1.8	Conclusion

Based on clause 6.1.7 evaluation, the solution #1.6 (incorporating solution #1.3) in clause 6.1.6  is retained.

In order to cover the usual basic charging scenarios, it is also retained to have an event-based charging solution in addition.  Since the session-based solution #1.6 relies on NSACF (CTF) architecture of figure 6.1.2.4-1, the same architecture applies for the event-based solution, therefore solution #1.2 is retained.

It is concluded not to introduce in normative work for this release, the CCS enforcement of "NS Reg UEs Quota limit" beyond the configured "NSACF max Nb of Reg UEs" in the NSACF, described under solution #1.1.

It is concluded not to introduce in normative work for this release the new Nchf_NSConvergedCharging service, instead the existing Nchf_ConvergedCharging service will be re-used. Further evaluations will be needed for a different service to be specified.Therefore solutions #1.6 and #1.2 are concluded for normative work with Nchf_ConvergedCharging service.

## 6.2	Key Issue #2: Converged Charging for simultaneous number of PDU sessions/max. number of PDU sessions per network slice

### 6.2.0	General Description

This key issue is for investigating how to support network slice access based converged charging service. on concurrent number of PDU sessions/max. number of PDU sessions, considering REQ-NSCH-01, REQ-NSCH-03, REQ-NSCH-05, REQ-NSCH-07.

The maximum number of PDU sessions is one of the attributes in the GST documented in GSMA 5GJA NG.116 [6] characterising the network slice (one S-NSSAI) and defined as maximum number of simultaneous PDU supported by the network slice. The number of max. number of PDU sessions is one criteria for usage control of network slice, it is therefore an aspect to be considered for converged charging. Since the maximum number of simultaneous PDU sessions can be part of the offering and in the extension part of the price for the offering, then also the number of PDU sessions that use the network slice simultaneous can be a criterion for the charging (i.e. rate and cost). For example, if during a period it never goes above a specific number of simultaneous PDU sessions one price and if it does go above another price.

This investigation covers the following:

-	determination of number of simultaneous PDU sessions/max. number of PDU sessions per slice that can use the network slice simultaneously to be used as a criteria for converged charging service;

-	identification of the main charging information to be collected in addition to number of simultaneous PDU sessions/max. number of PDU sessions;

-	determination of the main interactions with the NF or service usage to obtain the network slice authorization and usage based on max number of PDU sessions;

- 	determination of all needed charging information for online and offline charging methods.

### 6.2.1	Solution#2.1 NSACF (CTF) – Event based charging

#### 6.2.1.1	General

This solution is the same as solution#1.2 with:

-	"number of UEs" and "max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

-	NS admission control triggered from SMF (instead of AMF) per 3GPP TS 23.502 [11] clause 4.2.11 Number of PDU sessions per network slice availability check and update procedure.

### 6.2.2	Solution#2.2 NSACF (CTF) - Session based charging

#### 6.2.2.1	General

This solution is the same as solution#1.3 with:

-	"number of UEs" " and "max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

-	NS admission control triggered from SMF (instead of AMF) per 3GPP TS 23.502 [11] clause 4.2.11 Number of PDU sessions per network slice availability check and update procedure.

### 6.2.3 	Solution#2.3 NSACF and CEF - Event based charging

#### 6.2.3.1	General

This solution is the same as solution#1.4 with:

-	"number of UEs" and "max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

-	Use of Nnsacf_SliceEventExposure_Subscribe/Unsubscribe service exposed by NSACF to subscribe to be notified on " Number of PDU sessions" intermediate thresholds or "max number of PDU sessions"

### 6.2.4 	Solution#2.4 NSACF and CEF - Session based charging

This solution is the same as solution#1.5 with:

-	"number of UEs" and "max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

-	Use of Nnsacf_SliceEventExposure_Subscribe/Unsubscribe service exposed by NSACF to subscribe to be notified on " Number of PDU sessions" intermediate thresholds or "max number of PDU sessions".

### 6.2.5	Solution#2.5 NSACF (CTF) – NS quota management

This solution is the same as solution#1.6 with:

-	NS_quota management between the NSACF and CHF of solution#1.1 combined with solution #2.2;

-	"number of UEs" and " max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

- 	"NS Reg UEs Quota limit" replaced by "NS PDU sessions Quota limit";

-	NS admission control on number of PDU sessions triggered from SMF (instead of AMF) per 3GPP TS 23.502 [11] clause 4.2.11.4 Number of PDU sessions per network slice availability check and update procedure.

### 6.2.6	Evaluation

This clause evaluates the solutions for Key issue#2, they are all based on the Nchf_ConvergedCharging service.

The same 6.1.7 Evaluation applies, with following differences:

- Key issue#1 replaced by Key issue#2;

- The "number of UEs" and "max number of UEs" are replaced by "number of the PDU Sessions" and “max number of PDU Sessions”;

- "NS Reg UEs Quota limit" replaced by "NS PDU sessions Quota limit";

- Solution #2.1 instead of solution #1.2;

- Solution #2.2 instead of solution #1.3;

- Solution #2.3 instead of solution #1.4;

- Solution #2.4 instead of solution #1.5;

- Solution #2.5 instead of solution #1.6.

For all solutions, the particular REQ-NSCH-07 is covered under the NSACF behavior which counts PDN connections in EPS when EPS counting is required. In this case, the network description of the solution relates to interworking with EPC procedures instead of 5GC procedures involving the SMF.

For all solutions, the particular REQ-NSCH-05 cannot be fulfilled: based on clause 5.15.11.0 of TS 23.501 [11], counts in the NSACF also include emergency PDN connections undifferentiated from PDN connections.

### 6.2.7	Conclusion

The same 6.1.8 conclusion applies, with following differences:

- Clause 6.1.7 evaluation replaced by clause 6.2.6 evaluation;

- The "NSACF Nb of Reg UEs" replaced by "NSACF number of the PDU Sessions";

- "NS Reg UEs Quota limit" replaced by "NS PDU sessions Quota limit";

- Solution #2.1 instead of solution #1.2;

- Solution #2.5 instead of solution #1.6.

## 6.3	Key Issue #3: Volume based Converged Charging per network slice

### 6.3.1	General Description

This key issue is for investigating how to support network slice usage converged charging based on total PDU sessions volume per S-NSSAI.

The volume considered is the aggregated PDU sessions volume.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on volume;

-	determination of the main interactions required to obtain network slice usage information based on volume.

-	identification of the main charging information to be collected;

### 6.3.2	Solution#3.1 Volume aggregation by NS Tenant CCS

#### 6.3.2.1	General description

This solution addresses the Key Issue#3 for REQ-NSCH-09 potential requirement and is based on functional separation between NS Tenant Converged Charging System (CCS) and UE CCS with exchanges between them.

Two types of subscribers are considered in the context of Network Slice: the Tenant the Network Slice is assigned to, which is referred to as "NS-Tenant", and the individual end-user "UE" served under the Network Slice.

Each type of subscriber is handled under a dedicated Converged Charging System (CCS): "Tenant CCS" and "UE CCS" respectively.

"UE CCS" performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF hosting individual UE account balance, provisioned with all necessary information related to subscribed-to services the UE has paid for.

- The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

"Tenant CCS" performs charging functionalities for NS-Tenant, and includes in particular:

-	CHF exposing Charging service(s) associated to NS-Tenant.

-	ABMF hosting NS-Tenant account balance, provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for,

-	The subscription description includes in particular the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

How the account balance is provisioned with the subscriber's profile (for both NS-tenant and individual UE) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The UE 5G data connectivity converged charging from SMF is specified in 3GPP TS 32.255 [9] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE PDU session charging.

A new charging service exposed by the CHF (Tenant CCS) is defined for enabling association between individual UE charging and NS tenant charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the Tenant CCS.

In this solution, the volume is the charging information used for this association: when individual UE PDU session volume charging starts, the "UE allowed volume" as determined by UE-CCS is sent to the Tenant CCS, and the Tenant CCS is then able to aggregate UEs PDU sessions volume per S-NSSAI per Tenant. When the individual UE PDU session volume charging terminates, the "UE consumed volume" is sent to the Tenant CCS which recalculates the aggregation.

There is a possible option for intermediate reporting of "UE allowed volume" and "UE consumed volume" to the Tenant CCS for a better volume control by the Tenant CCS.

When Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same in the model roles of network slicing defined in clause 4.1, deployments can choose to collocate Tenant CCS with UE CCS.

#### 6.3.2.2	Architecture description

Figure 6.3.2.2-1: Distributed charging architecture in reference point representation.

ChX for interaction between CHF (UE CCS) and CHF (Tenant CCS)

Deployments can choose to collocate Tenant CCS with UE CCS.

#### 6.3.2.3	Flow description (Tenant specific S-NSSAI)

The figures below describe the high level charging procedure for Volume based Network slice converged charging using the distributed architecture when the S-NSSAI is exclusive by the tenant only.

Figure 6.3.2.3-1 describes the procedure when new UE PDU session starts:

Figure 6.3.2.3-1: Volume based Network slice converged charging – UE PDU session start

1.	NS tenant volume based charging per S-NSSAI covered by Tenant CCS

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.2 for individual UE charging covered by UE CCS.

9ch-b1.	UE PDU session volume charging has a dependency with NS volume charging for this S-NSSAI: determination to trigger ChX.

9ch-b2a.	Charging service request to NS Tenant CCS with S-NSSAI, UE PDU session charging information, Tenant ID (optional) and UE requested Network Slice Charging information including the "UE allowed volume" which is determined by UE-CCS.

9ch-b2b.	Account, Rating and reservation control for the S-NSSAI based on new S-NSSAI total volume (increased by "UE allowed volume" received in UE PDU session charging information).

9ch-b2c.	CHF (NS Tenant CCS) provides appropriate answer in Charging service response.

9ch-b3.	Account, Rating and reservation control for individual UE.

9ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.

Figure 6.3.2.3-2 describes the procedure when UE PDU session is released:

Figure 6.3.2.3-2: Volume based Network slice converged charging – UE PDU session release

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.4 for individual UE charging covered by NS UE CCS.

2ch-b1.	UE PDU session volume charging has a dependency with NS volume charging for this S-NSSAI

2ch-b2a.	Charging service request to NS Tenant CCS with S-NSSAI, UE PDU session charging information, and UE requested Network Slice Charging information including the " UE consumed volume."

2ch-b2b.	Account, Rating and reservation control for the S-NSSAI based on new S-NSSAI total volume recalculated from received UE consumed volume.

2ch-b2c.	CHF (NS Tenant CCS) provides appropriate answer in Charging service response.

#### 6.3.2.4	Flow description (Shared S-NSSAI)

The figures describes the high level charging procedure for Volume based Network slice converged charging using the distributed architecture when the tenants share the S-NSSAI is the same with the figure 6.3.2.3-1 and  6.3.2.3-2.

The procedure for new UE PDU session starts when the tenants share the S-NSSAI is the same with the figure 6.3.2.3-1 with the difference is descried as the following:

9ch-b1.	UE PDU session volume charging has a dependency with NS volume charging: determination to trigger ChX.

9ch-b2a.	Charging service request to Tenant CCS with S-NSSAI, Tenant ID, UE PDU session charging information, and UE requested Network Slice Charging information including the "UE allowed volume" which is determined by UE-CCS.

9ch-b2b.	Account, Rating and reservation control for the Tenant ID of the S-NSSAI based on new S-NSSAI total volume (increased by "UE allowed volume" received in UE PDU session charging information).

The procedure for UE PDU session release when the tenants share the S-NSSAI is the same with the figure 6.3.2.3-2 with the difference is descried as the following:

2ch-b2a.	Charging service request to NS Tenant CCS with S-NSSAI, Tenant ID (optional), UE PDU session charging information, and UE requested Network Slice Charging information including the "UE consumed volume."

2ch-b2b.	Account, Rating and reservation control for Tenant ID of the S-NSSAI based on new S-NSSAI total volume recalculated from received UE consumed volume.

2ch-b2c.	CHF (NS Tenant CCS) provides appropriate answer in Charging service response.

### 6.3.3	Solution#3.2 Volume control by CCS based on performance collection

#### 6.3.3.1	General description

This solution addresses the Key Issue#3 for REQ-NSCH-09 potential requirement and is based on volume control by CCS based on performance collection for a S-NSSAI via CEF.

The Tenant the Network Slice is assigned to, is referred to as "NS-Tenant" and handled under a Converged Charging System (CCS) "Tenant CCS" hosting NS-Tenant account balance.

This solution is based on 3GPP TS 28.201 [4] mechanisms, considering the total volume of aggregated PDU sessions per S-NSSAI is part of the Network slice performance and analytics (NSPA) charging information. The Mns Producer is responsible to collect volume from UPFs, and to aggregate per S-NSsAI.  and to notify on volume thresholds triggers supplied by CCS.

This solution is based on successive volume blocks reservation by CCS which are gradually debited based on volume collected under NSPA Charging.

During this mechanism, interaction with BSS takes place to either call for a recharge, notify the cost for recorded extra-volume, or revoke the NS-Tenant from using the S-NSSAI.

#### 6.3.3.2	Architecture description

The architecture for this solution is 3GPP TS 28.201 [4] Figure 4.2.2-1: Network slice performance and analytics converged charging architecture.

#### 6.3.3.3	Flow description

The following figure 6.3.3.3-1 describes the high level charging procedure for Volume control by CCS based on performance collection:

Figure 6.3.3.3-1: Volume control by CCS based on performance collection

1.	NS tenant volume based charging per S-NSSAI covered by Tenant CCS; account with maximum volume allowed Vm. Account, rating and reservation control for V1, V2, ... Vn (with V1+V2+..Vn < Vm ), with V1 V2, ... Vn also configured in CEF.

Steps according to 3GPP TS 28.201 [4] clauses 5.2.2.4 and 5.2.2.2:

2.	CEF configured with subscription conditions set to Volume thresholds: V1, V2, … Vn.

3. 	NSPA Charging (aggregated UPFs volume) using MnS with V1, V2, … Vn.

4. 	Account, Rating control for V1 to Vn-1.

5. 	NSPA Charging (aggregated UPFs volume until Vn is reached).

6.	Account, Rating for Vn and reserve additional blocks Vn+1, Vn+2 (with Vn+1, Vn+2 < Vm).

7.	Charging data response supplied with Vn+1 and Vn+2 thresholds.

8. 	NSPA Charging (aggregated UPFs volume until Vn+2 is reached).

9.	Account, Rating for Vn+2. No more volume can be reserved.

10.	Charging data response indicating "volume after Vn+2 to be considered in offline charging mode".

11. NSPA Charging for extra-volume under offline charging mode.

During the process BSS is notified by the CCS when the volume is approaching the allowed limit or at any time for taking appropriate action

### 6.3.4	Evaluation

Both solutions #3.1 and #3.2 solve Key Issue #3.

Solutions #3.1 allows to support network slice usage converged charging based on total PDU sessions volume per S-NSSAI aggregation by NS Tenant CCS interacting UE CCS during with individual UE charging.

Pros:

-	Same NF source (i.e. SMF) used for volume usage collection by both individual UE and aggregation

-	Good level of accuracy of NS volume usage

-	No need for a new NF in the signalling path for PDU session handling since it relies on existing individual UE PDU session charging with UE CCS.

-	When NS volume limit is reached, new PDU sessions can be rejected.

-	Approaching NS volume limit can be anticipated.

Cons:

-	high level signalling between NS tenant CHFs interfacing many UE CHFs (based on each CCS can have multiple CHFs).

-	NS tenant CCS has to maintain the actual NS total volume usage.

Solutions #3.2 allows to support network slice usage converged charging based on volume control by CCS based on performance collection for a S-NSSAI via CEF

Pros:

-	NS tenant CCS interfacing limited number of entities (aggregation handled by MnS producers)

-	No need for NS tenant CCS to maintain the actual NS total volume usage

Cons:

-	Different sources used for volume usage collection (i.e. SMF for individual UE and UPFs for NS tenant) resulting in different nature of volume being counted.

-	Risk of high mismatch between sum of individual UEs volume and NS total volume

-	Risk of significant delay between when NS volume was actually consumed and when account and rating control is processed for this volume.

-	No possibility  for immediate stop by NS tenant CCS when NS volume crosses the limit.

The major drawback of the solution #3.2 is to be more in "offline charging" fashion by the lack of "online charging" control capabilities on actual usage which can be better achieved by solution #3.1.

Drawbacks of the solution #3.1 can be resolved under appropriate deployment options. When Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same, deployments can choose to collocate Tenant CCS with UE CCS to simplify implementation. This deployment option is valid when the "Maximum Number of UEs" allowed to access the NS Tenant(s) S-NSSAI is not too high. For large deployments, the full solution#3.1 with separate UE CCS and NS Tenant CCS distributed architecture is more appropriate to enable multiple UE CCSs to interact with a particular NS Tenant CCS.

### 6.3.5	Conclusions

It is concluded for Key issue #3, to specify partial solution #3.1 in Rel-17 to cover the Combined UE CCS - Tenant CCS solution option for tenant specific S-NSSAI and Shared S-NSSAI for when the Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same. The internal structure for this combined CCS will not be detailed.

It is concluded for Key Issue #3 to specify full solution #3.1, with interface between UE CCS and Tenant CCS specified based on SID conclusion, per conclusion for Key Issue #x in clause 6.x.5.

## 6.4	void

## 6.5	void

## 6.6	Key Issue #6: UE Converged Charging based on network slice charging

### 6.6.1	General Description

This key issue is for investigating how to support UE Converged Charging influenced by network slice converged charging of the network slice serving the UE.

UE Converged Charging is based on criteria related to the Network Slice serving a UE. For example, the tenant has paid for a Network Slice allowing a large number of UEs and when the load level of network slice is low, wants some UE to be allowed with a lower tariff for using Network resources.

This investigation covers the following:

-	determination of which entity/entities is/are suitable to provide the network slice level of charging information for UE Converged Charging.

-	determination of the main interactions for UE Converged Charging to be influenced by network slice converged charging.

-	identification of the main charging information.

### 6.6.2	Solution#6.1 NS Tenant CCS separated from UE CCS

#### 6.6.2.1	General description

This solution addresses the Key Issue#6 for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is based on distributed charging architecture split between NS Tenant Converged Charging System (CCS) and UE CCS and exchanges between them.

Two types of subscribers are considered in the context of Network Slice: the Tenant the Network Slice is assigned to, which is referred to as "NS-Tenant", and the individual end-user "UE" served under the Network Slice.

Each type of subscriber is handled under a dedicated Converged Charging System (CCS): "Tenant CCS" and "UE CCS" respectively.

"UE CCS" performs charging functionalities for individual UEs, and includes in particular:

-	CHF exposing Nchf services associated to individual UEs.

-	ABMF hosting individual UE account balance, provisioned with all necessary information related to subscribed-to services the UE has paid for.

-	The subscription description includes in particular the subscribed NSSAI (set of S-NSSAI(s)).

"Tenant CCS" performs charging functionalities for NS-Tenant, and includes in particular:

-	CHF exposing Charging service(s) associated to NS-Tenant.

-	ABMF contains NS-Tenant account balance, provisioned with all necessary information related to the Network Slice matching SLS requirements the NS-Tenant has paid for,

- 	The subscription description includes in Particular. the list of service profiles subscribed-to by the NS-Tenant, each service profile with assigned group of S-NSSAI(s).

How the account balance is provisioned with the subscriber's profile (for both NS-Tenant and individual UE) and various subscribed-to services is unspecified and expected to be covered at BSS level.

The UE 5G data connectivity converged charging from SMF is specified in 3GPP TS 32.255 [9] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE PDU session charging.

The Registration management converged charging from AMF is specified in 3GPP TS 32.256 [10] and uses Nchf_ConvergedCharging service exposed by UE CCS for individual UE registration.

A new charging service exposed by the CHF (Tenant CCS) is defined for enabling association between individual UE charging and NS tenant charging. This association is achieved by UE CCS responsible for individual UE charging consuming this new charging service exposed by the Tenant CCS.

In this solution, the NS Tenant charging information used for individual UE charging can include a various set of information such as "Max nb of PDU sessions", "Nb of PDU sessions", "load level", "Max nb of registered UEs", "Nb of registered UEs".

When Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same in the model roles of network slicing defined in clause 4.1, deployments can choose to collocate Tenant CCS with UE CCS.

#### 6.6.2.2	Architecture description

Figure 6.6.2.2-1: Distributed charging architecture in reference point representation.

ChX for interaction between CHF (UE CCS) and CHF (Tenant CCS)

Deployments can choose to collocate Tenant CCS with UE CCS.

#### 6.6.2.3	Flow description

The figure 6.6.2.3-1 describes the high level charging procedure for UE PDU session converged Charging influenced by Network slice converged charging using distributed charging architecture.

Figure 6.6.2.3-1: UE PDU session converged Charging influenced by Network slice converged charging

1ch. NS Tenant NSM charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI in service profile: "Max nb of PDU sessions", "Max nb of UEs"…

2ch. NS Tenant NSPA based charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI KPIs.

Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.2 for individual UE charging covered by UE CCS.

9ch-b1.	UE PDU session charging needs input from NS charging for this S-NSSAI. Examples of requested input: "Max nb of PDU sessions", "Nb of PDU sessions", "load level"…

9ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE PDU session charging information, UE requested NS charging information).

9ch-b2b.	Based on received "UE requested NS charging information", Tenant CCS correlates with existing NS charging information for the S-NSSAI:  NSM charging information ("Max nb of PDU sessions"..), NSPA charging information ("Nb of PDU sessions (increased by one for considering the new PDU session)", "load level",..)…

9ch-b2c.	CHF (Tenant CCS) provides "UE supplied NS charging information" in Charging service response.

9ch-b3.	Account, Rating and reservation control for individual UE based on "UE supplied NS charging information".

9ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.

The figure 6.6.2.3-2 describes the high level charging procedure for UE registration converged Charging influenced by Network slice converged charging using distributed charging architecture.

Figure 6.6.2.3-2: UE Registration converged Charging influenced by Network slice converged charging

1ch. NS Tenant NSM charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI in service profile: "Max nb of PDU sessions", "Max nb of UEs…"

2ch. NS Tenant NSPA based charging per S-NSSAI covered by Tenant CCS: obtained S-NSSAI KPIs

Steps according to 3GPP TS 32.256 [10] clause 5.2.2.2.4 for individual UE registration charging covered by UE CCS.

16ch-b1.	Allowed S-NSSAI(s) in the Allowed NSSAI received for UE registration converged charging, needs input from NS charging. Examples for criteria: "Max nb of registered UEs", "Nb of registered UEs", "load level"…

16ch-b2. For each allowed S-NSSAI for which this association is needed:

16ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE registration charging information, UE requested NS charging information).

16ch-b2b.	Based on received "UE requested NS charging information", correlates with existing NS charging information for the S-NSSAI:  NSM charging information ("Max nb of UEs"..), NSPA charging information ("Nb of registered UEs (increased by one for considering the new registration)", "load level",..)…

16ch-b2c.	CHF (Tenant CCS) provides "UE supplied NS charging information" in Charging service response.

16ch-b3.	Account, Rating and reservation control for individual UE based on "UE supplied NS charging information".

16ch-c.	CHF (UE CCS) provides appropriate answer for the individual UE.

### 6.6.3	Solution#6.2  NWDAF based Network slice charging

#### 6.6.3.1	General description

This solution addresses the Key Issue#6 for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is similar as solution#6.1, where the NWDAF is used instead of Mns Producer for NS-Tenant Charging information.

The following set of Analytics of Table 7.1-2 of 3GPP TS 23.288 [12] are not relevant for criteria at "Network Slice" level:

-	NF Load information (NF level)

-	Network Performance information (Area of interest, RAN via OAM)

-	UE mobility information (UE level)

-	UE Communication information (UE level)

-	Expected UE behavioural parameters (UE level)

-	UE Abnormal behaviour information (UE level)

-	User Data Congestion information (Area of Interest)

-	QoS Sustainability (Area of Interest)

-	Session Management Congestion Control Experience (SMF level)

-	Redundant Transmission Experience (SMF level)

-	WLAN performance (WLAN level)

-	DN Performance (DN level)

-	Dispersion (UE level)

#### 6.6.3.2	Architecture description

Figure 6.6.3.2-1: NWDAF based Network slice charging

NOTE: 	The architecture part representing CEF/NWDAF can be part of architecture in 3GPP TS 32.240 [16] figure 4.2.4.1: Logical ubiquitous charging architecture for management domain.

#### 6.6.3.3	Flow description

The flows are the same as:

- Figure 6.6.2.3-1: UE PDU session converged Charging influenced by Network slice converged charging

- Figure 6.6.2.3-2: UE Registration converged Charging influenced by Network slice converged charging

With the difference the CEF (consumer of the NWDAF) is used for steps 1ch and 2ch, instead of MnS Producer/CEF for the purpose of NS charging collection of S-NSSAI charging information (KPIs...). Instead of  NSM charging information and NSPA charging information,  analytics which could be obtained from NWDAF are those of Table 6.2.1.3-1 in 3GPP TS 28.201[4] and "max Nb of UEs" is assumed as known by the NS Tenant CCS.

### 6.6.4	Solution#6.3  NSACF based Network slice charging – CEF based

#### 6.6.4.1	General description

This solution addresses the Key Issue#6 for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is based Network slice charging performed based on NSACF and CEF.

#### 6.6.4.2	Architecture description

Figure 6.6.4.2-1: NSACF based Network slice charging

#### 6.6.4.3	Flow description

The following figure 6.6.4.3-1 describes the high level charging procedure for UE PDU session Converged Charging influenced by Network slice converged charging based on NSACF.

Figure 6.6.4.3-1: UE PDU session converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.4.3-1 for solution #2.3 or Figure  6.1.5.3-1. for solution #2.4: CEF use of Nnsacf_SliceEventExposure_Subscribe/Unsubscribe service exposed by NSACF for getting notified on "number of PDU Sessions" intermediate thresholds or "maximum number of PDU Sessions" reported to CHF.

2. Same steps as Figure 6.6.2.3-1 between UE, SMF, UE CCF and Tenant CCS with:

9ch-b1.	UE PDU session charging needs NS charging information "Number of PDU sessions" and "maximum number of PDU Sessions" for this S-NSSAI.

9ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE PDU session charging information, UE requested NS charging information=" Number of PDU sessions" and "maximum number of PDU Sessions").

9ch-b2b.	Tenant CCS correlates received UE charging information with existing content of  NS charging information for the S-NSSAI for "Number of PDU sessions" and "maximum number of PDU Sessions"

9ch-b2c.	CHF (Tenant CCS) provides "Number of PDU sessions" and "maximum number of PDU Sessions" in Charging service response.

3. Account, Rating and reservation control for individual UE based on NS charging information "Number of PDU sessions" and "maximum number of PDU Sessions".

The following figure 6.6.4.3-2 describes the high-level charging procedure for UE Registration Converged Charging influenced by Network slice converged charging based on NSACF.

Figure 6.6.4.3-2: UE Registration converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.4.3-1 or Figure  6.1.5.3-1: CEF use of Nnsacf_SliceEventExposure_Subscribe/Unsubscribe service exposed by NSACF for getting notified on "number of ”UEs registered" intermediate thresholds or "maximum number of UEs" reported to CHF.

2. Same steps as Figure 6.6.2.3-2 between UE, AMF, UE CCF and Tenant CCS with:

16ch-b1.	Allowed S-NSSAI(s) in the Allowed NSSAI received for UE registration converged charging, needs NS charging information "Number of registered UEs" and "Maximum number of UEs".

16ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE registration charging information, UE requested NS charging information "Number of registered UEs" and "Maximum number of UEs" ).

16ch-b2b.	Tenant CCS correlates received UE charging information with existing content of NS charging information for the S-NSSAI for "Number of registered UEs" and "Maximum number of UEs".

16ch-b2c.	CHF (Tenant CCS) provides "Number of registered UEs" and "Maximum number of UEs" in Charging service response.

3. Account, Rating and reservation control for individual UE based on NS charging information "Number of registered UEs" and "Maximum number of UEs".

### 6.6.5	Solution#6.4  NSACF based Network slice charging – CTF based

#### 6.6.5.1	General description

This solution addresses the Key Issue#6 for REQ-NSCH-01, and REQ-NSCH-11 potential requirements and is based Network slice charging performed based on NSACF (CTF).

#### 6.6.5.2	Architecture description

Figure 6.6.5.2-1: NSACF (CTF) based Network slice charging

#### 6.6.5.3	Flow description

The following figure 6.6.5.3-1 describes the high level charging procedure for UE PDU session Converged Charging influenced by Network slice converged charging based on NSACF (CTF).

Figure 6.6.5.3-1: UE PDU session converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.6.3-1 referred-to for solution #2.5: Tenant CCS control of NS_quota under use of Nchf_ConvergedCharging service.

2. Same steps as Figure 6.6.2.3-1 between UE, SMF, UE CCF and Tenant CCS with:

9ch-b1.	UE PDU session charging needs NS charging information related to "Nb of PDU sessions" for this S-NSSAI.

9ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE PDU session charging information, UE requested NS charging information related to "Nb of PDU sessions").

9ch-b2b.	Tenant CCS correlates received UE charging information with existing content of  NS charging information for the S-NSSAI related to "Nb of PDU sessions" .

The following figure 6.6.5.3-2 describes the high-level charging procedure for UE Registration Converged Charging influenced by Network slice converged charging based on NSACF(CTF).

Figure 6.6.5.3-2: UE Registration converged Charging influenced by Network slice converged charging based on NSACF

1. Same steps as Figure 6.1.6.3-1: Tenant CCS control of NS_quota under use of Nchf_ConvergedCharging service .

2. Same steps as Figure 6.6.2.3-2 between UE, AMF, UE CCS and Tenant CCS with:

16ch-b1.	Allowed S-NSSAI(s) in the Allowed NSSAI received for UE registration converged charging, needs NS charging information related to "Nb of Reg UEs".

16ch-b2a.	Charging service request to Tenant CCS (S-NSSAI, UE registration charging information, UE requested NS charging information related to "Nb of Reg  UEs").

16ch-b2b.	Tenant CCS correlates received UE charging information with existing content of NS charging information for the S-NSSAI related to "Nb of Reg UEs"  ".

Editor”s Note: This solution is ffs.

### 6.6.6	Evaluation

All Solutions #6.1, #6.2 solve Key Issue #6, the one selected depends on which criteria are retained for NS charging to influence UE charging.These solutions can coexist and be invoked simultaneously.

### 6.6.7	Conclusion

It is concluded on solutions #6.1 (MnS solution), #6.2 (NWDAF solution) to be normative with interface between UE CCS and Tenant CCS per conclusion for Key Issue#6 in clause 6.6.5.

## 6.7	Key Issue #7: Converged Charging for actual duration per network slice

### 6.7.0	General Description

This key issue is for investigating how to support network slice usage converged charging based on actual duration per S-NSSAI.

The duration a network slice is in use can part of the offering and in the extension part of the price for the offering. The duration a network slice is in use is defined as the duration an S-NSSAI (used to identify the network slice) have at least one active PDU session.

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide the network slice usage information based on actual duration;

-	determination of the main interactions required to obtain network slice usage information based on actual  duration.

-	identification of the main charging information to be collected;

### 6.7.1	Solution#7.1 Duration determination by NS Tenant CCS

#### 6.7.1.1	General description

This solution addresses the Key Issue#7 for REQ-NSCH-12 potential requirement and is based on functional separation between NS Tenant Converged Charging System (CCS) and UE CCS as described for solution#3.1 and solution#6.1.

For a given S-NSSAI, the Tenant CCS determines the start when receiving the first request for a UE PDU session from a UE CCS, and determines the end when receiving the release of the last UE PDU session from a UE CCS.

When Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same in the model roles of network slicing defined in clause 4.1, deployments can choose to collocate Tenant CCS with UE CCS.

#### 6.7.1.2	Architecture description

Same architecture as clause 6.3.2.2.

#### 6.7.1.3	Flow description

The figure 6.7.1.3-1 below describes the high level charging procedure for Duration determination by NS Tenant CCS, in IEC scenario for the NS Tenant CCS.

Figure 6.7.1.3-1: Duration determination by Tenant CCS

1-2. The first UE PDU session request initiates PDU session charging

3. UE CCS sends a Charging service request [Event] to Tenant CCS with S-NSSAI, UE PDU session charging start.

4.	Tenant CCS receiving the first UE PDU session charging start, determines the start time for this S-NSSAI.

5.	Tenant CCS provides appropriate answer in Charging service response.

6. UE PDU session procedure continuation

7-9. A second UE PDU session request initiates UE PDU session charging to a UE CCS different from the first UE PDU session, which sends a Charging service request [Event] to Tenant CCS

10-11.	Tenant CCS receiving the second UE PDU session charging start for this S-NSSAI, increases the counter by one and provides appropriate answer in Charging service response.

12. The last UE PDU session release initiates PDU session charging release

13.	Tenant CCS receiving the last UE PDU session charging release, determines the end time for this S-NSSAI.

### 6.7.2	Solution#7.2  Network slice actual duration charging from the SMF

#### 6.7.2.1	General description

This solution addresses the Key Issue#7 for REQ-NSCH-12 potential requirement and is NS actual duration measured by CHF based on the PDU session information from the SMF.

#### 6.7.2.2	Architecture description

The architecture for this solution is present as the following.

Figure 6.7.2.2-1: the NS actual duration from SMF Reporting

#### 6.7.2.3	Flow description

The following figure 6.7.2.3-1 describes the high level charging procedure for NS actual duration based on the PDU session establishment from the SMF.

Figure 6.7.2.3-1: NS actual duration based on the SMF reporting

1.	NS actual duration charging per S-NSSAI covered by Tenant CCS. NS duration charging is configurated in the UE CCS and Tenant CCS, including the relationship between S-NSSAI/User ID and Tenant ID, etc.

2. 	Steps according to 3GPP TS 32.255 [9] clause 5.2.2.2.2 for individual UE charging covered by UE CCS, including the PDU session establishment and PDU Session release.

2ch-a.	SMF determinates to trigger charging service request to CHF (UE CCS) because the UE PDU session establishment and UE PDU session release of the S-NSSAI for the tenant.

2ch-b.	CHF (UE CCS) generates the UE CDR.

2ch-c.	CHF (UE CCS) provides appropriate charging data response.

3ch-a.	CHF (UE CCS) reports to CHF (Tenant CCS) each received Charging Data Request related to PDU session establishment and PDU session release for update of PDU sessions of the S-NSSAI for the tenant.

3ch-b.	Charging service request to Tenant CCS with S-NSSAI, UE PDU session charging information, and generate the CDRs. The billing domain can analysis the charging information in the CDRs to get the NS actual duration.

3ch-c.	CHF (Tenant CCS) provides appropriate charging data response.

### 6.7.3	Solution#7.3  Network slice actual duration charging from the NSACF

#### 6.7.3.1	General description

This solution addresses the Key Issue#7 for REQ-NSCH-12 potential requirement and is NS actual duration measured by CHF based on the concurrency number of PDU sessions for an S-NSSAI.

#### 6.7.3.2	Architecture description

The architecture for this solution is present as the following figure 6.7.3.2-1.

Figure 6.7.3.2-1: the NS actual duration from NSACF Reporting

#### 6.7.3.3	Flow description

The following figure 6.7.3.3-1 describes the high level charging procedure for NS actual duration measured by Tenant CCS based on NSACF reporting.

Figure 6.7.3.3-1: NS actual duration based on the NSACF reporting

1.	NS actual duration charging per S-NSSAI covered by Tenant CCS. NS during charging is configurated in the Tenant CCS, including the relationship between S-NSSAI and Tenant ID, etc.

2. 	Steps according to 3GPP TS 23.501 [11] for the network slice access control, including the control and count the number of the PDU session.

3.	NSACF counts and aggregates the number of the PDU sessions, when NSACF counts the number of PDU session from 0 to greater than 0, “he trigger of "change of the number of ”he PDU session" is armed.

3ch-a.	NSACF determinates to trigger charging service request to CHF (Tenant CCS) with the number of PDU sessions.

3ch-b.	CHF (Tenant CCS) counts the duration of the network slice for tenant.

3ch-c.	CHF (Tenant CCS) provides appropriate charging data response.

4.	NSACF counts and aggregates the number of the PDU session, when NSACF counts the number of PDU session from greater than 0 to 0, “he trigger of "change of the number of ”he PDU session" is armed.

4ch-a.	NSACF determinates to trigger charging service request to CHF (Tenant CCS) with the number of PDU sessions and time stamp.

4ch-b.	CHF (Tenant CCS) counts the duration of the network slice for tenant.

4ch-c.	CHF (Tenant CCS) provides appropriate charging data response.

### 6.7.4	Solution#7.4  Network slice actual duration charging from the NWDAF

#### 6.7.4.1	General description

This solution addresses the Key Issue#7 for REQ-NSCH-12 potential requirement and is NS actual duration measured by Tenant CCS based on performance collection for an S-NSSAI via NWDAF.

#### 6.7.4.2	Architecture description

The architecture for this solution is present as the following figure 6.7.4.2-1.

Figure 6.7.4.2-1: the NS actual duration from NWDAF

#### 6.7.4.3	Flow description

The following figure 6.7.4.3-1 describes the high level charging procedure for NS actual duration measured by CCS based on performance collection:

Figure 6.7.4.3-1: the NS actual duration from NWDAF

1.	NS actual duration charging per S-NSSAI covered by Tenant CCS. NS during charging is configurated in the Tenant CCS, including the relationship between S-NSSAI and Tenant ID, etc.

2.	CEF subscribes the change of the number of PDU session, when NWDAF analyses the number of PDU session from 0 to greater than 0 and counts the number of PDU session from greater than 0 to 0, NWDAF notifies the CEF with the status of number of PDU session.

3.   CEF counts the duration of NS based on the status of the number of PDU session.

3ch-a.	CEF determinates to trigger charging service request to CHF (Tenant CCS) with S-NSSAI, time stamp and the number of PDU sessions.

3ch-b.	CHF (Tenant CCS) counts the duration of the network slice for tenant.

3ch-c.	CHF (Tenant CCS) provides appropriate charging data response.

### 6.7.5	Evaluation

All Solutions #7.1, #7.2, #7.3, and #7.4 solve Key issue #7.

Solutions #7.1 and #7.2 involve UE CCS, whereas solutions #7.3 and #7.4 are independant from UE CCS.Since it is expected for a UE to always be served by a UE CCS, Tenant may be served by different UE CCS(s), at least one of the solutions between Solution #7.1 and #7.2 or both of them are relevant.

It can also be noted that Solutions #7.1 and #7.2 have a simplified option described by existing TS 32.255 [9] Annex D with the Combined UE CCS - Tenant CCS when the Communication Service Provider (CSP) and Network Slice Provider (NSP) are the same.

### The difference between Solution #7.1 and Solution #7.2 is the handling of the NS duration by the Tenant CCS: the Solution #7.2 relies on post-processing analysis of CDRs, whereas the Solution #7.1 needs  the Tenant CCS to be enhanced to support the calculation of NS duration in real-time.6.7.6	Conclusions

It is concluded on Solution #7.2 to be normative work, without the interface between UE CCS and Tenant CCS per conclusion for Key Issue #x in clause 6.x.5.

## 6.8	Key Issue #8: Converged Charging for NSSAA

### 6.8.1	General Description

This key issue is for investigating how to support UE Network Slice-specific Authentication and Authorization (NSSAA) network slice access converged charging.

NSSAA functionality specified in clause 5.15.10 3GPP TS 23.501 [7] is an additional AAA-S EAP based authorization for UE to access a network slice, after the primary authorization was granted by the PLMN under the list of allowed S-NSSAI(s). This capability can be offered and monetized by Operators (PLMN) to 3rd party enterprises owning AAA-S.

The use case for this key issue is:

A UE has a subscription with the MNO. The MNO is the owner of NSSAF and AAA-P.

The MNO has a business relationship with the 3rd party MNO for access to network slice(s) by MNO UEs requiring Slice-Specific Authentication and Authorisation by the 3rd party. The 3rd pary is the owner of AAA-S.

The MNO to 3rd party charging could be based number of invocations by MNO UEs for this additional Authentication and Authorization towards the 3rd party:

This investigation covers the following:

-	determination of which entity/entities in the 5G system is suitable to provide charging information for usage of NSSAA by UE;

-	determination of the main interactions required to obtain charging information for usage of NSSAA.

-	identification of the main charging information to be collected;

### 6.8.2	Solution#8.1 Converged Charging for NSSAA

#### 6.8.2.1	General description

This solution addresses the Key Issue#8 for REQ-NSCH-13 and is based on use of Nchf converged charging service by 5GS Network Functions involved in UE NSSAA procedures.

AMF interacts with CHF during NSSAA procedure with the Network Slice-Specific Authentication and Authorization Function (NSSAAF) for a given UE and S-NSSAI for CHF to produce appropriate CDR.

NSSAAF interacts with CHF during NSSAA procedure for a given UE and S-NSSAI for CHF to produce appropriate CDR.

The solution applies for NSSAA, AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure and AAA Server triggered Slice-Specific Authorization Revocation.

The AMF Charging characteristics (Annex A of 3GPP TS 32.256[10]) is extended to include NSSAA related configuration.

NSSAAF Charging characteristics pre-configured in NSSAAF includes NSSAA related information.

#### 6.8.2.2	Architecture description

Figure 6.8.2.2-1: NSSAAF and AMF converged charging architecture

#### 6.8.2.3	Flow description

The figure 6.8.2.3-1 below describe the high level charging procedure for Converged Charging in PEC mode for Network Slice-Specific Authentication and Authorization (NSSAA) procedure based on figure 4.2.9.2-1 3GPP TS 23.502 [11]:

Figure 6.8.2.3-1: Converged Charging for NSSAA - PEC

Steps are based on steps of figure 4.2.9.2-1 3GPP TS 23.502 [11], with the following additions:

0ch. NSSAAF Charging characteristics is pre-configured in NSSAAF indicating: Charging active (Yes/No) for NS authentication/auth, re-authentication/re-auth, revocation per S-NSSAI.

17ch-a to c: Charging active in NSSAAF: Charging Data Request [Event] is sent to CHF with the result of NSSAA Authentication (EAP-Success/Failure, S-NSSAI, GPSI) for CHF NSSAAF CDR generation

17ch-d to e: Charging active in AMF: Charging Data Request [Event] is sent to CHF by AMF with the result of NSSAA Authentication (EAP-Success/Failure, S-NSSAI, GPSI) for CHF AMF CDR generation

The solution for Converged Charging AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure are based on figure 4.2.9.3-1 3GPP TS 23.502 [11] with additional interactions with CHF from NSSAAF and AMF on Nnssaaf_NSSAA_Re-AuthNotification.

The solution for Converged Charging AAA Server triggered Slice-Specific Authorization Revocation procedure are based on figure 4.2.9.4-11 3GPP TS 23.502 [11] with additional interactions with CHF from NSSAAF and AMF on Nnssaaf_NSSAA_RevocationNotification.

### 6.8.3	Solution#8.2 Converged Charging for NSSAA from AAA-P

#### 6.8.3.1	General description

This solution addresses the Key Issue#8 for REQ-NSCH-13 and is based on use of Nchf converged charging service by AAA-P involved in UE NSSAA procedures towards a AAA-S belonging to a third party.

AAA-P interacts with CHF during NSSAA procedure for a given UE and S-NSSAI for CHF to produce appropriate CDR for charging for "requesting (re-)authentication/authorization" between the owner of the NSSAAF and the owner of the AAA-S.

The solution applies for AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure and AAA Server triggered Slice-Specific Authorization Revocation.

AAA-P Charging characteristics pre-configured in AAA-P includes NSSAA related configuration.

#### 6.8.3.2	Architecture description

Figure 6.8.3.2-1: AAA-P converged charging architecture

#### 6.8.3.3	Flow description

The figure 6.8.3.3-1 below describes the high-level charging procedure for Converged Charging in PEC mode for Network Slice-Specific Authentication and Authorization (NSSAA) procedure based on figure 4.2.9.2-1 3GPP TS 23.502 [11]:

Figure 6.8.3.3-1: Converged Charging for NSSAA from AAA-P - PEC

Steps are based on steps of figure 4.2.9.2-1 3GPP TS 23.502 [11], with the following additions:

0ch. AAA-P Charging characteristics is pre-configured in AAA-P indicating: Charging active (Yes/No) for NS authentication/auth, re-authentication/re-auth, revocation per S-NSSAI.

16ch-a to c: Charging active in AAA-P: Charging Data Request [Event] is sent to CHF with the result of NSSAA Authentication (EAP-Success/Failure, S-NSSAI, GPSI) for CHF CDR generation

The solution for Converged Charging AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure are based on figure 4.2.9.3-1 3GPP TS 23.502 [11] with additional interactions with CHF from AAA-P  on AAA Protocol Re-Auth Response.

The solution for Converged Charging AAA Server triggered Slice-Specific Authorization Revocation procedure are based on figure 4.2.9.4-11 3GPP TS 23.502 [11] with additional interactions with CHF from AAA-P on AAA Protocol Revocation Re-Auth Response.

### 6.8.4	Evaluation

Both solutions #8.1 and #8.2 solve Key Issue#8. Solution #8.1 relies on adding a new 5GS NF (i.e. NSSAAF) to the charging architecture with corresponding functionalities and extending charging from AMF: these principles are in the continuity of the charging framework evolution.

Solution #8.x relies on adding AAA-P to the charging architecture: Although 3GPP architectures have considered interfacing AAA framework since earlier releases, it has never been considered to introduce charging from AAA elements, based on the AAA framework definition is not under 3GPP scope. Also the AAA framework includes accounting functionality used by this separate infrastructure to address sub-set of of charging capabilities. With the same rationale, the AAA-P is not approriate.

### 6.8.5	Conclusion

It is concluded on solution #8.1 to be part of normative work.

## 6.9	Key Issue #9: Service based interface between UE CCS and Tenant CCS

### 6.9.1	General Description

This Key Issue is for investigating on the selection for the service based interface between UE CCS and Tenant CCS to support solutions introduced with this distributed architecture under Key Issue #3, Key Issue #6 and Key Issue #7.

The investigation covers the following main capabilities for the service:

- Individual UE charging information from UE CCS to Tenant CCS: e.g. UE PDU session charging information in solution #3.1;

- UE CHF configured with indication on "dependency between UE charging and NS Charging": e.g. NS duration charging in solutions #7.2;

-	NS Tenant Charging information exchanged between UE CHF and Tenant CHF in both directions: e.g. UE charging conditioned to NS charging information in solutions #6.1;

This key issue relates to when standard interface between UE CCS and Tenant CCS is required.

### 6.9.2	Solution #9.1 Reusing Nchf_ConvergedCharging service between UE CHF and Tenant CHF

#### 6.9.2.1	General description

This solution is based on the re-use of Nchf_ConvergedCharging service between UE CHF and Tenant CHF to address the Key Issue #9.

UE CHF is a new "NF service consumer" (i.e. embedding a CTF) for Nchf_ConvergedCharging service exposed by NS Tenant (the NF service producer).

Nchf_ConvergedCharging service is enhanced to simultaneously include both UE level and NS level charging information.

Standard interface between UE CCS and Tenant CCS is required.

#### 6.9.2.2	Architecture description

Figure 6.9.2.2-1: UE CCS – Tenant CCS distributed architecture

#### 6.9.2.3	Flow description

The flows between UE CHF - Tenant CHF are described in clauses 6.3.2.3, 6.6.2.3, 6.6.3.3, 6.6.x.3, 6.7.1.3 and 6.7.2.3, with "ChS Charging service Request" replaced by the Nchf_ConvergedCharging service enhanced for UE CHF -Tenant CHF.

In the flows, between the UE CHF and Tenant CHF, in addition to the S-NSSAI, a "Tenant Id" is considered for shared NS scenarios.

### 6.9.3	Solution #9.2 New charging service API between UE CHF and Tenant CHF

#### 6.9.3.1	General description

This solution is based on the introduction of a new charging service API between UE CHF and Tenant CHF to address the Key Issue #9.

UE CHF is the "NF service consumer" of this new charging service exposed by NS Tenant CHF (the NF service producer).

The charging service simultaneously includes both UE level and NS level charging information.

Standard interface between UE CCS and Tenant CCS is required.

#### 6.9.3.2	Architecture description

Same architecture as clause 6.9.2.2.

#### 6.9.3.3	Flow description

The flows between UE CHF - Tenant CHF are described in clauses 6.3.2.3, 6.6.2.3, 6.6.3.3, 6.6.x.3, 6.7.1.3 and 6.7.2.3, with "ChS Charging service Request" replaced by the new charging service.

In the flows, between the UE CHF and Tenant CHF, in addition to the S-NSSAI, a "Tenant Id" is considered for shared NS scenarios.

### 6.9.4	Evaluation

Both solutions #9.1 and #9.2 can solve Key Issue #9.

The need to address both UE level and NS Tenant level converged charging is already supported by existing Nchf_ConvergedCharging service of the solution #9.1.

The solution #9.2 does not bring any new charging functionality, compared to solution #9.1.

The solution #9.1 introduces a new paradigm with CHF having two roles regarding the Nchf_ConvergedCharging service: as a "NF service consumer" of the Nchf_ConvergedCharging service (i.e. UE CHF) and as a "NF service provider" of the Nchf_ConvergedCharging service (i.e. NS Tenant CHF). This behavior can be introduced without changing the existing Nchf_ConvergedCharging service principles. .

### 6.9.5	Conclusion

It is concluded on solution #9.1 to be normative, i.e.:

- the standard interface between UE CCS and Tenant CCS is required;

- enhancing existing Nchf_ConvergedCharging service between UE CHF and Tenant CHF.

# 7	Conclusions and recommendations

The following is concluded for normative work:

-	Solutions #1.2 and #1.6 for Key Issue #1 (Converged Charging for simultaneous number of UEs/max. number of UEs per network slice) per clause 6.1.8 conclusion;

-	Solutions #2.1 and #2.5 for Key Issue #2 (Converged Charging for simultaneous number of PDU sessions/max. number of PDU sessions per network slice) per clause 6.2.7 conclusion.

It is concluded following solutions to go to normative phase:

-	Solution #3.1 for Key Issue #3 (Volume based Converged Charging per network slice) per clause 6.3.5 conclusion;

-	Solutions #6.1 and #6.2 for Key Issue #6 (UE Converged Charging based on network slice charging) per clause 6.6.5 conclusion;

-	Solution #7.2 for Key Issue #7 (Converged Charging for actual duration per network slice) per clause 6.7.6 conclusion;

-	Solution #8.1 for Key Issue #8 (Converged Charging for NSSAA) per clause 6.8.5 conclusion.

And it is recommended to incorporate the description for this normative work into a revision of the WID on Charging Aspects of Network Slicing Phase 2.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 01-2021 | SA5#135e | S5-211042 |  |  |  | Initial skeleton | 0.0.0 |
| 02-2021 | SA5#135e | S5-211416 S5-211418 S5-211044 S5-211045 S5-211421 S5-211385 S5-211387 |  |  |  | Update on skeleton Add scope Rel-Add new subclause on background Add new subclause on key Issues Introduction of key Issue #1 & #2 Introduction of background on NS charging information Introduction of potential requirements | 0.1.0 |
| 02-2021 | SA5#135e |  |  |  |  | EditHelp review | 0.1.1 |
| 03-2021 | SA5#136e | S5-212475 S5-212476 S5-212477 S5-212478 |  |  |  | Update subclause on background from GSMA Introduction of new requirements and Key issues - volume and duration Adding key issue for number of UEs per network slice Adding key issue for number of PDU sessions per network slice | 0.2.0 |
| 05-2021 | SA5#137e | S5-213121 S5-213359 S5-213360 S5-213617 S5-213618 S5-213619 S5-213620 S5-213621 S5-213643 |  |  |  | Introduction of Key issue#X Individual UE charging based on Network Slice Adding event based solution to key issue 5 Adding session based solution to key issue 5 Introduction of a distributed charging architecture solution for KI#3 Introduction of a solution for KI#3 based on KPI Solutions evaluation for KI#3 Introduction of a solution based on distributed charging architecture for KI#X Solution for Key Issue #1 based on new charging service Intro new requirements and KI - NS actual duration | 0.3.0 |
| 08-2021 | SA5#138e | S5-214079 S5-214506 S5-214701 S5-214713 S5-214715 S5-214723 S5-214724 |  |  |  | New solution for KI#9 Introduction of a new Key issue and solution – NSSAA charging Combination of Key issues Clarification on the solution 1 Adding session based solution to key issue 5 Additional solution for NS Tenant Charging in solution 3.1 Introduce solution for NS actual duration | 0.4.0 |
| 10-2021 | SA5#139e | S5-215069 S5-215070 S5-215071 S5-215072 S5-215587 |  |  |  | Update Solution#3.1 Volume aggregation by NS Tenant CCS Update Solution#6.1 NS Tenant CCS separated from UE CCS Update Solution#7.1 Duration determination by NS Tenant CCS Update Evaluation of Key issue #3 Addition of background about NSPA Charging | 0.5.0 |
| 11-2021 | SA5#140e | S5-216522 |  |  |  | New key issue on types of maximum | 0.6.0 |
| 2021-12 | SA#94e | SP-211424 |  |  |  | Review by EditHelp, presented for information | 1.0.0 |
| 01-2022 | SA5#141e | S5-221654 |  |  |  | Conclusions for KI#3 | 1.1.0 |
| 04-2022 | SA5#142e | S5-222538 |  |  |  | Add evaluation for Key issue #7 | 1.2.0 |
| 05-2022 | SA5#143e | S5-223226 S5-223227 S5-223696 S5-223697 S5-223711 |  |  |  | Add evaluation and conclusion for Key issue#8 Add NSACF based NS charging new solution for Key issue#6 Add NWDAF based NS charging new solution for Key issue#6 Add new solution for Key issue#8 Clarify the Evaluation for the key issue 7 | 1.3.0 |
| 08-2022 | SA5#145e | S5-225651 S5-225725 S5-225726 S5-225727 S5-225728 S5-225729 S5-225736 S5-225755 S5-225762 S5-225763 S5-224402 S5-225404 |  |  |  | Convert the Key issue#9 into background Solve Editor's Note on solution#1.1 Solve Editors Notes on solutions#6.2 Solve Editor’s Note on clause 4.1 New NSACF (CTF) - NS quota management solution for Key issue#1 Solve Editors Note on Key Issue #5 Evaluation and conclusion for Key issue#2 Solve Editors Notes on solutions#1.3 Evaluation and conclusion for Key issue#1 New NSACF (CTF) - NS quota management solution for Key issue#2 Remove Key issue#4 Clarify Potential requirements in Key issues#1 | 1.4.0 |
| 11-2022 | SA5#146 | S5-226693 S5-226848 S5-226849 S5-226850 S5-226851 S5-226852 S5-226853 S5-226854 |  |  |  | Solve Editors Notes on evaluation for KI#1 Clarify solution #1.6 description Add conclusion for Key issue#1 Correction on evaluation for KI#2 Add conclusion for Key issue#2 Correction on the uniform terminology Clarification on the Maximum Conclusion on the Key Issue #7 | 1.5.0 |
| 01-2023 | SA5#146Bis-e | S5-231252 S5-231256 S5-231257 S5-231258 S5-231259 S5-231261 S5-231262 S5-231264 S5-231265 S5-231267 |  |  |  | Final conclusion for Key Issue #3 Solve Editor's Note in solution #7.2 Refinement on evaluation and conclusion for Key Issue #7 Add a solution for Key Issue #x - reuse Nchf_ConvergedCharging Add a solution for Key Issue #x - new charging service API Add new Key Issue on SBI between UE CHF and Tenant CHF Update solution #6.3 and add new solution #6 Add evaluation and conclusion for Key Issue #6 Add conclusions and recommendations Add evaluation and conclusion for Key Issue #x. | 1.6.0 |