3GPP TSG-SA5 Meeting #147	S5-233020

# Athens, Greece, 27 February – 3 March 2023

# Source:	Huawei, CMCC

# Title:	pCR 28.830 add concept of anomaly report

# Document for:	Approval

Agenda Item:	6.7.7.1

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	S5-225661: "draft TR 28.830 Fault supervision evolution"; v0.3.0

[3]	TS 32.111-1: "Telecommunication management; Fault Management; Part 1: 3G fault management requirements (Release 17)”

[4]	TS 28.532: “Management and orchestration; Generic management services; (Release 17)”

[5]	TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"; v17.1.1

[6]	TR 28.809: "Management and orchestration; Study on enhancement of Management Data Analytics (MDA)"; v17.0.0

3	Rationale

This tdoc discusses a new concept of anomaly report in FSEV study [1, 2].

In [3], the definition of alarm is provided as follows:

Alarm: An alarm signifies an undesired condition of a resource (e.g. network element, link) for which an operator action is required. It emphasizes a key requirement that operators (above Itf-N) should not be informed about an undesired condition unless it requires operator action. Use of this emphasis does not exclude this case: In certain context, it is not possible for alarm reporters (below Itf-N) to know whether a particular undesired condition requires operator action or not. In such context, the NM may receive alarms that do not require operator action [3].

It is expected that the capability of identifying alarms which require further actions should be supported. So that important anomalies such as mass faults, major network faults, service and network performance deterioration, and potential risks can be clearly reported to the consumer.

Observation 1: More works may be needed to identify different alarm types, consequences and further actions.

In [1], the relation and interaction with MDAS is one of the objectives to study in FS_FSEV.

Relation and interaction with MDAS and COSLA for evolved fault supervision, e.g., how to take advantage of and integrate MDAS capabilities into the solutions and if any, recommended capabilities needed for MDAS enhancements.

In [5], MDA capability of “failure prediction” in “MDA assisted fault management” has been specified.

[…] Due to the fact that failure prediction could depend on the existing alarm incidents and relevant historical and real-time data (performance measurement information, configuration data, network topology information, etc.), there is a possibility for MDA to be used in conjunction with AI/ML technologies and model training to predict potential failures [5].

In order to avoid the occurrence of failures and abnormal network status, it is necessary for consumers of analytics to obtain the required details of potential failure and the corresponding degradation trend (abnormal KPI, performance measurement information, possible alarm type, fault root cause, etc.) [5]. […]

Failure prediction is supported by MDA MnS producer. Other types of fault related analysis capabilities may also be considered, e.g., fault analysis, performance degradation analysis, quality of service degradation prediction etc.

In [6], more analysis capabilities of MDA have been described, e.g., KPI anomaly analysis, Alarm incident analysis, Alarm malfunction analytics etc.

Observation 2: More advanced analytics capabilities are needed to assist fault management. The enahcements and integration of MDA assisted fault management should be studied in FS_FSEV.

In summary, more analytics are needed to identify whether actions are needed, and to predict potential failure risks etc. A generic term “anomaly report” can be introduced to cover various types of analytical results for MDA assisted fault management in the FSEV study.

Anomaly report

The alarm/fault related analytics or prediction information to indicate that actions are needed to mitigatethe impacts on 3GPP system.

The anomaly report may be fault or performance degradation or service failure risks which have occurred or may occur in the future. To identify anomalies, existing alarm notifications, performance data, historical data and configuration data etc may be used. Since it is the analytical result of the MDA MnS producer to assist the fault management, it does not modify exiting FM interface.

Summary

Observation 1: More works may be needed to identify different alarm types, consequences and further actions.

Observation 2: More advanced analytics capabilities are needed to assist fault management. The enahcements and integration of MDA assisted fault management should be studied in FS_FSEV.

Proposal 1: It is proposed to introduce the concept of anomaly report.

4	Detailed proposal

1st Change

5.1	Key Issue 1: Missing definitions

5.1.1	Description

TS 28.532, clause 11.2, which defines the Fault Supervision MnS, does not provide the necessary definitions and descriptions required to understand the current state of art as to alarm management. This is because much of the material specified and available for the IRP Framework was not moved to SBMA.

5.1.2	Potential solutions

TS 28.532, clause 11.2 should provide all necessary definitions. This clause provides an (non exhaustive) overview on available definitions.

[……]

Root cause analysis:

RFC 8632: An alarm can indicate candidate root-cause resources, for example, a database issue alarm referring to a full-disk partition.

SA5 IRP: Root Cause Analysis is a process that can determine and identify the network condition (e.g. fault, mis configuration) causing the alarms.

Discussion: The SA5 definition is not compliant to what is done in the interface definition. The interface definition clearly refers to alarms as the result of the root cause analysis and not to network conditions.

SA5 needs to discuss the old definition of the term root cause analysis.

MDA assisted fault management

MDA type of “failure prediction” in “MDA assisted fault management” has been specified in TS 28.104 [x]. Other types of fault related analysis capabilities may also be considered, e.g., fault analysis, performance degradation analysis, quality of service degradation prediction etc.

Proposed new definition:

Editor's note: A new definition is not agreed yet. The following proposal will be subject to further modifications and is just provided here as baseline for further discussions.

Anomaly report: The alarm/fault related analytics or prediction information to indicate that actions are needed to mitigate the impacts on 3GPP system.

The anomaly report may be fault or performance degradation or service failure risks which have occurred or may occur in the future. To identify anomalies, existing alarm notifications, performance data, historical data and configuration data etc may be used.

5.1.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

End of change