3GPP TSG-SA5 Meeting #147	S5-233133

Athens, Greece, 27 February – 3 March 2023

# Source:	Huawei

# Title:	pCR 28.865 Add solultion of service and network analysis

# Document for:	Approval

Agenda Item:	6.9.5.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-211442: "New SID on deterministic communication service assurance"

[2]	3GPP TR 28.865: "Study on deterministic communication service assurance"; v0.3.1

[3]	3GPP TS 28.541: "5G Network Resource Model (NRM); Stage 2 and stage 3"; v18.2.2

[4]	TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.3.0

[5]	TS 22.261: "Service requirements for cyber-physical control applications in vertical domains; stage 1"; v18.8.0

[6]	SP-220981: "New Rel-18 WID Management Data Analytics phase 2"

[7]	TS 28.104: "Management and orchestration; Management Data Analytics (MDA)"; v18.2.0

[8]	TS 28.535: "Management services for communication service assurance; Requirements"; v17.5.0

[9]	TS 28.536: "Management services for communication service assurance; Stage 2 and stage 3"; v17.4.1

[10]	NG.116: "Generic Network Slice Template"; v7.0; 17 June 2022.

3	Rationale

This tdoc addresses the solution of service and network analysis related to DCSA [1, 2].

3GPP management system should provide the analytical information of service experience and network performance related to deterministic communication service, especially for latency, throughput, and positioning because there are more stringent requirements. MDA capabilities of SLS analysis may be consumed or enhanced for this purpose. MDA capabilities of SLS analysis include service experience analysis, network slice throughput analysis, E2E latency analysis etc.

Service experience and network performance degradation issues will be identified and demarcated, the source and location of the issues and the root causes will be indicated, potential optimization solutions may also recommended.

Some information of the analytical report are listed as follows:

Latency analytics:

MDA type of E2E latency related issue analysis in SLS analysis group may be consumed or enhanced for latency analysis, currently the analytics output contain e2ELatencyIssueId, e2ELatencyIssueType and affectedObjects. The e2ELatencyIssueType can indicate the enumerated values RAN latency issue and CN latency issue, further information may be provided as follows.

- Latency issues which are out of time boundary constraints, e.g. latency of RAN or CN part exceeding the configured PDB (packet delay budget);

- Jitter exceeds the predefined threshold etc;

- BurstArrivalTime issues,

- data transfer or reception interval issues, e.g. some of the transfer intervals do not conform to the configured periodicy;

- Possible causes of the latency issues, e.g., coverage, interference, and parameter configurations etc;

Throughput analytics:

MDA type network slice throughput analysis in SLS analysis group may be consumed or enhanced for throughput analysis. Currently some throughput statistics and predictions are provided. More analytics regarding the guaranteed flow bit rate may be considered.

- Throughput below the guranteed requirements etc;

Positioning analytics:

More accuracy of positioning is required for deterministic communications, the positioning related analysis may be considered.

- Positioning accuracy issues;

- Possible causes of the latency issues, e.g., coverage, interference, and parameter configurations etc;

The corresponding network are optimized if needed based on the analytical report. The optimization results may be reported to the consumer for monitoring or further processing.

Proposal 1: 3GPP management system should have the capability to provide the service and network analysis  related to deterministic communication service. MDA capabilities of SLS analysis may be consumed or enhanced for this purpose.

4	Detailed proposal

This document proposes the following changes in TR 28.865.

1st Change

5	Issues and potential solutions

Editor's note: this clause will contain the issues and potential solutions for deterministic communication service assurance. Relation and potential enhancements to eCOSLA will also be studied for the related issues.

5.X.2	Potential solutions

5.X.2.a	Potential solution #3: Service and network analysis

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution for issue#1 at a high-level.

The purpose of service and network analysis is to identify service experience and network performance degradation issues for network optimization. Based on the analysis results, network optimization solutions are used to improve the service experience and network performance, e.g. reconfiguration of network resource and/or parameters to reduce the latency and increase data rates. e

3GPP management system should provide the analytical information of service experience and network performance related to deterministic communication service, especially for latency, throughput, and positioning because there are more stringent requirements. MDA capabilities of SLS analysis may be consumed or enhanced for this purpose. MDA capabilities of SLS analysis include service experience analysis, network slice throughput analysis, E2E latency analysis etc.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made for issue#1.

The analytics information may include issues and possible causes of the performance metrics that do not meet the deterministic communication services requirements, as listed in the following aspects:

Latency analytics:

MDA type of E2E latency related issue analysis in SLS analysis group may be consumed or enhanced for latency analysis, currently the analytics output contain e2ELatencyIssueId, e2ELatencyIssueType and affectedObjects. The e2ELatencyIssueType can indicate the enumerated values RAN latency issue and CN latency issue, further information may be provided as follows.

- Latency issues which are out of time boundary constraints, e.g. latency of RAN or CN part exceeding the configured PDB (packet delay budget);

- Jitter exceeds the predefined threshold etc;

- BurstArrivalTime issues,

- data transfer or reception interval issues, e.g. some of the transfer intervals do not conform to the configured periodicy;

- Possible causes of the latency issues, e.g., coverage, interference, and parameter configurations etc;

Throughput analytics:

MDA type network slice throughput analysis in SLS analysis group may be consumed or enhanced for throughput analysis. Currently some throughput statistics and predictions are provided. More analytics regarding the guaranteed flow bit rate may be considered.

- Throughput below the guranteed requirement etc;

Positioning analytics:

More accuracy of positioning is required for deterministic communications, the positioning related analysis may be considered.

- Positioning accuracy issues;

- Possible causes of the latency issues, e.g., coverage, interference, and parameter configurations etc;

Editor's Note: These outputs will be revisited.

The corresponding network are optimized if needed based on the analytical report. The optimization results may be reported to the consumer for monitoring or further processing.

End of change