3GPP TSG-SA5 Meeting #147	S5-232077

# Athens, Greece, 27th Feb 2023 - 3rd Mar 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.160 | CR | 0032 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarifications for lifecycleStatus property |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_MetDep |  |  |  |  |  | Date: |  |  | 2023-02-13 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clarify the documentation of lifeCycleStatus |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that lifeCycleStatus shall be specified if and only if its value is deprecated. Added example about documenting lifeCycleStatus for attributes. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unneeded or non-uniform documentation of the lifeCycleStatus property. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

W4.3.a.1	Definition

This clause is written in natural language. The <definition> clause refers to the class itself.

Classes (and datatypes) have a lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this clause.

Optionally, information on traceability back to one or more requirements supported by this class may be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| TS 28.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| TS 28.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Next change

W4.3.a.3	Attribute constraints

This clause presents constraints for the attributes, and one use is to present the predicates for conditional qualifiers (CM/CO).

This information is provided in a table. An example of such a table is given here below:

| Name | Definition |
| --- | --- |
| configuredMaxTxPower CM support qualifier | Condition: The sector-carrier has a downlink [4]. |
| sNSSAIList CM support qualifier | Condition: Network slicing feature is supported [4]. LifecycleStatus: Deprecated. |

Attributes have a  lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this table.

This clause shall state "None." when there is no attribute constraint to define.

Next change

Yb.1.a.1.1	Description

This subclause shall be written in natural language.

Operations have a lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this operation should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

Next change

Yb.1.a.1	Definition

This subclause shall be written in natural language.

Notifications have a lifecycleStatus property as defined by [3] clause 5.2.A. If and only if the lifecycleStatus is not current (its default value), that shall be indicated in this subclause.

Information on traceability back to one or more requirements supported by this notification should also be defined here, in the following form:

| Referenced TS | Requirement label | Comment |
| --- | --- | --- |
| 3GPP TS 32.xyz [xy] | REQ-SM-CON-23 | Optional clarification |
| 3GPP TS 32.xyz [xy] | REQ-SM-FUN-11 | Optional clarification |

End of  changes