3GPP TSG-SA5 Meeting #147 	S5-232170

Athens, Greece, 27th February – 3rd March 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.550 | CR | 0076 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.550 Enhance the streaming format of PM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-02-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Currently, only static frames are streamed over the wire for PM streaming. - Existing framework does not facilitate the transmission of adaptive frames. - MnS producers of PMs are mandated to send only static frames irrespective of the rate of change of PMs over time. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Existing streaming format of PMs is enhanced by an adaptive frame structure, wherein no change and small changes can be streamed with relatively less resources over the wire. The problem and the solution are described in a discussion paper via companion contribution S5-232146. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inefficient use of resources for streaming of PMs as static frames have to be streamed irrespective of the nature of change of the PMs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | G.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

Annex G (normative): 
ASN.1 definition for performance data stream units

## G.1	ASN.1 definition rule

For performance data streaming, the type of WebSocket Data frame shall be binary (with opcode of 0x2).

This clause specifies the abstract syntax notation one (ASN.1) definition for the Performance Data Stream Units (see Annex C) as Payload data of WebSocket Data frame.

The Performance Data Stream Units are described using ASN.1 as specified in ITU-T Rec. X.680 [15] and X.681 [16]. Transfer syntax for Performance Data Stream Units is derived from their ASN.1 definitions by use of Packed Encoding Rules (PER), aligned as specified in ITU-T Rec. X.691 [17].

The following encoding rules apply in addition to what has been specified in ITU-T Rec. X.691 [17]:

-	When a bit string value is placed in a bit-field as specified in clause 15.6 to 15.11 in ITU-T Rec X.691 [c], the leading bit of the bit string value shall be placed in the leading bit of the bit-field, and the trailing bit of the bit string value shall be placed in the trailing bit of the bit-field;

-	When decoding a BIT STRING or OCTET STRING constrained with a Contents Constraint, PER decoders are required to never report an error if there are extraneous zero or non-zero bits at the end of the BIT STRING or OCTET STRING.

The serialization method encompasses adaptive frame structure to enable to producer to optimize the amount of data streamed over the wire for measurements that does not change fast over time.

NOTE:	The terms 'leading bit' and 'trailing bit' are defined in ITU-T Rec. X.680 [15]. When using the 'bstring' notation, the leading bit of the bit string value is on the left, and the trailing bit of the bit string value is on the right.

## G.2	ASN.1 definition

-- ASN1START

PerformanceDataStreamUnits-Schema DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- PDSUs-START

PDSUs ::=                      SEQUENCE OF PDSU

PDSU ::=  SEQUENCE {

streamId                   INTEGER,

granularityPeriodEndTime   DATE-TIME,

abbreviatedPdsuIndicator   BOOLEAN,

standardizedMeasResults    SEQUENCE OF MeasValue,

vendorSpecificMeasResults  SEQUENCE OF MeasValue OPTIONAL -- may be omitted

}

MeasValue ::= CHOICE {

integerValue   INTEGER,

realValue      REAL,

stringValue    VisibleString,

subCounters    SubCounterListType,

abbreviatedValue	OCTET STRING (SIZE(1)),

noChangeIndicator	BOOLEAN,

...                                             -- allow extension in further version

}

-- uses recursion for the value to support multi-dimensional measurements

SubCounterListType ::= SEQUENCE {

subCounterIndex   SubCounterIndexType,

subCounterValue   MeasValue OPTIONAL            -- "empty" bins are allowed

}

SubCounterIndexType ::= CHOICE {

sum       VisibleString ("SUM"),

binIndex  INTEGER,

qOS-5QI   INTEGER,

qOS-QCI   INTEGER,

cause     INTEGER,

stringIndex   VisibleString,

plMN      OCTET STRING (SIZE(3)),              -- definition from TS 38.413

sNSSAI    SEQUENCE {                           -- definition from TS 38.413

sst   OCTET STRING (SIZE(1)),

sd    OCTET STRING (SIZE(3))

},

...                                            -- allow extension in futher version

}

-- PDSUs-STOP

END

-- ASN1STOP

End of Changes