3GPP TSG-SA5 Meeting #148e	S5-233442rev3

e-meeting, 17-25 April 2023

- Source:	Huawei Technologies

- Title:	pCR TR 28.843 Addition of solution about CHF-to-CHF

- Document for:	Approval

Agenda Item:	7.5.5

# 1	Discussion

This is a pCR to TR 28.843 introducing the solution about CHF-to-CHF.

# 2	References

[1]	3GPP TR 28.843 "Study on Structure of Charging for Verticals ".

# 3	Rationale

This pCR proposes to introduce the the solution about CHF-to-CHF in TR 28.843 "Study on Structure of Charging for Verticals".

# 4	Detailed propsosal

Propose to incorporate the following change into the TR 28.843 [1].

First changes

## 7.x	Solution #x: CHF-to-CHF

### 7.x.1	Solution Description

#### 7.x.1.1	General

This soluion #x resolves the key issues X. Based on the study about wholesale and roaming charging, CHF-to-CHF communication.

Editor’s note: The solution about SMF to multiple CHFs should be investigated.

#### 7.x.1.2	Solution

Figure 7.x.1.2-1: CHF to CHF communication

Two types of charging are considered in the context of vertical charging:

-	the retail charging (i.e.B2C) by A-CHF. which is out of 3GPP scope.

-	the wholesale charging by CHF for MVNO as the Vertical (i.e.B2B), please see the details of solution #4.3 in the TR 28.827.

End of changes