3GPP TSG-SA5 Meeting #148e 	S5-233266rev1

Electronic meeting, Online, 17 -25 April 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Add a potential solution for KI to Add a new key issue for Advertising NRM properties by MnS producer

- Document for:	Approval

Agenda Item:	6.8.2.3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below.

# 2	References

[1]          3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

[2]          S5-233264 Rel-18 pCR TR 28.831 Add a new key issue for Advertising NRM properties by MnS producer

[3]          S5-233265 Rel-18 pCR TR 28.831 Add potential requirements for KI for Advertising NRM properties by MnS producer

# 3	Rationale

pCR with tDoc# S5-233264 for this SA5#148e meeting proposes a new key issue for Advertising NRM properties by MnS producer, and pCR with tDoc# S5-233265 for this SA5#148e meeting proposes potential requirements for the same. This contribution proposes a potential solution for the mentioned key issue.

Revision history:

- S5-233266 is a revision of S5-233044

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

1st Change

## 4.x	Key Issue #x: Advertising NRM properties by MnS producer

### 4.x.1	Issue description

### 4.x.2	Current situation

### 4.x.3	Analysis

### 4.x.4	Potential requirements

### 4.x.5	Potential solution #1

- 4.x.5.1	Introduction

This potential solution introduces mechanism for MnS producer to advertise the following NRM properties to the MnS consumers.

- Advertise the list of IOCs supported by the MnS producer.

- Advertise the attributes of each IOC that is supported by the MnS Producer.

- 4.x.5.2	MnS Producer based on YANG-Netconf solution set

MnS producer using YANG-Netconf solution set is required to advertise supported IOCs, attributes and constrains or deviations, from the 3GPP defined standard NRM schema, to the MnS consumers using the YANG Library (ietf-yang-library defined in RFC8525).

- 4.x.5.3	MnS Producer based on OpenAPI solution set

This clause proposes a mechanism for MnS producer, based on OpenAPI solution set, to advertise the supported NRM properties to the MnS consumer.

The MnS producer advertises the schema that it supports, along with the information related to the corresponding 3GPP defined standard schema. The schema advertised by the MnS producer contains the IOCs and attributes supported. The MnS consumer(s) requiring this information can query this schema to retrieve the information about the NRM properties of the MnS producer.

4.x.6	CR proposal

### 4.x.7	Conclusion

End of change