3GPP TSG-SA5 Meeting #148e 	S5-233290

Electronic meeting, Online, 17 -25 April 2023

- Source:	Nokia, Nokia Shanghai Bell

- Title:	Rel-18 pCR 28.831 Clarify definition of the reason property for manipulating objects (error response format)

- Document for:	Approval

Agenda Item:	6.8.2.4 - FS_eSBMAe_WoP#3

# 1	Decision/action requested

The group is requested to discuss and approve the pCR below

# 2	References

[1]		3GPP TS 28.831: " Management and orchestration; Study on basic Service-Based Management Architecture (SBMA) enabler enhancements"

# 3	Rationale

None.

# 4	Detailed proposal

The following changes are proposed for TR 28.831[1].

Begin of modifications

X.4.4	Error reasons for object manipulations

The following reasons are defined for errors that may occur when attempting to create objects with PUT, POST. 3GPP JSON Merge Patch and 3GPP JSON Patch, or when attempting to delete objects with DELETE, 3GPP JSON Merge Patch and 3GPP JSON Patch:

- OBJECT_CREATION_NOT_ALLOWED (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): One or more objects cannot be created as requested because objects of this class cannot be created by MnS Consumers.

- OBJECT_DELETION_NOT_ALLOWED (related type: MODIFICATION_NOT_ALLOWED, 403 Forbidden): One or more objects cannot be deleted as requested, because objects of this class cannot be deleted by MnS Consumers.

- NEW_OBJECT_CLASS_NAME_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): One or more objects cannot be created as requested, because the specified object class name is unknown to the MnS Producer.

- NEW_OBJECT_REPRESENTATION_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): One or more objects cannot be created as requested, because the received object representation does not validate.

- NEW_OBJECT_CONTAINMENT_INVALID (related type: VALIDATION_ERROR, 400 Bad Request): One or more objects cannot be created under the specified parent as requested, because this containment is not allowed.

- NEW_OBJECTS_ID_EXISTS (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Content): One or more objects cannot be created as requested because the specified "id" exists already under the specified parent.

- NEW_OBJECTS_PARENT_NOT_FOUND (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Content): One or more objects cannot be created as requested, because their specified parents do not exist.

- NEW_OBJECT_ATTRIBUTE_VALUE_MISSING (related type: VALIDATION_ERROR, 400 Bad Request): One or more objects cannot be created as requested, because attribute or attribute field values, that shall be provided in the creation request, are not provided.

- OBJECTS_CARDINALITY_INVALID (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Content): One or more objects cannot be created or deleted as requested, because this would result in violating the cardinality constraints.

- OBJECT_NOT_A_LEAF (related type: REQUEST_OBJECTS_MISMATCH, 422 Unprocessable Content): One or more objects cannot be deleted as requested, because they are no leaf objects.

- OBJECT_NOT_FOUND (related type: IE_NOT_FOUND, 400 Bad Request): One or more objects cannot be deleted as requested, because they do not exist.

- OP_UNKNOWN (related type: VALIDATION_ERROR, 400 Bad Request): The patch operation specified by the "op" property is not known by the MnS Producer. This situation may occur, for example, when a patch operation is not supported or wrongly spelled.

The list of error reasons is not exhaustive. Other values may be defined and used as well.

Editor's note: When other values for "reason" are used they should follow a certain pattern (capital letters only, short, …). This aspect should be clarified here or by referencing to some general rules for string enums somewhere else.

X.4.4.3	Examples

3GPP JSON Patch example:

Assume the following patch is applied to an object tree, that has one "SubNetwork" instance only. The first operation requests to create a "ManagedElement". This operation is successful. The second operation requests to create a "HuhuFunction" object under the new object. The "HuhuFunction" is not known to the MnS Producer. The operation fails. The third operation fails as well, since it requests to create a new object under an object, that does not exist.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-json-patch+json [ { "op": "add", "path": "/ManagedElement=ME1", "value": { "id": "ME3", "objectClass": "ManagedElement", "attributes": { "userLabel": " Berlin NW 3", "vendorName": "Company XY", "location": "Spandau" } } }, { "op": "add", "path": "/ManagedElement=ME1/HuhuFunction=HUHUF1", "value": { "id": "XYZF1", "objectClass": "XyzFunction", "attributes": { "attrA": "xyz", "attrB": 771 } } }, { "op": "add", "path": "/ManagedElement=ME3/XyzFunction=XYZF1", "value": { "id": "XYZF2", "objectClass": "XyzFunction", "attributes": { "attrA": "abc", "attrB": 772 } } } ]

HTTP/1.1 207 Multi-Status Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json [ { "op": "add", "path": "/ManagedElement=ME1/HuhuFunction=HUHUF1", "value": { "id": "XYZF1", "objectClass": "XyzFunction", "attributes": { "attrA": "xyz", "attrB": 771 } }, "status": 400, "type": "VALIDATION_ERROR", "reason": "NEW_OBJECT_CLASS_NAME_INVALID", "title": "The class of the new object to be created is not known." }, { "op": "add", "path": "/ManagedElement=ME3/XyzFunction=XYZF1", "value": { "id": "XYZF2", "objectClass": "XyzFunction", "attributes": { "attrA": "abc", "attrB": 772 } }, "status": 422, "type": "REQUEST_OBJECTS_MISMATCH", "reason": "NEW_OBJECTS_PARENTS_NOT_FOUND", "title": "The parent object of the new object to be created does not exist." } ]

3GPP JSON Merge Patch example:

Assume the "ManagedElement" with the identifier "ME3" does not exist. Then the following message requests to create two new objects under a non-existent object. This request cannot be satisfied.

PATCH /SubNetwork=SN1 HTTP/1.1 Host: example.org Content-Type: application/3gpp-merge-patch+json { "id": "SN1", "ManagedElement": [ { "id": "ME3", "XyzFunction": [ { "id": "XYZF1", "objectClass": "XyzFunction", "attributes": { "attrA": "xyz", "attrB": 771 } }, { "id": "XYZF2", "objectClass": "XyzFunction", "attributes": { "attrA": "abc", "attrB": 772 } } ] } ] }

The error message may look like:

HTTP/1.1 422 Unprocessable Content Date: Tue, 06 Aug 2019 16:50:26 GMT Content-Type: application/json [ { "type": "REQUEST_OBJECTS_MISMATCH", "reason": "NEW_OBJECT_PARENT_NOT_FOUND", "title": "The object, below which new objects are requested to be created, does not exist.", "badObjects": [DC=example.org,Subnetwork=SN1,ManagedElement=ME3"] } ]

End of modifications