3GPP TSG-SA5 Meeting #148e 	S5-233394

Electronic meeting, Online, 17 -25 April 2023

- Source:	ZTE Corporation

- Title:	Clarification on Issue 4.7 Monitoring intent fulfilment information

- Document for:	Approval

Agenda Item:	6.7.3.2

# 1	Decision/action requested

In this box give a very clear / short /concise statement of what is wanted.

# 2	References

[1]	3GPP draft TR 28.912: "Management and orchestration; Study on enhanced intent driven management services for mobile networks v1.2.0".

[2]	3GPP TS 28.312 "Management and orchestration; Intent driven management services for mobile networks v17.0.1"

# 3	Rationale

# 4	Detailed proposal

1st Change

- 4.7	Issue#4.7:	Monitoring intent fulfilment information

- 4.7.1		Description

In TS 28.312 [2], MnS Producer can deploy or configure corresponding managed entities to satisfy the intent. During the execution of intention, MnS Producer continuously monitors intent fulfilment information. One scenario is that an intent may take a long period of policies execution to be satisfied. During the execution of the intent, something may happen, such as a new intent coming, a change in the execution environment. The MnS producer needs to monitor the intent fulfilment information so that policies can be adjusted in time. Another scenario is that the intent contains temporal information, such as  how long the consumer could wait before the intent is initially satisfied, or how long MnS consumer needs MnS producer to satisfy the intent continuously, which refers to maximum duration. For example, MnS consumer can express the expectation that the coverage targets for certain radio network needs to be ensured in one hour, which means MnS producer have one hour to take/adjust actions to ensure the expected coverage targets for radio network. When the maximum unfulfilled duration is specified, the MnS producer may optimize its intent fulfilment plan, e.g., to decide different actions to be applied on the managed entities or to use different managed services, so that the intent can be fulfilled within the duration.The MnS producer needs to response the intent fulfilment information to the MnS consumer at the beginning and during the execution of the intent.

REQ-Intent_execution_monitoring-CON-1: The intent driven MnS Producer should have the capability to allow the authorized MnS consumer to request intent fulfilment information periodically or event-triggered.

REQ-Intent_execution_monitoring-CON-2: The intent driven MnS Producer should have the capability to inform the authorized MnS consumer about intent fulfilment information periodically or event-triggered.

REQ-Intent_execution_monitoring-CON-3: The intent driven MnS Producer should have the capability to allow the authorized MnS Consumer to provide the deadline expressing the maximum duration before the intent is initially satisfied, the latest time point by which the intent should be satisfied, or the time cycleduring which the intent being satisfied continuously.

2nd Change

### 4.7.2		Potential solutions

See clause 4.5.2, intent fulfilment information is one kind of intent report information.

To support REQ-Intent_execution_monitoring-CON-3, an attribute “fullfillmentDeadline” needs to be introduced in Intent IOC to represent MnS’s requirements for the maximum unfulfilled duration before the intent is initially satisfied, and an attribute “fulfillmentTimecycle” needs to be introduced in Intent IOC to represent MnS requirements for the time cycle during which the intent is fulfilled continuously.

The fullfillmentDeadline and fulfilmentTimecycle may be attributes of the whole intent or of one of the expectations of the intent.

End of changes