3GPP TSG-SA5 Meeting #148 	S5-233470

- Electronic meeting, online, 17 Apr. – 21 Apr. 2023

- Source:	Huawei

- Title:	pCR TR 28.863 Issue # 3: Solution of KQIs for Video Uploading

- Document for:	Approval

Agenda Item:	6.9.3.3

# 1	Decision/action requested

This contribution is proposed to be approved.

# 2	References

[1]	 3GPP TS 28.863v0.5.0: "Study on Key Quality Indicators (KQIs) for 5G service experience"

# 3	Rationale

The document gives the KQIs for Video Uploading.

# 4	Detailed proposal

It is proposed to apply the following changes as below in TR 28.863 [1].

Start of the first change

### 4.3.2	Potential solutions

#### 4.3.2.b	Potential solution #X: KQIs for Video Uploading

##### 4.3.2.b.1	Introduction

In this potential solution, KQIs for Video Uploading is provided.

##### 4.3.2.b.2	Description

E2E uplink delay (UlDelayE2E)

E2E uplink delay indicates the end-to-end delay from a packet to be sent to the receiver of the packet. The measurements of E2E delay in TS28.554 6.3.1.8 in some certain scenarios applies. The KQI unit is ms and the KQI type is Float.

Delay jitter (Jitter)

Delay jitter indicates the deviation from the desired value to the actual value when assessing time parameters. The KQI unit is ms and the KQI type is Float.

Uplink mean throughput (UlThr)

Uplink mean throughput indicates the average throughput of uplink. The measurements in clause 6.3 in TS28.554 in some certain scenarios applies. The KQI unit is kbps and the KQI type is float.

Uplink peak throughput (UlPeakThr)

Uplink peak throughput indicates the maximum throughput that can be reached instantaneously. This indicator reflects the transmission capability to some extent. The KQI unit is kbps and the KQI type is Float.

Uplink average packet loss rate (UlAvePlr)

Uplink average packet loss rate indicates the average ratio of the number of uplink loss packets to the number of total packets. The KQI unit is a percentage and the KQI type is Float.

Uplink burst packet loss rate (ULBurstPlr)

Uplink burst packet loss rate indicates the burst ratio of the number of uplink loss packets to the number of total packets. The KQI unit is a percentage. The KQI unit is percentage and the KQI type is Float.

End of the first change