3GPP TSG-SA5 Meeting #148e 	S5-233544

Electronic meeting, Online, 17 -25 April 2023

Title:	LS on LS Reply on O-RAN – Transport Network Slicing Enhancement IM/DM TS28.541

Response to:	LS S-2022003 on LS on O-RAN – Transport Network Slicing Enhancement IM/DM TS28.541  from O-RAN

Release:	<release_identity(s)>

Work Item:	<work_item_name> (<work-item_code>)

Source:	SA5

To:	O-RAN WG9

Cc:	SA, SA3, RAN

Contact person:	Jan Groenendijk

jan.groenendijk@ericsson.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	TS 28.541 [Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3: v 18.3.1].

TS 28.531 [Management and orchestration; Provisioning, v 18.2.0]

TS 32.300 [Telecommunication management; Configuration Management (CM); Name convention for Managed Objects]

# 1	Overall description

3GPP SA5 would like to thank O-RAN for their interest and use of the SA5 specifications for network slicing. This reply documents the actions for SA5 and answers from SA5.

- O-RAN WG9 kindly asks 3GPP SA5 to take the information above into account and provide their feedback.

The feedback is provided under b and c.

- O-RAN WG9 kindly asks 3GPP SA5 to provide guideline on how transport network information may be reflected in TS 28.541 6.3.18.2  EP Transport.

The IOC for EP_Transport is documented in TS 28.541 clause 6.3.18 including stage 2 definitions. Existing attributes include identifiers of a transport connection point;

- VLAN ID

- MPLS Tag

- Segment Id

Additional identifiers are currently being discussed specifically an Attachment Circuit Id (See RFC 4664).

- O-RAN WG9 kindly asks 3GPP SA5 to provide guideline on how to extract the following key values information on per network slice fashion, such as for example:

- Unique ID of the slice [local attribute or proxy_class]. Index, related to SR/Label

- The NetworkSlice is uniquely identified by the Distinguished Name (DN) and can be used as key to find the associated NetworkSliceSubnet, the epTransportRef is a property of the NetworkSliceSubnet. The DN naming structure is defined in TS 32.300.

- Throughput constraints: Uplink_Demand and Downlink_Demand, Traffic direction

  - Uplink_Demand and Downlink_Demand are captured as properties of the NetworkSliceSubnet as maxULDataVolume and maxDLDataVolume. Attribute descriptions can be found in Clause 6.4.

- Latency constraint demand

  - Both ulLatency and dlLatency are captured as a requirement. Attribute descriptions can be found in clause 6.4.

- Transport protection constraint demand

  - The n6Protection attribute is captured as requirement and the description can be found in clause 6.4.

- Service Level Agreement Availability value

  - Service Level Agreement Availability is captured as a requirement, the Availability attribute description can be found in clause 6.4.

- Maximum MTU size [Payload of interest]

  - The maximum MTU packet size is defined for uplink in maxULPacketSize and for downlink maxDLPacketSize, descriptions can be found in clause 6.4.

- Maximum Jitter constraint

  - The jitter attribute is captured as requirement and the description can be found in clause 6.4.

- Temporary values [activation / deactivation time stamp – activate values from XX:XX to YY:YY]

  - To determine periods of activity and inactivity the values of administrative and operational state could be used.

- Security / Policy constraint to encode the propagation of the connectivity per domain/geography

  - Could you clarify the question?

# 2	Actions

To O-RAN WG9

ACTION: 	3GPP SA5 asks O-RAN WG9 to take the provided information into account.

# 3	Dates of next TSG SA WG 5 meetings

SA5#149		22 - 26 May 2023		Berlin (Germany)

SA5#150		21 - 25 August 2023	Goteborg (Sweden)