3GPP TSG-SA5 Meeting #149 	S5-233899

Berlin, Germany, 22 - 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0174 | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Procedure of Network Slice Instance Deallocation to support asynchronous operations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNETSLICE_PRO |  |  |  |  |  | Date: |  |  | 2023-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | S5-233155 was endorsed with the following: - Introduce a solution using NetworkSliceController IOC to monitor progress and result for network slice (subnet) allocation and deallocation as an enhancement to support async mode in 3GPP Rel-18. - The solution introduced above will support both network slice lifecycle management using allocate/deallocate operation and CRUD based operation. Network Slice and Network Slice Subnet LCM operations are long running and hence need to be asynchronous. The changes in this CR introduces support for asynchronous mode of operations for Procedure of Network Slice Instance Deallocation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update Procedure of Network Slice Instance Deallocation to support asynchronous or long running operations. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Support of shared objects as well as asynchronous or long running operations for Network Slice Instance Deallocation is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  | x |  | O&M Specifications |  |  |  | TS28.541 CR 0800 TS28.623 CR 0214 |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.4	Procedure of Network Slice Instance Deallocation

Figure 7.4-1 depicts the procedure of deallocating a NetworkSlice MOI by the network slice management service provider to satisfy the NetworkSlice MOI deallocation request received from an authorized consumer.

Figure 7.4-1: Network slice instance deallocation procedure

1a-1) If CRUD operations are not visible to the Network Slice Management Service Consumer (NSMS_C), then Network Slice Management Service Provider (NSMS_P) receives a DeallocateNsi request (see DeallocateNsi operation defined in clause 6.5.3) from NSMS_C indicating that the NetworkSlice MOI is no longer needed for the given requirements i.e ServiceProfile.

1a-2) The NSMS_P sends the DeallocateNsi response (see DeallocateNsi operation defined in clause 6.5.3) to NSMS_C.

1b-1)	If CRUD operations are visible to the NSMS_C, then NSMS_P receives a deleteMOI request (see deleteMOI operation defined in TS 28.532 [8]) from NSMS_C with network slice controller (see NetworkSliceController IOC and its attribute defined in TS 28.541[6])).

1b-2)	NSMS_P identifies the DN of NetworkSlice MOI from the attribute networkSliceRef and value of attribute serviceProfileId of the NetworkSliceController MOI to be deleted.

1b-3)	NSMS_P updates attribute networkSliceControllerRefList of NetworkSlice MOI (identified in step 1b-2) attribute to remove the DN of NetworkSliceController MOI to be deleted.

1b-4)	If the NSMS_C has subscribed for the notification, then NSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSlice MOI.

1b-5)	NSMS_P deletes NetworkSliceController MOI.

1b-6)	NSMS_P sends the response to NSMS_C for the deleteMOI request.

1b-7)	If the NSMS_C has subscribed for the notification, then NSMS_P sends notification for deletion (see notifyMOIDeletion operation defined in TS 28.532 [8]) of NetworkSliceController MOI.

2) The NSMS_P may decide to delete or modify the NetworkSlice MOI (identified in step 1b-2).

3a-1)	If NSMS_P decides to delete the NetworkSlice MOI, then it deletes the NetworkSlice MOI.

3a-2)	If the NSMS_C has subscribed for the notification, then NSMS_P sends notification for deletion (see notifyMOIDeletion operation defined in TS 28.532 [8]) of NetworkSlice MOI.

3a-3)	NSMS_P invokes the procedure of network slice subnet instance deallocation as described in clause 7.5.

3b-1) If NSMS_P decides to modify the NetworkSlice MOI, procedure of network slice subnet instance modification procedure as described in clause 7.6.

3b-2)	NSMS_P modifies the NetworkSlice MOI to update the serviceProfileList (using the serviceProfileId identified in step 1b-2) and may update other attributes.

3b-3)	If the NSMS_C has subscribed for the notification, then NSMS_P sends notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSlice MOI.

End of change