3GPP TSG-SA5 Meeting #149 	S5-233937

Berlin, Germany, 22 – 26 May 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 Use case and requirements for Efficient ML entity re-training |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of efficient ML entity re-training has been discussed and agreed upon (clause 5.1.4.2.1 in TR 28.908). This CR is to normatively propose the use case and requirements for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use case description and requirements have been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use case “Efficient retraining of ML entities” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.B (new clause added), 6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

#### 6.2.2.B	Efficient ML entity re-training

During inference phase of ML entity, a lot of potentially new data samples are processed and some of them are useful for a re-training and should therefore be labelled and added to the training set. However, using all inference data samples for retraining generates a high effort for data labelling, data provision (signaling) and model training, and this effort is not feasible in environments with limited resources.

In the case that re-training/model adaptation is performed at the entity with low processing power or limited energy the amount of data used for re-training and the time needed for model to converge towards the optimal performance is critical and needs to be minimized.

To optimize the re-training, it is necessary to reduce of the number of training samples, by extracting the most supporting data samples for re-training from all data samples (pool samples) that have been used for inference.

Start of First change

### 6.2.3	Requirements for ML training

Table 6.2.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing the consumer to request ML training. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the consumer to specify the inference type of the ML model entity to be trained. | ML training requested by consumer (clause 6.2.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result (including the location of the trained ML model entity) to the consumer. | ML training requested by consumer (clause 6.2.2.1), /ML training initiated by producer (clause 6.2.2.2) |
| REQ-ML_SELECT-01 | 3GPP management system shall have the capability to enable an authorized consumer to discover the characteristics of available models including the contexts under which each of the models was trained. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have the capability to enable an authorized consumer to select an ML model. | ML models and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have the capability to enable an authorized consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2.2.1), ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have the capability to enable an authorized consumer to request for information and be informed about the available alternative models of differing complexity and performance. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have the capability to enable an authorized consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML model/entity to the consumer. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the training of specific ML models or ML entities, e.g. to modify the characteristics of the request or to delete a request. | ML training requested by consumer (clause 6.2.2.1),Managing ML Training Processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-02 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | ML training requested by consumer (clause 6.2.2.1), Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have the capability to enable an authorized consumer (e.g. the function/entity different from the function that generated a request for ML model/entity training) to request for a report on the outcomes of a specific training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have the capability to enable an authorized consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have the capability to enable the MLT function to report to any authorized consumer about specific ML Training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML-inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of AI/ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-AIML_RETRAIN-0X: | The 3GPP management system shall have the capability for the authorized MnS consumer to request and receive from the ML inference producer the most supporting data samples for re-training from all data samples (pool samples) that have been used for ML inference. | Efficient ML entity re-training (clause 6.2.2.B) |
| REQ-AIML_RETRAIN-0Y: | The 3GPP management system shall have the capability for the ML MnS inference producer to provide to the ML training MnS producer the most supporting data samples for re-training. | Efficient ML entity re-training (clause 6.2.2.B) |

End of Changes