3GPP TSG-SA5 Meeting #149 S5-234427 # Berlin, Germany, 22 – 26 May, 2023 CR-Form-v12.2. CHANGE REQUEST. 28.552; CR; 0439; rev; -; Current version:; 18.2.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests..

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New measurement “Successful RRC connection establishments per RAT” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-05-08 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Successful RRC connection establishments in 5GS has been already defined in TS 28.552 clause 5.1.1.15.2. Similarly, Successful RRC connection establishments in EPS has been already defined in TS 32.425 clause 4.1.1.2. In this CR, it is proposed to define successful RRC connection establishments per RAT at the combined (EPS+5GS) NE. This performance measure is used to derive RRC retainability KPI. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a new performance measure capturing the aggregated number of RRCSetupComplete messages received by gNB following a RRC connection setup request as well as the aggregated number of RRCConnectionSetupComplete messages received by eNB following a RRC connection establishment request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Without this performance measure, NR and EUTRAN RRC retention rate KPI (TS 28.554 CR0125) cannot be derived. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.15.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 28.554 CR0125 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# >>>>BEGINNING OF CHANGES<<<<

##### 5.1.1.15.X	Successful RRC connection establishments per RAT

a)	This measurement provides the aggregated number of RRC setup complete for UEs per RAT.

b)	CC.

c) Aggregated number of RRCSetupComplete messages over causes received by gNB following a RRC connection setup request as well as the aggregated number of RRCConnectionSetupComplete messages over causes received by eNB following a RRC connection establishment request.

d)	Each measurement is an integer value.

e)	Combi.RRC.ConnEstabSucc		Combined

Combi.RRC.ConnEstabSucc.E		EPS

Combi.RRC.ConnEstabSucc.5G	5GS

f) 	Concatenation of EUtranCell (EUtranCellFDD and EUtranCellTDD) and NRCellCU.

g)	Valid for packet switched traffic

h)	Combined NE of EPS/5GS.

i)	This measurement is to support the RRC Retention KPI.

# >>>>END OF CHANGES<<<<