3GPP TSG-SA5 Meeting #149	S5-234503

# Berlin, Germany, 22-26 May 2023

# Source:	Huawei, CMCC

# Title:	pCR 28.830 Add description failure prediction enhancement

# Document for:	Approval

Agenda Item:	6.7.4.2

1	Decision/action requested

The group is asked to discuss and approve the proposal.

2	References

[1]	SP-220153: "New SID on Fault Supervision Evolution"

[2]	TR 28.830 Fault supervision evolution

[3]	TS 28.104: "Management Data Analytics (MDA)"

[4]	TS 28.532: "Generic management services"

[5]	ITU-T Recommendation X.733 (1992) | ISO/IEC 10164-4: 1992, Information technology – Open Systems Interconnection – Systems Management – Alarm reporting function.

3	Rationale

This tdoc discusses potential enhancements for failure prediction in FSEV study [1, 2].

Failure of service and network functions may occur during the network operation. It is necessary to predict potential failures and prevent more severe impacts. The MDA capability of failure prediction has been specified in TS 28.104 [3], the analytics output are as follows:

-	failurePredictionObject;

-	potentialFailureType;

-	eventTime;

-	issueID;

-	perceivedSeverity;

The definition of potentialFailureType in the “Analytics output” lists some example values: "Operational Violation", "Physical Violation" and "Time Domain Violation" with reference to alarmType described in TS 28.532 [4]. The attributes need to be defined more concretely in TS 28.104 [3].

The AlarmType in TS 28.532 [4] used the definitions in ITU-T Rec. X. 733 [5], including the following attributes:

Communications Alarm:

An alarm of this type is associated with the procedure and/or process required conveying information from one point to another (ITU-T Recommendation X.733 [4]).

Processing Error Alarm:

An alarm of this type is associated with a software or processing fault (ITU T Recommendation X.733 [4]).

Environmental Alarm:

An alarm of this type is associated with a condition related to an enclosure in which the equipment resides (ITU-T Recommendation X.733 [4]).

Quality of Service Alarm:

An alarm of this type is associated with degradation in the quality of a service (ITU T Recommendation X.733 [4]).

Equipment Alarm:

An alarm of this type is associated with an equipment fault (ITU-T Recommendation X.733 [4]).

Integrity Violation:

An indication that information may have been illegally modified, inserted or deleted.

Operational Violation:

An indication that the provision of the requested service was not possible due to the unavailability, malfunction or incorrect invocation of the service.

Physical Violation:

An indication that a physical resource has been violated in a way that suggests a security attack.

Security Service or Mechanism Violation:

An indication that a security attack has been detected by a security service or mechanism

The additionalInformation in TS 28.532 [4] may include some vendor defined alarm types.

In TR 28.830 [2], the definition of “error” and “failure” were proposed to an an appropriate TS:

Error: A state of the system different from the correct system state as defined by the service specification. An error may or may not lead to a service failure. An error has a begin and end time.

Failure: A state of inability to deliver the correct service as defined by the service specification. A service failure may be the result of an error or a poor service function design.

With the introduction of the definition of “error” and “failure”, it is expected that the management system should differentiate failure and error type. The MDA capability of failure prediction may help to indicate the satandardized value of alarmType. It may further indicate whether the prediction is failure or error through correlation analysis. The potentialFailureType may reference to the standardized AlarmTye. An attribute additionalInfo may also be used to indicate whether the alarmType is failure or error.

Proposal 1: It is proposed to include the potential enhancements for failure prediction analytics output.

4	Detailed proposal

1st Change

5.X	Key Issue #X: Potential enhancements for fault related analysis

5.X.1	Description

Editor’s note: This clause provides a description of the key issue.

If a potential fault/failure is predicted and reported to the consumer, the consumer would like to know the consequence. More analysis information on 3GPP system is helpful for the consumer to perform more proper actions, e.g., performance degradation analysis and predictions, KPI anomaly analysis and predictions, etc.

For example, the impacts on 3GPP system may not be perceived significantly in densely populated urban areas if there are overlapping coverage when a few sites encounter faults. However, service outage may occur due to faults in a site if there are few overlapping coverage in suburban areas. If this kind of information can be provided, the different handlings may be performed by the consumer.

Based on the topology relationship of the concerned managed objects, the impacted scope and degree are evaluated and provided. It may contain the following aspects:

-	scope and severity level of the impacted managed objects: the service types and coverage areas which are affected by the fault, e.g., the VoNR, URLLC service types, number of affected PDU sessions, and the coverage areas etc;

-	impacted managed objects: the managed objects which are affected by the fault, e.g. network slice, network slice subnet, network elements, network functions, number of affected gNBs and cells etc;

-	Rootcause types, name and descriptions of different types of rootcauses, etc.

-	Recommended actions for recovery;

5.X.2	Potential solutions

5.X.2.a	Potential solution #1: Failure prediction enhancement

5.X.2.a.1	Introduction

Editor's Note:	This clause describes briefly the potential solution at a high-level.

Some potential enhancements to failure prediction are provided. The concrete potentialFailureType is defined as standardized value of the alarmType in TS 28.532 [5], and failure or error cases may be further indicated with a separate attribute.

5.X.2.a.2	Description

Editor's Note:	This clause further details the potential solution and any assumptions made.

Failure of service and network functions may occur during the network operation. It is necessary to predict potential failures and prevent more severe impacts. The MDA capability of failure prediction has been specified in TS 28.104 [x], the analytics output are as follows:

-	failurePredictionObject;

-	potentialFailureType;

-	eventTime;

-	issueID;

-	perceivedSeverity;

With the optimization of concepts of error, failure and fault, the MDA capability of failure prediction should be extended to include failure and error cases. An error can arise as a result of unforeseen operating conditions or due to a fault within the system, subsystem or component being considered. An error may or may not lead to a service failure. For example, failure of the primary system with redundancy and back up may not result in service failure because the secondary system can take over to provide service. This is an error but not a failure. The potential failure or error may be service outage, network functions failure or error etc. Performance or KPI degradation may or may not lead to failure or error so that there may or may not be service impact. Failure or error may be caused by faults in RAN and/or CN network. Failure or error may be caused by faults in the managed object itself or other related managed objects. The scope of the failure or error may be small or large depending on the underline faults.The failure prediction capability also demarcate problems and provide corresponding recommendations to avoid failures or errors and reduce the impacts on services.

The potentialFailureType in the failure prediction analytics output need to be defined more concretely in TS 28.104 [3]. An attribute additionalDescription may also be used to indicate whether the alarmType is failure or error. The analytics output may help the consumer to take further actions. It may also coordinate with fault supervision for identification and analysis of alarm types and probable causes etc.

Table x: Additional analytics output for failure prediction

| Attribute Name | Description |
| --- | --- |
| potentialFailureType | Indication of type of issues that can cause the failures. The attribute values refer to the standardized values of alarmType defined in TS 28.532 [5]. |
| additionalDescription | Optionally provide more information for the potentialFailureType. It further indicates whether the potentialFailureType is failure or error. Following the standardized values of alarmType defined in TS 28.532 [5], the values of additionalDescription are: Communications failure, Communications error, Quality of Service failure, Quality of Service error, Equipment failure, Equipment error, Operational failure, Operational error. NOTE: Not all potentialFailureTypes contain the failure and error cases. |

5.X.3	Conclusion - Impact on normative work

Editor's Note:	This clause provides the conclusion from the aspect of impact on normative work.

End of change