3GPP TSG-SA5 Meeting #149 	S5-234622

Berlin, Germany, 22 - 26 May 2023

# Source:	Ericsson LM

# Title:	Rel-18 pCR 28.827 Adding user reporting solution in clause 7.6

# Document for:	Approval

Agenda Item:	7.5.2

# 1	Decision/action requested

Include the proposed changes in TR 28.827.

# 2	References

[1]		3GPP TR 28.827: "Study on 5G charging for additional roaming scenarios and actors"

# 3	Rationale

Adding solution in clause 7.6 for reconciliation using user reported usage information reporting.

# 4	Detailed proposal

First change

#### 7.6.4.x	Solution #6.x: Using user reported usage information reporting

##### 7.6.4.x.1	General

A possible solution for key issue #6a covering requirements REQ-CH_RECON-01, would be to have selected users or IoT devices, collect usage information and report this to the home MNO. The reporting can be done through an app provided by the operator (or an internal function in the UE or IoT device) and collected by the charging or billing system. The home MNO would have agreement with specific users for the collection and reporting.

##### 7.6.4.x.2	Reference architecture

This could reuse the distributed charging trigger function mechanism described in TS 32.240 [12] annex D. The UE would have Accounting Metrics Collection (AMC), which could be implemented as an app, which would do usage information reporting towards the Accounting Data Forwarding (ADF). The ADF could then be a function residing in an AF, within the home MNO.

The AF holding the CTF (ADF) could interact with the MNO’s charging or billing system. The UE CTF (AMC) would use another new reference point Nyy to communicate with the AF CTF (ADF). The Nyy would rely on 5GC user plane for transport (i.e., an "over IP" reference point).

Figure 7.6.4.x.2-1: Reconciliation using UE usage reporting in reference point representation

Figure 7.6.4.x.2-2: Reconciliation using UE measurements in service-based interface representation

Both the interface between UE and AF as well as between AF and billing or charging system could be considered outside the scope of 3GPP SA5.

##### 7.6.4.x.3	Potential message flows

This assumes that the UE would be configured with the address of the AF which holds the CTF (ADF) to be able to know where to send the reports.

Figure 7.6.4.x.3-1: Message flows for UE usage reporting - PEC

1.	UE is configured with reporting to the AF, this would include AF address and reporting criteria. The reporting triggers criteria could be based on e.g., PLMNs, volumes. Both the AF address, and reporting criteria could be provisioned to the UE when the UE is allowed to do local breakout roaming.

2.	UE (CTF-AMC) decides that reporting criteria are met, according to the configuration, the UE (CTF-AMC) creates the corresponding usage information report.

3.	UE triggers the usage reporting procedure sends the usage information report to the AF (CTF-ADF).

4.	AF decides that it should report the information received form the UE to a billing or charging system, this can be due to a reception of a usage information report from the UE (CTF-AMC), or some internal trigger e.g., after several received reports from the UE.

5.	AF (CTF-ADF) sends the collected information to the billing or charging system.

6.	The Billing or charging system can then use this in the reconciliation process.

End of changes