3GPP TSG-SA5 Meeting #151	S5-236600

Xiamen, China, 09 - 13 October 2023									Revision of SP-22XXXX

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.256 | CR | 0017 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Remove the triggers in charging data message |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP o. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the service description of Nchf_ConvergedCharging, the triggers definition in the Nchf_Convergedcharging Open API and ASN.1 is incomplete, only SMF triggers is supported. To keep alignment with the triggers specifed in the stage 3, the corresponding triggers in the serivce specifications should be removed, for example: - For AMF charging, trigger is reported in the charging data message specifed in the TS 32.256 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove the Triggers in Charging Data message for AMF Charging. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Nchf_Convergedcharging is not aligned in the stage 2 and stage 3. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2,6.1.1.3,6.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

First change

6.1.1.2	Charging Data Request message

Table 6.1.1.2.1 illustrates the basic structure of a Charging Data Request message from the AMF as used for 5G connection and mobility converged charging.

Table 6.1.1.2.1: Charging Data Request message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Subscriber Identifier | OM | Described in TS 32.290 [57] In case Subscriber Identifier (i.e. SUPI) is not present (for emergency service), the User Equipment Info in table 6.2.1.2.1. shall be present for identifying the user. |
| NF Consumer Identification | M | Described in TS 32.290 [57] |
| NF Functionality | M | Described in TS 32.290 [57] |
| NF Name | OC | Described in TS 32.290 [57] |
| NF Address | OC | Described in TS 32.290 [57] |
| NF PLMN ID | OC | Described in TS 32.290 [57] |
| Charging Identifier | - | This field is not applicable. |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in TS 32.290 [57] |
| One-time Event Type | OC | Described in TS 32.290 [57] |
| Notify URI | - | This field is not applicable. |
| Service Specification Information | OC | Described in TS 32.290 [57] |
| Triggers | - | This field is not applicable.. |
| Multiple Unit Usage | OC | Described in TS 32.290 [57]. |
| Rating Group | M | Described in TS 32.290 [57] |
| Requested Unit | OC | Described in TS 32.290 [57] |
| AMF Identifier | OC | This field holds the AMF identifier. |
| AMF Charging Profile | OC | This field holds the applicable AMF Charging Profile |
| Registration Charging Information | OM | This field holds the registration specific information described in clause 6.2.1.2 |
| N2 connection Charging Information | OM | This field holds the N2 connection specific information described in clause 6.2.1.3 |
| Location Reporting Charging information | OM | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

6.1.1.3	Charging data response message

Table 6.1.1.3.1 illustrates the basic structure of a Charging Data Response message from the CHF as used for 5G connection and mobility converged charging.

Table 6.1.1.3.1: Charging Data Response message contents

| Information Element | Category for converged charging | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in TS 32.290 [57] |
| Invocation Timestamp | M | Described in TS 32.290 [57] |
| Invocation Result | OC | Described in TS 32.290 [57] |
| Invocation Sequence Number | M | Described in TS 32.290 [57] |
| Session Failover | OC | This field is not applicable. |
| Triggers | - | This field is not applicable.. |
| Multiple Unit Information | OC | This field is applicable for ECUR. |
| Result Code | OC | Described in TS 32.290 [57] |
| Rating Group | OM | Described in TS 32.290 [57] |
| Granted Unit | OC | Described in TS 32.290 [57] |
| Validity Time | OC | Described in TS 32.290 [57] |
| AMF Charging Profile | OC | This field holds the AMF Charging Profile to be applied |
| Location Reporting Charging information | Oc | This field holds the Location Reporting specific information described in clause 6.2.1.4 |

Next change

6.2.3	Detailed message format for converged charging

The following clause specifies per Operation Type the charging data that are sent by AMF for 5G connection and mobility converged charging.

The Operation Types are listed in the following order: I (Initial)/T (Termination)/E (Event). Therefore, when all Operation Types are possible it is marked as ITE. If only some Operation Types are allowed for a node, only the appropriate letters are used (i.e. IT or E) as indicated in the table heading. The omission of an Operation Type for a particular field is marked with "-" (i.e. I-E). Also, when an entire field is not allowed in a node the entire cell is marked as "-".

Table 6.2.3.1 defines the basic structure of the supported fields in the Charging Data Request message for AMF converged charging.

Table 6.2.3.1: Supported fields in Charging Data Request message

| Information Element |  | Functionality of AMF | Reg. |  | N2 cnt |  | Loc. Report. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Supported Operation Types | I/T/E |  | E |  | E |  |
| Session Identifier |  |  | ITE |  | E |  | E |  |
| Subscriber Identifier |  |  | ITE |  | E |  | E |  |
| NF Consumer Identification |  |  | ITE |  | E |  | E |  |
| Charging Identifier |  |  | - |  | - |  | - |  |
| Invocation Timestamp |  |  | ITE |  | E |  | E |  |
| Invocation Sequence Number |  |  | ITE |  | E |  | E |  |
| Retransmission Indicator |  |  | - |  | - |  | - |  |
| One-time Event |  |  | --E |  | E |  | E |  |
| One-time Event Type |  |  | --E |  | E |  | E |  |
| Notify URI |  |  | - |  | - |  | - |  |
| Service Specification Information |  |  | ITE |  | E |  | E |  |
| Triggers |  |  | - |  | - |  | - |  |
| Multiple Unit Usage |  |  | ITE |  | - |  | - |  |
| AMF Identifier |  |  | ITE |  | E |  | E |  |
| AMF Charging Profile |  |  | I-E |  | E |  | E |  |
| Registration Charging Information |  |  |  |  |  |  |  |  |
| Registration Message type |  |  | ITE |  | - |  | - |  |
| User Information |  |  | ITE |  | - |  | - |  |
| UE 5GMM Core Network Capability |  |  | ITE |  | - |  | - |  |
| MICO Mode Indication |  |  | ITE |  | - |  | - |  |
| SMS Supported Indication |  |  | ITE |  | - |  | - |  |
| Access Type |  |  | ITE |  | - |  | - |  |
| RAT Type |  |  | ITE |  | - |  | - |  |
| TAI List |  |  | ITE |  | - |  | - |  |
| User Location Info |  |  | ITE |  | - |  | - |  |
| PSCell Information |  |  | ITE |  | - |  | - |  |
| UE Time Zone |  |  | ITE |  | - |  | - |  |
| Mobility Restrictions |  |  | ITE |  | - |  | - |  |
| Requested NSSAI |  |  | ITE |  | - |  | - |  |
| Allowed NSSAI |  |  | ITE |  | - |  | - |  |
| Rejected NSSAI |  |  | ITE |  | - |  | - |  |
| NSSAI mapping list |  |  | ITE |  | - |  | - |  |
| AMF UE NGAP ID |  |  | ITE |  | - |  | - |  |
| RAN UE NGAP ID |  |  | ITE |  | - |  | - |  |
| RAN Node Id |  |  | ITE |  | - |  | - |  |
| SNPN ID |  |  | ITE |  | - |  | - |  |
| CAG ID List |  |  | ITE |  | - |  | - |  |
| N2 Connection Charging Information |  |  |  |  |  |  |  |  |
| N2 Connection Message type |  |  | - |  | E |  | - |  |
| User Information |  |  | - |  | E |  | - |  |
| User Location Info |  |  | - |  | E |  | - |  |
| PSCell Information |  |  | - |  | E |  | - |  |
| UE Time Zone |  |  | - |  | E |  | - |  |
| RAT Type |  |  | - |  | E |  | - |  |
| AMF UE NGAP ID |  |  | - |  | E |  | - |  |
| RAN UE NGAP ID |  |  | - |  | E |  | - |  |
| RAN Node Id |  |  | - |  | E |  | - |  |
| Mobility Restrictions |  |  | - |  | E |  | - |  |
| Allowed NSSAI |  |  | - |  | E |  | - |  |
| NSSAI mapping list |  |  | - |  | E |  | - |  |
| RRC Establishment Cause |  |  | - |  | E |  | - |  |
| Location Reporting Charging information |  |  |  |  |  |  |  |  |
| Location reporting Message type |  |  | - |  | - |  | E |  |
| User Information |  |  | - |  | - |  | E |  |
| User Location Info |  |  | - |  | - |  | E |  |
| PSCell Information |  |  | - |  | - |  | E |  |
| UE Time Zone |  |  | - |  | - |  | E |  |
| Presence Reporting Area Information |  |  | - |  | - |  | E |  |
| RAT Type |  |  | - |  | - |  | E |  |

Table 6.2.3.2 defines the basic structure of the supported fields in the Charging Data Response message for AMF converged charging.

Table 6.2.3.2: Supported fields in Charging Data Response message

| Information Element | Functionality of AMF | Reg. | N2 cnt | Loc. Report. |
| --- | --- | --- | --- | --- |
|  | Supported Operation Types | ITE | E | E |
| Session Identifier |  | ITE | E | E |
| Invocation Timestamp |  | ITE | E | E |
| Invocation Result |  | ITE | E | E |
| Invocation Sequence Number |  | ITE | E | E |
| Session Failover |  | I-- | - | - |
| Triggers |  | - | - | - |
| Multiple Unit information |  | - | - | - |
| AMF Charging Profile |  | I-E | E | E |
| Location Reporting Charging information. |
| Location reporting Message type |  | - | - | - |
| User Information |  | - | - | - |
| User Location Info |  | - | - | - |
| PSCell Information |  | - | - | - |
| UE Time Zone |  | - | - | - |
| Presence Reporting Area Information |  | - | - | E |
| RAT Type |  | - | - | - |

End of change