3GPP TSG-SA5 Meeting #151	S5-236657

Xiamen, China, 9 - 13 October 2023 									revision of S5-236461

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to draft CR TS 28.105 ML inference emulation NRM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case of ML inference emulation has been discussed and agreed (in TR 28.908). This CR is to normatively propose the solution for the mentioned use case. |  |  |  |  |  |  |  |  |
| Summary of change: |  | sage 2 solution has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No support for the use case “ML inference emulation” |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.V(new clauses added) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 7.3	Information model definitions for AI/ML operational phases

### 7.3.2	Information model definitions for ML emulation Phase

#### 7.3.2.1	Class diagram

##### 7.3.2.1.1	Relationships

@startuml

scale max 350 height

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class ManagedEntity <<ProxyClass>>

class MLInferenceEmulator <<InformationObjectClass>>

class MLInferenceEmulationProcess << InformationObjectClass >>

class MLEntity << InformationObjectClass >>

class MLInferenceEmulationRequest << InformationObjectClass >>

ManagedEntity "1" *-- "*" MLInferenceEmulator: <<names>>

MLInferenceEmulator "1" -r-> "*" MLEntity

MLInferenceEmulator "1" *-- "*" MLInferenceEmulationProcess : <<names>>

MLEntity "1" -d-> "1" MLInferenceEmulationRequest

MLInferenceEmulator "1" *-- "*" MLInferenceEmulationRequest: <<names>>

MLInferenceEmulationRequest "1" -r-> "1" MLInferenceEmulationProcess

MLEntity "1" --> "1" MLInferenceEmulationProcess

note left of ManagedEntity

Represents the following IOCs:

SubNetwork or

ManagedFunction

end note

@enduml

Figure 7.G.1.1-1: NRM fragment for ML inference emulation Control

##### 7.3.2.1.2	Inheritance

@startuml

skinparam ClassStereotypeFontStyle normal

skinparam ClassBackgroundColor White

skinparam shadowing false

skinparam monochrome true

hide members

hide circle

'skinparam maxMessageSize 250

class Top <<InformationObjectClass>>

class ManagedFunction <<InformationObjectClass>>

class MLInferenceEmulator << InformationObjectClass >>

class MLInferenceEmulationProcess  << InformationObjectClass >>

class MLInferenceEmulationRequest << InformationObjectClass >>

ManagedFunction <|-- MLInferenceEmulator

Top <|-- MLInferenceEmulationProcess

Top <|-- MLInferenceEmulationRequest

@enduml

Figure 7.G.1.2-1: ML inference emulation Inheritance Relations

#### 7.3.2.2	Class definitions

##### 7.3.2.2.1 	MLInferenceEmulator

###### 7.3.2.2.1.1	Definition

This IOC represents the properties of MLInferenceEmulator.

Each MLInferenceEmulator is a managed object instantiable from the MLInferenceEmulator information object class and name-contained in either a Subnetwork, a ManagedFunction. The MLInferenceEmulator is a type of managedFunction, i.e., the MLInferenceEmulator is a subclass of and inherits the capabilities of a ManagedFunction

- The MLInferenceEmulator shall be associated with one or more MLEntity(s). E.g., the MLEntity(s) associated with MLInferenceEmulator may be associated via a list of MLEntityId(s). The MLEntityId should specify the MLEntity and any specific version thereof.

- The MLInferenceEmulator contains one or more instances of MLInferenceEmulationProcess, where each MLInferenceEmulationProcess is instantiated following or in response to a received request to create the MLInferenceEmulationProcess ManagedObjectInstance (MOI). Alternatively, the MLInferenceEmulator may allow to instantiate one or more MLInferenceEmulationRequest(s) and as such may name-contain one or more MLInferenceEmulationRequest(s)

- MLInferenceEmulator is associated with one or more mLInferenceEmulationReport(s) generated for any one or more MLInferenceEmulationProcess(s).

- Each MLInferenceEmulator may indicate the resources consumed by ongoing ML Inference emulation processes. This may be indicated by a percentage or number of available resources (e.g., 30% CPU or 3 GB).

7.3.2.2.1.2		Attributes

The MLInferenceEmulator IOC includes attributes inherited from ManagedFunction IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLEntityList | M | T | F | F | F |
| Attributes related to Role |  |  |  |  |  |
| mLInferenceEmulationReportRef | O | T | F | F | F |

###### 7.3.2.2.1.3	Attribute constraints

None.

###### 7.3.2.2.1.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3.2.2.2 	MLInferenceEmulationRequest

###### 7.3.2.2.2.1	Definition

This IOC represents the properties of MLInferenceEmulationRequest.

For each request to undertake Inference emulation, a consumer may create a new MLInferenceEmulationRequest on the MLInferenceEmulator, i.e., MLInferenceEmulationRequest shall be an information object class that is instantiated for each request for Inference emulation.

- Each MLInferenceEmulationRequest is associated to exactly one MLEntity.

- The RequestStatus is written by the MLInferenceEmulator when there is a change in status of the Inference emulation progress. The RequestStatus is an enumeration with the values: "Pending", "Triggered", "Suspended", "Served".

- The reporting requirements contained in the MLInferenceEmulationRequest must be mapped to an existing MLInferenceEmulationProcess instance or create a new instance as needed.

7.3.2.2.2.2		Attributes

The MLInferenceEmulationRequest IOC includes attributes inherited from Top IOC (defined in TS 28.622[30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceEmulationRequestId | M | T | F | F | F |
| mLEntityID | M | T | F | F | F |
| RequestStatus | M | T | T | F | T |

###### 7.3.2.2.2.3	Attribute constraints

None.

###### 7.3.2.2.2.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

##### 7.3.2.2.3	MLInferenceEmulationProcess

###### 7.3.2.2.3.1	Definition

This IOC represents the properties of MLInferenceEmulationProcess.

For each MLEntity to be executed under Inference emulation, a MLInferenceEmulationProcess is required. The MLInferenceEmulationProcess may be instantiated for each MLEntity or may apply to multiple MLEntity(s), i.e., the MLInferenceEmulationProcess is associated with one or more MLEntity(s).

MLInferenceEmulationProcess can be instantiated directly by an authorized consumer. Alternatively, MLInferenceEmulationProcess can be instantiated by MLInferenceEmulator in response to a MLInferenceEmulationRequest instantiated directly by an authorized consumer. In this latter case, the MLInferenceEmulationProcess shall be associated with one or more MLInferenceEmulationRequest(s). Correspondingly, the MLInferenceEmulationRequest(s) and the MLInferenceEmulationProcess s are conditionally mandatory in that at least one of them must be associated with an instance of MLInferenceEmulationProcess .

- Each MLInferenceEmulationProcess may have a status attribute used to indicate the level of success of the MLInferenceEmulationProcess. For example, in the case where a MLInferenceEmulationProcess is instantiated for exactly one MLEntity and one AI/MLinference, the status may reflect the status of the internal job (e.g., training) executed for that MLEntity.

- The reporting requirements contained in the MLInferenceEmulationProcess must be mapped to an existing MLInferenceEmulationProcess instance or create a new instance as needed.

The MLInferenceEmulationProcess also represents the capability of compiling and delivering reports and notifications about MLInferenceEmulator or its associated MLInferenceEmulationRequest(s), and MLInferenceEmulationProcess s. The MLInferenceEmulator shall be associated with one or more instances of MLInferenceEmulationProcess ,

- The MLInferenceEmulationProcess may report on one or more MLInferenceEmulationRequest(s) or one or more MLInferenceEmulationProcess(s)

The MLInferenceEmulationProcess shall be associated with one or more mLInferenceEmulationReport(s),

7.3.2.2.3.2		Attributes

The MLInferenceEmulationProcess IOC includes attributes inherited from Top IOC (defined in TS 28.622 [30]) and the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceEmulationProcessId | M | T | F | F | F |
| mLEntityId | M | T | F | F | F |
| progressStatus | M | T | T | F | T |
| Attributes related to Role |  |  |  |  |  |
| MLInferenceEmulationRequestRef | O | T | T | F | T |
| mLInferenceEmulationReportRef | O | T | F | F | F |

###### 7.3.2.2.3.3	Attribute constraints

None.

###### 7.3.2.2.3.4	Notifications

The common notifications defined in clause 7.6 are valid for this IOC, without exceptions or additions.

## 7.4	Data type definitions

##### 7.4.N 	MLInferenceEmulationReport <<dataType>>

###### 7.4.N.1	Definition

This dataType represents the properties of MLInferenceEmulationReport.

- MLInferenceEmulator may generate one or more mLInferenceEmulationReport(s),

- Each MLInferenceEmulationReport is associated to one or more MLEntity(s).

- All mLInferenceEmulationReport(s) must be associated with the MLInferenceEmulationProcess instance, i.e., the MLInferenceEmulationProcess provides/instantiates reports about MLEntity(s) or about the MLInferenceEmulationProcess that is associated with the MLEntity(s) for which inference emulation is requested and/or executed. It may also report on specific MLInferenceEmulationRequest(s).

7.4.N.2		Attributes

The MLInferenceEmulationReport includes the following attributes:

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| mLInferenceEmulationReportId | M | T | F | F | F |
| mLEntityList | M | T | F | F | F |
| Attributes related to Role |  |  |  |  |  |
| MLInferenceEmulationRequestId | O | T | F | F | F |
| MLInferenceEmulationProcessId | M | T | F | F | F |

#### 7.4.N.3	Attribute constraints

None.

#### 7.4.N.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

## 7.5	Attribute definitions

### 7.5.1	Attribute properties

Table 7.5.1-1

| Attribute Name | Documentation and Allowed Values |  | Properties |
| --- | --- | --- | --- |
| mLEntityId |  | It identifies the ML entity. It is unique in each MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| candidateTrainingDataSource |  | It provides the address(es) of the candidate training data source provided by MnS consumer. The detailed training data format is vendor specific. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| inferenceType |  | It indicates the type of inference that the ML model supports. allowedValues: the values of the MDA type (see 3GPP TS 28.104 [2]), Analytics ID(s) of NWDAF (see 3GPP TS 23.288 [3]), types of inference for RAN-intelligence, and vendor's specific extensions. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| areConsumerTrainingDataUsed |  | It indicates whether the consumer provided training data have been used for the ML model training. allowedValues: ALL, PARTIALLY, NONE. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| usedConsumerTrainingData |  | It provides the address(es) where lists of the consumer-provided training data are located, which have been used for the ML model training. allowedValues: N/A. | type: String multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingRequestRef |  | It is the DN(s) of the related MLTrainingRequest MOI(s). allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| trainingProcessRef |  | It is the DN(s) of the related MLTrainingProcess MOI(s) that produced the MLTrainingReport. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| trainingReportRef |  | It is the DN of the MLTrainingReport MOI that represents the reports of the ML training. allowedValues: DN. | type: DN (see TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| lastTrainingRef |  | It is the DN of the MLTrainingReport MOI that represents the reports for the last training of the ML model. allowedValues: DN. | type: DN (see 3GPP TS 32.156 [13]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| confidenceIndication |  | It indicates the confidence (in unit of percentage) that the ML model would perform for inference on the data with the same distribution as training data. allowedValues: { 0..100 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityList |  | It describes the list of MLEntity. | type: MLEntity multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| trainingRequestSource |  | It describes the entity that requested to instantiate the MLTrainingRequest MOI. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLTrainingRequest.requestStatus |  | It describes the status of a particular ML training request. allowedValues: NOT_STARTED, TRAINING_IN_PROGRESS, CANCELLING, SUSPENDED, FINISHED, and CANCELLED. | type: Enum multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLTrainingProcessId |  | It identifies the training process. It is unique in each instantiated process in the MnS producer. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| priority |  | It indicates the priority of the training process. The priority may be used by the ML training to schedule the training processes. Lower value indicates a higher priority. allowedValues: { 0..65535 }. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| terminationConditions |  | It indicates the conditions to be considered by the MLtraining MnS producer to terminate a specific training process. allowedValues: MODEL UPDATED_IN_INFERENCE_FUNCTION, INFERENCE FUNCTION_TERMINATED, INFERENCE FUNCTION_UPGRADED, INFERENCE_CONTEXT_CHANGED. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| progressStatus |  | It indicates the status of the ML training process. allowedValues: N/A. | type: ProcessMonitor (see TS 28.622 [12]) multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mLEntityVersion |  | It indicates the version number of the ML entity. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceRequirements |  | It indicates the expected performance for a trained ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| modelperformanceTraining |  | It indicates the performance score of the ML entity when performing on the training data. allowedValues: N/A. | type: ModelPerformance multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| mLTrainingProcess.progressStatus.progressStateInfo |  | It provides the following specialization for the "progressStateInfo" attribute of the "ProcessMonitor" data type for the "MLTrainingProcess". When the ML training is in progress, and the " mLTrainingProcess.progressStatus.status " is equal to "RUNNING", it provides the more detailed progress information. allowedValues for " mLTrainingProcess.progressStatus.status " = "RUNNING": - COLLECTING_DATA - PREPARING_TRAINING_DATA - TRAINING The allowed values for " mLTrainingProcess.progressStatus.status " = "CANCELLED" are vendor specific. | Type: String multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| inferenceOutputName |  | It indicates the name of an inference output of an ML entity. allowedValues: the name of the MDA output IEs (see 3GPP TS 28.104 [2]), name of analytics output IEs of NWDAF (see TS 23.288 [3]), RAN-intelligence inference output IE name(s), and vendor's specific extensions. | Type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| performanceMetric |  | It indicates the performance metric used to evaluate the performance of an ML entity, e.g. "accuracy", "precision", "F1 score", etc. allowedValues: N/A. | Type: String multiplicity: 1 isOrdered: N/A isUnique: True defaultValue: None isNullable: False |
| performanceScore |  | It indicates the performance score (in unit of percentage) of an ML entity when performing inference on a specific data set (Note). The performance metrics may be different for different kinds of ML models depending on the nature of the model. For instance, for numeric prediction, the metric may be accuracy; for classification, the metric may be a combination of precision and recall, like the "F1 score". allowedValues: { 0..100 }. | Type: Real multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cancelRequest |  | It indicates whether the ML training MnS consumer cancels the ML training request. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the requestStatus is the "NOT_STARTED", " TRAINING_IN_PROGRESS", and "SUSPENDED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendRequest |  | It indicates whether the ML training MnS consumer suspends the /ML training request. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the requestStatus is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| cancelProcess |  | It indicates whether the ML training MnS consumer cancels the ML training process. Setting this attribute to "TRUE" cancels the ML training request. Cancellation is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| suspendProcess |  | It indicates whether the ML training MnS consumer suspends the ML training process. Setting this attribute to "TRUE" suspends the ML training request. Suspension is possible when the " mLTrainingProcess.progressStatus.status" is not the "FINISHED", "CANCELLING" or "CANCELLED" state. Setting the attribute to "FALSE" has no observable result. Default value is set to "FALSE". allowedValues: TRUE, FALSE. | Type: Boolean multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: FALSE isNullable: False |
| inferenceEntityRef |  | It describes the target entities that will use the ML entity for inference. | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| dataProviderRef |  | It describes the entities that have provided or should provide data needed by the ML entity e.g. for training or inference | Type: DN (see 3GPP TS 32.156 [13]) multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: True |
| areNewTrainingDataUsed |  | It indicates whether the other new training data have been used for the ML model training. allowedValues: TRUE, FALSE. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingDataQualityScore |  | It indicates numerical value that represents the dependability/quality of a given observation and measurement type. The lowest value indicates the lowest level of dependability of the data, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| decisionConfidenceScore |  | It is the numerical value that represents the dependability/quality of a given decision generated by the AI/ML inference function. The lowest value indicates the lowest level of dependability of the decisions, i.e. that the data is not usable at all. allowedValues: { 0..100 }. | Type: Real multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| expectedRuntimeContext |  | This describes the context where an MLEntity is expected to be applied or/and the RunTimeContext which is the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 0..1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| trainingContext |  | This specify the context under which the MLEntity has been trained. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| runTimeContext |  | This specifies the context where the MLmodel or entity is being applied. allowedValues: N/A | Type: MLContext multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| MLInferenceEmulationRequestID |  | It indicates an identifier for a specific instantiated MLInferenceEmulationRequest. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLInferenceEmulationProcessId |  | It indicates an identifier for a specific instantiated MLInferenceEmulationProcess. allowedValues: N/A. | type: String multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: True |
| mLInferenceEmulationReportId |  | It indicates an identifier for a specific MLInferenceEmulator report. allowedValues: N/A. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| emulatorResourceConsumption |  | It indicates the quantity of resources used by ongoing MLInferenceEmulationProcess(s), or the percentage / amount of available resources e.g., CPU. allowedValues: N/A. | type: integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| mLInferenceEmulationReportRefs |  | It indicates the DNs of set of reports generated on mLInferenceEmulation for different MLInferenceEmulationRequest(s) or MLInferenceEmulationProcess(s) allowedValues: N/A. | type: mLInferenceEmulationReportDN multiplicity: 1..* isOrdered: N/A isUnique: N/A defaultValue: 0 isNullable: False |
| NOTE: When the performanceScore is to indicate the performance score for ML training, the data set is the training data set. When the performanceScore is to indicate the performance score for ML validation, the data set is the validation data set. When the performanceScore is to indicate the performance score for ML testing, the data set is the testing data set. |  |  |  |

End of Changes