3GPP TSG-SA5 Meeting #151 	S5-236722

Xiamen, China, 9 - 13 October 2023

# Source:	Ericsson

# Title:	Update of triggers

# Document for:	Approval

Agenda Item:	7.4.1

# 1	Decision/action requested

Include the proposed changes in TS 28.203.

# 2	References

[1]		3GPP TS 28.203: "Network slice admission control charging in the 5G System (5GS)"

# 3	Rationale

Adding the possibility to abort charging for the CHF. Clarification of initial and termination trigger handling and counters handling by the NSACF.

# 4	Detailed proposal

First change

##### 5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details of charging information such as S-NSSAI.

Each trigger condition (i.e. chargeable event) defined for the Network Slice Admission Control converged charging functionality with the associated behaviour when they are met, is specified this document, and the basic trigger mechanism is specified in the 3GPP TS 32.290 [5].

For session based charging the following applies:

-	A Charging Data Request [Initial] is invoked by the NSACF towards the CHF when a trigger for "initial" is met for an S-NSSAI, and no charging session exists for this S-NSSAI.

-	A Charging Data Request [Termination] is invoked by the NSACF towards the CHF when a trigger for "termination" is met for an S-NSSAI.

-	A Charging Data Request [Update] is invoked by the NSACF towards the CHF, when any trigger which is not a trigger for "initial" nor a trigger for "termination" is met, and the charging session exists for an S-NSSAI.

Table 5.2.1.2.1.1 summarizes the set of default trigger conditions which shall be supported by the NSACF when charging is active for the corresponding NSACF functionality, with the associated Charging Data Request message sent from NSACF towards the CHF.

Table 5.2.1.2.1.1: Default trigger conditions in NSACF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Initial |  |  |  |  |  |
| Number of UEs threshold reached for initial | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Initial |
| Number of PDU sessions threshold reached for initial | - | Immediate | Not Applicable | Not Applicable |  |
| Change of charging conditions |  |  |  |  |  |
| Number of UEs threshold crossed upwards | - | Immediate | Yes | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of UEs threshold crossed downwards | - | Immediate | Yes | Yes | PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of PDU sessions threshold crossed upwards | - | Immediate | Yes | Yes | IEC, PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Number of PDU sessions threshold crossed downwards | - | Immediate | Yes | Yes | PEC: Charging Data Request [Event] SCUR: Charging Data Request [Update] |
| Quota management |  |  |  |  |  |
| Number of UEs quota threshold reached | - | Immediate | Not Applicable | Yes | SCUR: Charging Data Request [Update] |
| Number of PDU sessions quota threshold reached | - | Immediate | Not Applicable | Yes |  |
| Quota exhausted | - | Immediate | Not Applicable | Yes |  |
| Expiry of quota validity time | - | Immediate | Not Applicable | Yes |  |
| Expiry of quota holding time | - | Immediate | Not Applicable | Yes |  |
| Re-authorization request by CHF | - | Immediate | Not Applicable | No |  |
| Termination |  |  |  |  |  |
| Number of UEs threshold reached for termination | - | Immediate | Not Applicable | Not Applicable | SCUR: Charging Data Request [Termination] |
| Number of PDU sessions threshold reached for termination | - | Immediate | Not Applicable | Not Applicable |  |
| Abort charging request by CHF | - | Immediate | Not Applicable | No |  |
| Network slice termination | - | Immediate | Not Applicable | Not Applicable | PEC: Charging Data Request [Event] SCUR: Charging Data Request [Termination] |

Table 5.2.1.2.1.2 details the set of trigger conditions and NSACF behaviour.

Table 5.2.1.2.1.2: Chargeable events and their related actions in NSACF

| Chargeable event | Conditions | NSACF action |
| --- | --- | --- |
| Number of UEs threshold reached for initial | If no charging session exist for the S-NSSAI | Charging Data Request [Initial] with a possible request quota for later use. |
|  | If the charging session exists for the S-NSSAI | The trigger will be ignored |
| Number of PDU sessions threshold reached for initial | If no charging session exist for the S-NSSAI | Charging Data Request [Initial] with a possible request quota for later use. |
|  | If the charging session exists for the S-NSSAI | The trigger will be ignored |
| Number of UEs threshold crossed upwards | If the category is set to "immediate reporting" | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
|  | If the category is set to "deferred reporting" | Close the counts and start new counts with time stamps |
| Number of UEs threshold crossed downwards | If the category is set to "immediate reporting" | Charging Data Request [Update] Close the counts and start new counts with time stamps |
|  | If the category is set to "deferred reporting" | Close the counts and start new counts with time stamps |
| Number of PDU sessions threshold crossed upwards | If the category is set to "immediate reporting" | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
|  | If the category is set to "deferred reporting" | Close the counts and start new counts with time stamps |
| Number of PDU sessions threshold crossed downwards | If the category is set to "immediate reporting" | Charging Data Request [Update] Close the counts and start new counts with time stamps |
|  | If the category is set to "deferred reporting" | Close the counts and start new counts with time stamps |
| Number of UEs quota threshold reached |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Number of PDU sessions quota threshold reached |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Quota exhausted |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Expiry of quota validity time |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Expiry of quota holding time |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Re-authorization request by CHF |  | Charging Data Request [Update] with a possible request quota. Close the counts and start new counts with time stamps |
| Number of UEs threshold reached for termination | If no charging session exist for the S-NSSAI | The trigger will be ignored |
|  | If the charging session exists for the S-NSSAI, then if number of PDU sessions threshold is active and has not reached for termination | The trigger will be ignored |
|  | If the charging session exists for the S-NSSAI, then if number of PDU sessions threshold is active and has reached for termination | Charging Data Request [Termination] Close the counts with time stamps |
| Number of PDU sessions threshold reached for termination | If no charging session exist for the S-NSSAI | The trigger will be ignored |
|  | If the charging session exists for the S-NSSAI, then if number of PDU sessions threshold is active and has not reached for termination | The trigger will be ignored |
|  | If the charging session exists for the S-NSSAI, then if number of UEs threshold reached for termination is active and has reached for termination | Charging Data Request [Termination] Close the counts with time stamps |
| Abort charging request by CHF |  | Charging Data Request [Termination] Close the counts with time stamps |
| Network slice termination |  | Charging Data Request [Termination] Close the counts with time stamps |

End of changes