3GPP TSG-SA5 Meeting #151 	S5-236914

Xiamen, China, 9 - 13 October 2023

# Source:	China Mobile, Huawei

# Title:	Add basic principles for MBS session charging

# Document for:	Approval

Agenda Item:	7.4.5

1	Decision/action requested

Agree the basic principles for MBS session charging of TS 32.27x.

2	References

[1] 		3GPP TS 32.27x: "Charging management; 5G Multicast-broadcast Services charging".

3	Rationale

This contribution proposes basic principles for MBS session charging in TS 32.27x.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.27x [1].

First change

X.X.X.A	General

Converged charging may be performed by the MB-SMF interacting with CHF using Nchf specified in TS 32.290 [x] and TS 32.291 [y]. The MB-SMF shall be able to perform converged charging for each of the following:

-	Charging data related to MBS session;

-	Charging data related to service data flows within the MBS session.

The MB-SMF shall be able to report charging events to CDF for CDR generation.

The MB-SMF shall be able to perform convergent charging by interacting with CHF, for charging data related to MBS sessions. The Charging Data Request and Charging Data Response are exchanged between the MB-SMF and the CHF, based on SCUR scenarios specified in TS 32.290 [x]. The Charging Data Request is issued by the MB-SMF towards the CHF when certain conditions (chargeable events) are met.

The MB-SMF initiates a charging session with Charging Data Request/Response [Initial], updates the charging session with Charging Data Request/Response [Update], and terminates the charging session with Charging Data Request/Response [Termination].

Next change

X.X.X.B	Applicable Triggers in the MB-SMF

When a charging event is issued towards the CHF, it includes details such as MBS charging identifier and also containers identifying the volume count, with charging condition change information.

Each trigger condition (i.e. chargeable event) defined for the 5G data connectivity converged charging functionality with the associated behaviours when met is specified in this document and the basic trigger mechanism is specified in the TS 32.290[y].

Two categories of chargeable events are identified:

-	immediate report: chargeable events for which, when occurring, the current counts are closed and sent together with the charging data generated by the MB-SMF towards the CHF in a Charging Data Request. New counts are started by the MB-SMF.

-	deferred report: chargeable events for which, when occurring, the current counts are closed and stored together with the charging data generated by the MB-SMF. The stored counts will be sent to the CHF in next a Charging Data Request. New counts are started by the MB-SMF.

When more than one trigger condition to be met at same time (i.e. time stamp of triggers is the same) for the same count in the MB-SMF, the MB-SMF reports the used unit container with these triggers.

When a MBS session starts, and the converged charging is activated, the MB-SMF invokes a Charging Data Request [Initial] towards the CHF to get authorization to start based on the default triggers. The MB-SMF is optionally provided in a Charging Data Response [Initial] to override the default triggers, with a set of chargeable event triggers to be enabled, and the associated category (i.e. immediate or deferred report).

The triggers remain active until they are updated or disabled by subsequent Charging Data Response [Update] from the CHF or the MBS session is terminated.

Table X.X.X.B-X summarizes the set of default trigger conditions and their category which shall be supported by the MB-SMF. For "immediate report" category, the table also provides the corresponding Charging Data Request [Initial, Update, Termination] message sent from MB-SMF towards the CHF.

Table X.X.X.B-X: Default Trigger conditions in MB-SMF

| Trigger Conditions | Trigger level | Converged Charging default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Start of MBS Session. | MBS session | Immediate | Not Applicable | Not Applicable | Charging Data Request [Initial] |
| Change of Charging conditions |  |  |  |  | Charging Data Request [Update] |
| Connection established with NG-RAN | MBS session | Deferred | Yes | Yes |  |
| Connection released with NG-RAN | MBS session | Deferred | Yes | Yes |  |
| Connection established with UPF | MBS session | Deferred | Yes | Yes |  |
| Connection released with UPF | MBS session | Deferred | Yes | Yes |  |
| Quota management |  |  |  |  |  |
| Time threshold reached | MBS session | Deferred | No | Yes |  |
| Time quota exhausted | MBS session | Deferred | No | Yes |  |
| Limit per MBS session |  |  |  |  |  |
| Expiry of data time limit per MBS session | MBS session | Immediate | No | Yes |  |
| Expiry of data volume limit per MBS session | MBS session | Immediate | No | Yes |  |
| Expiry of limit of number of charging condition changes | MBS session | Immediate | No | Yes |  |
| End of MBS session | MBS session | Immediate | No | No | Charging Data Request [Termination] |

For converged charging, the following details of chargeable events and corresponding actions in the MB-SMF are defined in Table X.X.X.B-Y:

Table X.X.X.B-Y: Chargeable events and their related actions in MB-SMF

| Chargeable event | Conditions | MB-SMF action |
| --- | --- | --- |
| Start of MBS session |  | Charging Data Request [Initial]. |
| Connection established with NG-RAN | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection released with NG-RAN | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection established with UPF | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Connection released with UPF | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Time threshold reached | If the corresponding trigger is enabled | Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Time quota exhausted | If the corresponding trigger is enabled | Charging Data Request [Update] with a possible request quota Close the counts and start new counts with time stamps |
| Expiry of data volume limit per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Expiry of time limit per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| Expiry of a limit of number of charging condition changes per MBS session | If the corresponding trigger is enabled | Close the counts and start new counts with time stamps |
| End of MBS session |  | Charging Data Request [Termination] Close the counts with time stamps |

Next change

X.X.X.C	MBS session charging

Converged charging allows the MB-SMF to collect charging information related to data volumes of MBS data packets received from AF or MBSTF per MBS session.

If MBS session specific converged charging is supported, this is achieved by FBC charging, see clause X.X.X.B.

End of change