3GPP TSG-SA5 Meeting #151	S5-236329

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.531 | CR | 0220 | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.531 Clarify initiation of Procedure of network slice subnet instance deallocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2023-09-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Procedure of network slice subnet instance deallocation may be initated with DeallocateNssi operation (when the network slice subnet allocation was initiated using AllocateNssi operation) or using deleteMOI operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC). This is not mentioned in the description. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Description enhanced to clarify that Procedure of network slice subnet instance deallocation may be initated with DeallocateNssi operation (when the network slice subnet allocation was initiated using AllocateNssi operation) or using deleteMOI operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC). |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent/incomplete specification leads to confusion and incorrect implementation |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

## 7.5	Procedure of network slice subnet instance deallocation

Figure 7.5-1 depicts the procedure of deallocating a network slice subnet instance by the network slice subnet management service provider to satisfy the NSSI deallocation request received from an authorized consumer.

Figure 7.5-1: Network slice subnet instance deallocation procedure

1a-1) The network slice subnet management service provider (NSSMS_P) receives DeallocateNssi request (see DeallocateNssi operation defined in clause 6.5.4) from network slice subnet management service consumer (NSSMS_C) indicating that the NetworkSliceSubnet MOI is no longer needed for the given requirements i.e SliceProfile.

1a-2)	NSSMS_P identifies the DN of NetworkSliceSubnet MOI from the attribute networkSliceSubnetRef and value of attribute sliceProfileId from the NetworkSliceSubnetController MOI to be deleted.

1a-3)	NSSMS_P updates NetworkSliceSubnet MOI (identified in step 1b-2) attribute networkSliceSubnetControllerRef to remove the DN of NetworkSliceSubnetController MOI to be deleted.

1a-4)	NSSMS_P deletes NetworkSliceSubnetController MOI.

1a-5) NSSMS_P sends response (see DeallocateNssi operation defined in clause 6.5.4) of NSSI deallocation service to NSSMS_C.

1b-1)	NSSMS_P receives a deleteMOI request (see deleteMOI operation defined in TS 28.532 [8]) from NSSMS_C with network slice controller (see NetworkSliceSubnetController IOC and its attribute defined in TS 28.541[6])).

1b-2)	NSSMS_P identifies the DN of NetworkSliceSubnet MOI from the attribute networkSliceSubnetRef and value of attribute sliceProfileId from the NetworkSliceSubnetController MOI to be deleted.

1b-3)	NSSMS_P updates NetworkSliceSubnet MOI (identified in step 1b-2) attribute networkSliceSubnetControllerRef to remove the DN of NetworkSliceSubnetController MOI to be deleted.

1b-4)	NSSMS_P deletes NetworkSliceSubnetController MOI.

1b-5)	NSSMS_P sends the response to NSSMS_C for the deleteMOI request.

NOTE: Network slice subnet deallocation may be initiated using DeallocateNssi operation (when the network slice subnet allocation was initiated using AllocateNssi operation) as described in steps 1a or using deleteMOI operation (when the network slice subnet allocation was initiated using createMOI operation with NetworkSliceSubnetController IOC) as described in steps 1b.

2) The NSSMS_P may decide to delete or modify the NetworkSliceSubnet MOI (identified in step 1b-2).

3a)	If NSSMS_P decides to delete the NetworkSliceSubnet MOI, then it performs the following steps:

3a-1)	NSSMS_P invokes procedure of network slice subnet instance deallocation as described in clause 7.5 for each constituent NetworkSliceSubnet MOI(s) that is configured in attribute networkSliceSubnetRef of NetworkSliceSubnet MOI (identified in step 1b-2).

3a-2) For each constituent NF instance (ManagedFunction MOI DN that is configured in attribute of managedFunctionRef of NetworkSliceSubnet MOI identified in step 1b-2), NSSMS_P invokes procedure of NF instance deletion as described in clause 7.12 only if the NF is dedicated for the NetworkSliceSubnet MOI (identified in step 1b-2)  and not being used by any other NetworkSliceSubnet MOIs in the network, otherwise, NSSMS_P procedure of NF instance modification as described in clause 7.11.

3a-3) NSSMS_P invokes TN related coordination procedure with responsible manager as described in clause 7.9 if the NetworkSliceSubnet MOI consists of TN part.

3a-4) NSSMS_P invokes NS termination procedure if the NetworkSliceSubnet MOI contains virtualized part.

Note:	NS termination procedure is described in TS 28.526 [7].

3a-5)	NSSMS_P deletes the NetworkSliceSubnet MOI.

3b)	If NSSMS_P decides to modify the NetworkSliceSubnet MOI, then it performs the following sub steps:

3b-1)	For each constituent NetworkSliceSubnet MOI, NSSMS_P invokes the procedure of network slice subnet instance modificationas described in clause 7.6.

3b-2)	NSSMS_P modifies the NetworkSliceSubnet MOI to update the sliceProfileList (using the sliceProfileId identified in step 1b-2) and may update other attributes.

If the NSSMS_C has subscribed for the notification, then NSSMS_P sends notification for deletion (see notifyMOIDeletion operation defined in TS 28.532 [8]) and notification for the attribute value changes (see notifyMOIAttributeValueChanges operation defined in TS 28.532 [8]) of NetworkSliceSubnetController MOI and NetworkSliceSubnet MOI.

End of change