3GPP TSG-SA5 Meeting #151	S5-236372

# Xiamen, China, 9th Oct 2023 – 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0463 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Addition of measurements related to Number of successful RRC connection setups in relation to the time between successful RRC connection setup and last RRC connection release |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In E-UTRAN in the related spec 3GPP TS 32.425 dedicated Measurements related to Number of successful RRC connection setups in relation to the time between successful RRC connection setup and last RRC connection release are defined within the chapter 4.1.6. In NR even there is more urgency on energy saving and prolonging the UE’s battery life such measurement is missing in the related 3GPP TS 28.552 spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of “Number of successful RRC connection setups in relation to the time between successful RRC connection setup and last RRC connection release” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Operator cannot proactivelly tune the inactivity timer from the perspective of energy saving and prolonging the UE’s battery life. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.x.y (new), A.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

### 5.1.1.x	Inactivity timer

#### 5.1.1.x.y	Number of successful RRC connection setups in relation to the time between successful RRC connection setup and last RRC connection release

a)	This measurement provides the histogram as function of the number of successful RRC Connection setups and the time between the successful RRC Connection setup attempt and last RRC Connection release for the UE with the same S-TMSI and long inactivity timer configured.
It is not counted to the histogram in case last RRC Connection release of the UE with the same S-TMSI is caused by call drops or handover.

b)	CC.

c)	On receipt by the eNodeB of an RRCConnectionSetupComplete message from the UE. Each RRCConnectionSetupComplete message received is added to the relevant measurement per time bin for the time between the successful RRC connection setup and last RRC Connection release for the UE with the same S-TMSI.

d)	Each measurement is an integer value.

e)	The measurement name has the form RRCConn.Setup.TimeBin 
where TimeBin identifies the given bin from the histogram

f)	NRCellCU

g)	Valid for packet switched traffic.

h)	5GS

End of Modified Section

2nd Modified Section

# A.z	Evaluation of long inactivity timer

The services from the point of view the time between the newly successful RRC connection setup and the last RRC Connection Release can be divided into the two groups. First group is built from the services the time is quite long (tens of minutes or hours) while for the second group it is tens of seconds. Typical example of the service from the first group can be voice call with a common characteristic that data transmission is accumulated into the time period the call is active without any long interruptions and sufficiently long period between the two consequent calls from the same UE. A second type of service can be web browsing with a common characteristic that data transmission is accumulated into the smaller time periods the service is active i.e. with long interruptions between data transmissions.

In the past operators had only option to set the inactivity timer commonly for all kind of services. This may lead to the situation that for the services from the first group UE had to spend ineffectively longer time in RRC Connected state which had negative impact on UE’s battery live. On the other hand for the services from the second group the inactivity timer setting can be insufficient which then may lead to significant increase of the UE movements from RRC Idle to RRC Connected state.

Operators shall therefore accurately tune the inactivity timers to avoid some bottlenecks in the RRC Connection setup on one side and prolongation of the UE’s battery life on the second side. Especially regarding the services where long inactivity timer applies operator needs to have an option to estimate how intended prolongation of the long inactivity timer is going to decrease the number of RRC Connection Setups on one side and prolong the UEs’ state in RRC Connected state on second side, using the "Number of successful RRC connection setups in relation to the time between successful RRC connection setup and last RRC connection release".

End of Modified Section