3GPP TSG-SA5 Meeting #151 	S5-236419

Xiamen, China, 9 - 13 Oct 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.557 | CR | XXXX | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.557 Add use case and requirements for dedicated resource demand for Smart Grid Utilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | OAM_NPN_Ph2 |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The key issue of dedicated resource demand for Smart Grid Utilities has been studied in clause 5.3 of TR 28.907, and is recommended to be introduced in the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed to add use case and requirements for dedicated resource demand for Smart Grid Utilities |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The resource isolation for verticals will be missing as one of the Rel-18 NPN features. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clause 5.1.0.X, 5.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

### 5.1.0	Generic use cases

#### 5.1.0.X	Dedicated resource demand for Smart Grid Utilities

Smart Grid is a representative utility with NPN. The power grid business is mainly divided into two working categories: production control and information management. The production control can be further divided into safety zone I and safety zone II. The information management also can be further divided into safety zone III and safety zone IV, see clause 5.9 of TR 22.867 [X]. Different safety zones support different energy applications and have distinct communication requirements, which results in the requirement of dedicated and shared network resource for verticals.

For 5G system, the physical isolation communication service means dedicated core network element with dedicated computing resources and dedicated radio resource e.g. PRB pool, spectrum etc. The logical isolation communication service on the other hand may be supported by dedicated logical network resources (e.g., network slice instances).

Table 5.1.0.X: The relations between isolation-sharing on physical and logical network resources

| Relations | Logical isolation | Logical sharing |
| --- | --- | --- |
| Physical isolation | The two situations exist at the same time | The two situations could not co-exist |
| Physical sharing | The two situations could exist at the same time | The two situations could exist at the same time |

The relations between isolation-sharing on physical and logical network resources is shown in Table 5.1.0.X.

- Physical isolation – Logical isolation: The two situations exist at the same time. For two application which respectively require the dedicated physical network resource to support the communication services, these two applications use dedicated logical network resources by default. For instance, Application 1 and Application 5 shown in Figure 5.1.0.X are physically isolated and therefore are logically isolated by default.

- Physical isolation – Logical sharing: The two situations could not co-exist. For two application which respectively require the dedicated physical network resource to support the communication services, these two applications cannot use shared logical network resources.

- Physical sharing – Logical isolation: The two situations could exist at the same time. For two applications which accept the shared physical network resource to support the communication services, these two applications can use dedicated logical network resources. For instance, Application 1 and Application 3 shown in Figure 5.1.0.X share physical network resources but are logically isolated.

- Physical sharing – Logical sharing: The two situations could exist at the same time. For two applications which accept the shared physical network resource to support the communication services, these two applications can use shared logical network resources. For instance, Application 1 and Application 2 shown in Figure 5.1.0.X share physical network resources and share logical network resources.

Figure 5.1.0.X: Example of dedicated-shared physical and logical groups

Different kinds of resource sharing requirements are applied to different safety zones, see Figure 5.1.0.X:

- The energy applications belonging to production control category (i.e. safety zones I and II) need to be physically isolated from other applications which do not belong to production control working category. Figure 5.1.0.X shows that applications belonging to Safety Zone I and II are divided into P-Group 1, while applications belonging to Safety Zone III and IV are divided into P-Group 2. Dedicated physical network resources are required to be allocated for each P-Group.

- The energy applications belonging to a same working category can be logically isolated each other. Figure 5.1.0.X shows that applications belonging to Safety Zone I/ II/ III/ IV are divided into L-Group 1/ 2/ 3/ 4 respectively, though Safety Zone I and II belongs to same working category and Safety Zone III and IV belongs to same working category. Dedicated logical network resources are required to be allocated for each L-Group.

NPN may be deployed to support the different use cases and service requirements for multiple Smart Grid applications. For example, multiple NPNs (e.g. multiple PNI-NPNs, each having on-prem NFs and a dedicated portion of PLMN NFs as a slice of the PLMN) may be deployed to meet the requirements of dedicated communication service. The shared communication service on the other hand may be supported by shared network element or shared network resource in one NPN for Smart Grid.

To provide management mechanism to assurance resource isolation for different sharing requirements, 3GPP management system may need to configure different network parameters whose purpose is for energy applications with different sharing requirements.

Therefore, 3GPP management system needs to have the capability to meet the distinct communication requirements with different resource sharing demands for some smart grid applications supported by NPNs.

Next change

### 5.2.1	Generic requirements for management of NPN

REQ-NPN-FUN-01 The 3GPP management system shall have the capability to monitor the performance measurements and KPIs associated with an NPN.

REQ-NPN-FUN-02 The 3GPP management system shall have the capability to provide the performance measurements and KPIs associated with an NPN to authorized entity, either NPN-SC (when NPN-SP and NPN-OP are both played by the same actor) or NPN-SP (when NPN-SP and NPN-OP are played by different actors).

REQ-NPN-FUN-03 The 3GPP management system shall have the capability to receive SLA requirements from authorized NPN-SC and then translating the SLA requirements into service and network resources related requirements.

REQ-NPN-FUN-04 The 3GPP management system shall have the capability to evaluate SLS assurance related to an NPN.

REQ-NPN-FUN-05 The 3GPP management system shall have the capability to restrict the exposure of management capabilities and corresponding managed resources to NPN-SC.

REQ-NPN-FUN-06 The 3GPP management system shall have the capability to support management capabilities exposure, which includes management capabilities of network provisioning, fault supervision and performance assurance to the authorized NPN-SC.

REQ-NPN-FUN-07 The 3GPP management system shall have the capability to provision both physical and virtual NPNs.

REQ-NPN-FUN-08 The 3GPP management system shall have the capability to provision different NPNs intended to different NPN-SCs.

REQ-NPN-FUN-09 The 3GPP management system shall have the capability to provision an NPN which serves different NPN-SCs.

REQ-NPN-FUN-10 The 3GPP management system shall offer the NPN-SC the ability to manage its own NPN(s) and its private slice(s) in the PLMN in a combined manner.

REQ-NPN-FUN-y The 3GPP management system shall have the capability to utilize physical isolated communication resource including core network and radio network to support dedicated communication service for vertical applications.

REQ-NPN-FUN-z The 3GPP management system shall have the capability to utilize logical isolated communication resource including core network and radio network to support shared communication service for vertical applications.

REQ-NPN-FUN-m The 3GPP management system shall have the capability to simultaneously support the management of multiple communication services with different network resource sharing requirements according to vertical management regulation.

End of changes