3GPP TSG-SA5 Meeting #151	S5-236468

# Xiamen, China, 9th Oct 2023 - 13th Oct 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0467 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR TS 28.552 Add measurements for NG-RAN Initiated paging sent by gNB-CU |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecomunication Corp., ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | PM_KPI_5G_Ph3 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For UE in RRC Inactive state, in order to further save the cost of paging, the concept of "RAN-based Notification Area" (RNA), which is smaller than TA, is introduced. In RRC_INACTIVE state, RAN paging is based on RNA. It is important for operator to know the successful paging ratio so as to optimize RNA planning and configuration. The exsiting paging measurements in TS28.552 have some limitations, such as missing successful paging measurement and the possibility of duplicate counts for paging on the same UE. Total paging records and successful paging records of NG-RAN initiated paging sent by the gNB-CU can improve the accurancy of paging stastics. So it needs to be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add measurements for NG-RAN Initiated paging sent by gNB-CU. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stastics of successful NG-RAN Initiated paging may be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.2, 5.1.1.27.X(new), 5.1.1.27.Y(new), A.70 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st change

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1], TS 23.501 [4] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1] and TS 23.501 [4].

CHO	Conditional Handover

CLI	Cross Link Interference

DAPS	Dual Active Protocol Stack

GP	Guard Period

HO	Handover

kbit	kilobit (1000 bits)

LHO	Legacy Handover

MA PDU	Multi-Access PDU

MN	Master Node.

MPQUIC	Multi-Path QUIC

MPTCP	Multi-Path TCP Protocol

NG-RAN	Next Generation Radio Access Network

PI	Performance Indicator

RNA	RAN-based Notification Area

SA PDU	Single-Access PDU

SN	Secondary Node.

SRS	Sounding Reference Signal

TEID	Tunnel Endpoint IDentifier

next change

5.1.1.27	Paging Measurement

5.1.1.27. X	Number of NG-RAN Initiated paging records sent by the gNB-CU

a) This measurement provides number of NR RAN initiated paging records sent by the gNB-CU.

b) CC.

c) Reception of a RAN PAGING message from NR RAN (See in TS 38.304 [37] and TS 38.423 [13]), and RAN PAGING message is sent by the gNB-CU on the F1 interface which is used to request the gNB-DU to page UEs. The IE in "CHOICE Paging Identity" of this message is "RAN UE Paging identity". (3GPP TS 38.473[6]).

d)  A single integer value. Multiple paging for the same UE are counted as one.

e)  PAG.SentNbrRanIntiated

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

5.1.1.27. Y	Number of successful NG-RAN Initiated paging sent by the gNB-CU

a) This measurement provides number of successful NR RAN Initiated paging sent by the gNB-CU.

b) CC.

c) On reception of RRC RESUME Request (RRCResumeRequest or RRCResumeRequest1) message from the UE, or reception of RETRIEVE UE CONTEXT REQUEST message (which carries the I-RNTI of the UE and the MAC-I verification passed) from another gNB through the Xn interface (3GPP TS 38.423) which indicating a successful NR RAN Initiated paging sent by the gNB-CU, the counter is stepped by 1.

d)  A single integer value.

e)  PAG.SentNbrRanIntiatedSucc

f)	NRCellCU

g)	Valid for packet switched traffic

h)	5GS

next change

A.70	Monitor of paging performance

In NR, Paging is under the control of the 5GC or NG-RAN (aka RAN initiated paging and CN initiated paging). When the 5GC wants to page (CN initiated paging) a UE, it has to page it in all cells that belong to the TA(s) to which the UE is registered.

The paging load per cell and gNB is an important measure for the operator as it allows the operator to properly dimension the resources for paging in the NR Cell and gNB.

At an NR Cell and gNB it makes sense to measure the number of discarded paging messages if this is due to some problem in the gNB, such as paging occasion overflow. In that scenario the periodicity of paging occasions can be reconfigured in order to ensure that all paging messages are transmitted by the gNB in the first available paging occasion, thereby avoiding paging delays and extended call setup delay.

Operators need to know when such an event occurs, in order to identify if the problem is at the NR cell or gNB level or not.

In addition to discarded paging records measurement, it is important to know total paging records received so that discarded paging records ratio can be derived.

Total number of paging records received is important in the sense that, it may be fine if the discarded paging records are high if discarded paging records ratio is small. On the other hand, it may be problematic if discarded paging records are low, if discarded paging records ratio turn out to be high.

In RRC_INACTIVE state, RAN paging is based on RNA which is introduced in order to resume a suspended RRC connection fastly. In this scenario, the gNB who triggered the RAN Paging can know if the paging is success or not by the response from UE or other gNB. Measurements for NG-RAN initiated paging sent by gNB-CU can avoid duplicate counts of RAN Paging and improve accuracy of RAN paging success rate statistics. The total paging number and successful paging number of NG-RAN initiated paging sent by the gNB-CU can derive RAN paging success rate, which can be used to optimize the RNA range configuration. It is helpful and important for operators.

End of changes