3GPP TSG-SA5 Meeting #151 	S5-236584

Xiaman, China, 09 - 13 October 2023

# Source:	Samsung

# Title:	Rel-18 CR 28.538 Federation Procedure.docx

# Document for:	Approval

Agenda Item:	6.5.8.3

# 1	Decision/action requested

For approval.

# 2	References

[1]	S5-235989 Rel-18 draftCR 28.538

# 3	Rationale

This contribution provides federation procedure for 28.538 which is very much needed to support GSMA requirements.

# 4	Detailed proposal

The following changes are proposed to [1].

First Modified Section

## 7.x	Federation management

### 7.x.1	Description

The clause contains procedures associated with federation management.

### 7.x.2	Edge Federation Establishment

Figure 7.x.2.1-1 depicts a procedure that describes how a federation relationship is established between LO and PO.

Figure 7.x.2.1-1: Federation Establishment

- In order to establish the edge federation with the PO (Producer), LO (Consumer) send createMOI request to instantiate EdgeFederation IOC.

- Producer send the response

- Producer instantiate the OperatorEdgeFederation IOC providing details on the offered EDN.

- Producer send notification for creation of the OperatorEdgeFederation IOC using notifyMOICreation as defined in 3GPP TS 28.532.

- Consumer reads the OperatorEdgeFederation MOI using GetMOIAttributes operation to know the offered EDN and other related information.

- Producer send the response

- Based on the offered EDN list consumer decides on to which EDN it want to accept.

- Consumer updates the value of acceptedEDN attribute, indicating the accepted EDN, using  ModifyMOIAttributes operation.

- Producer send the response. At this point the federation establishment is completed.

- Based on the accepted EDN information, producer instantiate OperatorEdgeDataNetwork IOC to represent the accepted EDN which is shared with the LO.

- The LO, behaving as Provisioning MnS Producer, instantiate the FederationIOC, OperatorFederation IOC and OperatorEdgeDataNetwork IOC.

- The Producer may decide to modify or delete the OperatorEdgeFederation MOI in order to update the federation relationship. The modification may be necessary in order to update the shared EDN. Whereas the deletion may be done in order to un-share an EDN.

- The notification of the update will be send to the consumer using the notification defined in 3GPP TS 28.532.

- The L-OP, behaving as Provisioning MnS Producer, update the FederationIOC, OperatorFederation IOC and OperatorEdgeDataNetwork IOC for each update notification received.

End Of Modified Section