3GPP TSG-SA5 Meeting #151	S5-236649

Xiamen, China, 9 - 13 October 2023 									revision of S5-236453

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draft CR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to Draft CR TS 28.105 Add ML performance metrics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shangai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-10-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use case on ML performance has been agreed in the TS including capabilities to select ML performance metrics. However, the specific metrics have not been introduced. This CR is to normatively propose the list of applicable performance metrics for ML data, training, testing and inference. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A list of performance metrics isintroduced |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There remians a gap on ML perfromance metrics |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.1.2.2, 7.3.1.2.3, 7.3.1.2.6, 7.3.1.2.7, 7.4.x (new clause added), 7.5.1, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of First change

## 7.4	Data type definitions

### 7.4.1	ModelPerformance <<dataType>>

#### 7.4.1.1	Definition

This data type specifies the performance of an ML entity when performing inference. The performance score is provided for each inference output.

#### 7.4.1.2	Attributes

Table 7.4.1.2-1

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| inferenceOutputName | M | T | F | F | T |
| performanceScore | M | T | F | F | T |
| performanceMetric | M | T | F | F | T |
| decisionConfidenceScore | O | T | F | F | T |

#### 7.4.1.3	Attribute constraints

None.

#### 7.4.1.4	Notifications

The notifications specified for the IOC using this <<dataType>> for its attribute(s), shall be applicable.

### 7.5.2	Constraints

None.

#### 7.5.2.X	performance metrics

The following performance Metrics may be applicable to ML performance. The list is not exhaustive, and others may be added.

Table 7.4.1.2-1

| Attribute Name | Documentation and Allowed Values | Type | For Data | For Training | For Testing | For Inference |
| --- | --- | --- | --- | --- | --- | --- |
| COUNT_INVALID_DROP | At training, if the source sends “None” or “Null” or “N/A” kind of values, those may be dropped before feeding the data to the ML training. This counter provides the absolute number of None/Null/Zero values dropped by the ML training for every source of input. E.g., if a performance metric is collected from 100 BTSs, for each BTS, one such counter should be maintained every time the data is used for AI/ML training | Array of Integers | T |  |  |  |
| PERCENT_INVALID_DROP | This is the ratio of the number of “Null/None/Zero/out-of-range/invalid” values received to the total number of samples received per collection interval expressed as a percentage. It is different form the missingness ratio as it includes those that are not missing but are invalid (e.g. when wrongly formatted) allowedValues: [0.0 - 100.0] This should be computed for every source of input data. | Array of Floating points numbers | T |  |  |  |
| COUNT _OUTLIERS | This is the anomaly detection based identification of number of outliers in input received from each data source. | integer (Array of Integers) | T |  |  |  |
| PERCENT_OUTLIERS | (Number of outliers / Total number of samples received per collection interval) * 100 allowedValues: [0.0 - 100.0] This should be computed for every source of input data. | Array of Floating points numbers | T |  |  |  |
| FIRST_ORDER_STATISTIC | Average value of feature of received input samples (e.g., each KPI) | Array of Floating points numbers | T |  |  |  |
| SECOND_ORDER_ STATISTIC | Mean-squared value of input samples received (e.g., each KPI) | Float | T |  |  |  |
| STD_FIRST_ORDER_ STATISTIC | Standard deviation of mean value (KPI) | Float | T |  |  |  |
| STD_SECOND_ORDER_ STATISTIC_ | Standard deviation of second order stat (KPI) | Float | T |  |  |  |
| P_VALUE_STATIONARITY | This is the p-value which indicates the stationarity of data. The more negative this value is, the more stationary is the feature of the data e.g., a counter or KPI in a dataset | array of <real> numbers | T |  |  |  |
| SKEWNESS_MEASURE | This is a measure of overall asymmetry of the distribution the data (also called the skewness of the data) | array of <real> numbers | T |  |  |  |
| MANHATTAN_DISTANCE | A quantitative measure for fairness that measures the average distance between the samples from two subsets of a dataset. | Float | T |  |  |  |
| MISSINGNES_RATIO | A quantitative measure for robustness that measures the percentage of missing values in the training dataset. | Float | T |  |  |  |
| DISPARATE_IMPACT | A quantitative measure for fairness that measures the ratio of rate of favourable outcome for the underrepresented group to that of the overrepresented group. | Float |  | T | T |  |
| AVERAGE_ODDS_DIFFERENCE | A quantitative measure for fairness that measures the average difference of false positive rate and true positive rate between underrepresented and overrepresented groups. | Float |  | T | T |  |
| MONOTONICITY | A quantitative metric in the range [0,1] that measures the effect of individual features on ML entity performance by evaluating the effect on ML entity performance by incrementally adding each feature in order of increasing importance. It is easier expressed on the scale [1,100] Allowed values 0-100 | Float |  | T | T |  |

End of Changes