3GPP TSG-SA5 Meeting #151 	S5-236663

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.312 | CR | 0155 | rev | - | Current version: | 18.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add description and potential requirement for intent report |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | IDMS_MN_ph2 |  |  |  |  |  | Date: |  |  | 2023-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Base on the description in clause 6.2.1.2.2, as follows: “Different MnS consumers can use the "getMOIAttributeValue" operation to query different attributes of the IntentReport <<IOC>> to obtain corresponding intent report information. Different MnS consumers can subscribe attribute value change notifications for IntentReport <<IOC>> to obtain the notification for different intent report information.” Some description and potential requirement is needed to add to generic use case clause 5.3.2 to be consistent with the description in clause 6.2.1.2.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add description to clarify MnS consumer can request different information type and different attribute value of intent report, and can also subscribe specific attribute value change notifications for intent report. Add new potential requirement enabling MnS consumer to subscribe specific attribute value change notifications for intent report to MnS producer. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The contents in Clause 5.3.2 is inconsistent with the contents in clause 6.2.1.2.2. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

5.3.2	Intent report

5.3.2.1	Introduction

The Fulfilment information (including the intentFulfilmentInfo, expectationFulfilmentInfo and targetFulfilmentInfo) are defined for the MnS consumer to monitor the intent fulfilment information. The intent report also can contain the current and optional predicted value for performance indicated by corresponding expectation targets (e.g. WeakRSRPRatio for the weakRSRPRatioTarget, Average UL RAN UE Throughput for aveULRANUEThptTarget), which can be used by MnS consumer to validate whether the intent is really fulfilled and to evaluate whether the intent (especially for expectation targets) needs to be updated if needed (improve the target value when corresponding target is fulfilled or reduce the target value when corresponding target is not fulfilled or not fulfilled with the reason of target confliction). Besides, intent conflict information and intent fulfilment feasibility check information sent by MnS producer to MnS consumer is another type of intent report information. So, following are the three types of information needs to be monitored by MnS consumer:

-	Intent Fulfilment information, which represents the properties of a specific fulfilment information for an aspect of the intent (i.e. either an expectation, a target or the whole intent), including fulfilment status and achieved values for targets.

-	Intent conflict information, which represents intent with conflict. The information includes conflict type (i.e., intent conflict, expectation conflict and target conflict) and possible solutions (e.g. intent deletion, intent modification).

-	Intent fulfilment feasibility check information, which indicates that the intent is feasible or infeasible. Intent fulfilment feasibility check information is provided after MnS producer automatically performs feasibility check when receiving the intent creation and modification request from MnS consumer.

Different MnS consumer may have different requirements for intent report (e.g. Some MnS consumer may want to have corresponding performance value information while others do not want. Different MnS consumer may want to calculate or monitor the performance value in different period). Different MnS consumers can request MnS producer to report different information type and different attribute value of intent report based on its requirements.

MnS consumers can also subscribe specific attribute value change notifications for intent report, MnS producer is triggered to inform new intent report to MnS consumer when MnS producer monitors that specific attribute value of intent report changes.

5.3.2.2	Requirements

REQ-Intent_Driven_MnS_Report-1: The intent driven MnS shall have the capability to enable the MnS consumer to request intent report information.

REQ-Intent_Driven_MnS_Report-2: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment information (including fulfilment status and achieved values for targets).

REQ-Intent_Driven_MnS_Report-3: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent conflict information.

REQ-Intent_Driven_MnS_Report-4: The intent driven MnS shall have the capability to enable the MnS consumer to obtain intent report information with intent fulfilment feasibility check information.

REQ-Intent_Driven_MnS_Report-5: The intent driven MnS shall have capability enabling MnS consumer to specify the content of the report.

REQ-Intent_Driven_MnS_Report-6: The intent driven MnS shall have capability enabling MnS consumer to configure the frequency of the intent reporting.

REQ-Intent_Driven_MnS_Report-7: The intent driven MnS shall have the capability enabling MnS consumers to receive reports, with different content and intervals according to its specified requirements.

REQ-Intent_Driven_MnS_Report-8: The intent driven MnS shall have the capability enabling MnS consumer to subscribe specific attribute value change notifications for intent report to MnS producer.

End of changes