3GPP TSG-SA5 Meeting #151 	S5-236691

Xiamen, China, 9 - 13 October 2023

# Source:	L.M. Ericsson Limited, Deutsche Telekom AG

# Title:	Discussion paper on slice agnostic service profile solutions

# Document for:	Agreement

Agenda Item:	6.5.2.2 (NSRULE)

# 1	Decision/action requested

The group is asked to agree the proposal in 4.

# 2	References

[1]	3GPP TS 28.541 "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3"

[2]	3GPP TS 28.531 "Management and orchestration; Provisioning"

[3]	3GPP TS 28.312 "Management and orchestration; Intent driven management services for mobile networks"

# 3	Rationale

## 3.1	Introduction

A single instance of a ServiceProfile should describe the service requirements from a CSC to a 3GPP system. The CSP operating the 3GPP system decides how to fulfil the service requirements, the service requirements should therefore be network slice agnostic.

Currently some of the ServiceProfile attributes use network slice in the name and in the attribute description while the service requirements should be agnostic on how they will be fulfilled. A network slice agnostic service profile could also be used

The re-use of serviceProfile is limited to network slice. In the discussion on sharing of network slices there has to be a clear separation between service (described by service profile) and the network slice (described by the network slice), using a mixed terminology is confusing.

The following attributes in the ServiceProfile use slice in the name or hint that the solution provided to fulfil the requirements based on network slice;

- networkSliceSharingIndicator,

- sST,

- dLThptPerSlice,

- uLThptPerSlice,

- sliceSimultaneousUse,

- nssaaSupport.

## 3.2	Different approaches

The following approaches are discusssed:

Approach 1: attributes with slice in the name are properties of a network slice,

Approach 2a: attributes with slice in the name are service requirements and should be renamed accordingly,

Approach 2b: attributes with slice in the name are service requirements and added with new names,

Approach 3: each attribute is individually assessed as either being a network slice property or service requirement

Approach 1

Since the attributes are applicable to network slice, an attribute being a setting (configuration), a function or a characteristics requirement the information is on network slice level, therefore the attributes that have slice in the name are property of a NetworkSlice. The attributes are removed from the ServiceProfile, added to the NetworkSlice as shown in the tabel below.

| Current attribute name | Attribute name as NetworkSlice property |
| --- | --- |
| networkSliceSharingIndicator | networkSliceSharingIndicator |
| sST | sST |
| dLThptPerSlice | dLThptPerSlice |
| uLThptPerSlice | uLThptPerSlice |
| sliceSimultaneousUse | sliceSimultaneousUse |
| nssaaSupport | nssaaSupport |

Approach 2

Since the attributes are service profile attributes, the name should reflect this therefore they should be renamed to be slice agnostic. The proposed names are shown in the tabel below.

| Current attribute name | Slice agnostic attribute name |
| --- | --- |
| networkSliceSharingIndicator | sharingIndicator |
| sST | serviceType |
| dLThptPerSlice | dLThpt_requirement |
| uLThptPerSlice | uLThpt_requirement |
| sliceSimultaneousUse | simultaneousUse |
| nssaaSupport | saaSupport_requirement |

Approach 3

Since the attributes are service profile attributes, the name should reflect this therefore they should be renamed to be slice agnostic. In this approach the current attributes are not replaced, and the new attributes are added with slice agnostic name, see table below.

| Current attribute name | Proposed attribute names |
| --- | --- |
| networkSliceSharingIndicator | networkSliceSharingIndicator |
| sST | sST |
| dLThptPerSlice | dLThptPerSlice |
| uLThptPerSlice | uLThptPerSlice |
| sliceSimultaneousUse | sliceSimultaneousUse |
| nssaaSupport | nssaaSupport |
|  | sharingIndicator |
|  | serviceType |
|  | dLThpt_requirement |
|  | uLThpt_requirement |
|  | simultaneousUse |
|  | saaSupport_requirement |

Approach 4

Each attribute is individually assessed as either being a service requirement or network slice property. An exampel is shown in the tabel below.

| Current attribute name | Slice agnostic attribute name | Attribute name as NetworkSlice property |
| --- | --- | --- |
| networkSliceSharingIndicator | sharingIndicator |  |
| sST | serviceType |  |
| dLThptPerSlice | dLThpt_requirement |  |
| uLThptPerSlice | uLThpt_requirement |  |
| sliceSimultaneousUse | simultaneousUse |  |
| nssaaSupport |  | nssaaSupport |

## 3.3	Recommended approach

Approach 3 is recommended as this solution is backward compatible unlike the other approaches.

# 4	Detailed proposal

The group is asked to endorse the recommendation in section 3.3 in the rational.