3GPP TSG-SA5 Meeting #151	S5-236759

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | draftCR | rev | - | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel18 Input to draft CR TS28.105 Training data effectiveness reporting use cases and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The use cases of “Training data effectiveness reporting and analytics” have been discussed and agreed upon in the study (clause 5.1.6 in TR 28.908). This CR is to normatively propose the use cases and requirements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the use cases description and requirements |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for the use cases. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.M (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 6.M	ML training Data Management

### 6.M.1	Description

In analytics solutions, Performance Measurements (PMs) and Fault Reports (FRs) from various network function are collected and analytics applied on the PMs and FRs to derive events (statistical insights and predictions). For most algorithms, the prediction accuracy depends upon the amount of available relevant historical data, motivating the need to store ever more data, which correspondingly increases the storage and processing resource requirements. However, not all recorded data is useful as the derived events, e.g., captured through analytics processes, may have loss of information OR misinformation e.g., with respect to time of the event.

For ML model training, a large amount of data instances does not necessarily add value, e.g., only a portion of training data contribute to actual model training, the other portion may be discarded with limited impact to model training performance.

### 6.M.2	Use cases

#### 6.M.2.x	Reporting Training data effectiveness

Training data effectiveness refers to the process of evaluating the contribution of a single data instance to model training process.

To train a ML model, high quality and large volume of training data instances are mandatory. The general practice is to collect as much data as possible and feed them to the ML model for pre-processing and training, in the hope to get high quality of trained model. This is usually done without considering the effectiveness of training data instance, e.g., some portion of training data instance contribute to model performance, some portions are not or negligible.  However, this open use of all available data is expensive both on data collection and computation resources as even the unnecessary data samples are computed through the ML model. One solution is to leave the challenge to some specific ML algorithm (e.g., LightGBM) to optimize the training data usage during model training, down-sampling the training data to use only part of the collected training data. However, in mobile networks where the amount of data is large, resources are constrained yet models need to be very accurate, these methods are inappropriate. Instead, it is better that the training function evaluates the effectiveness of different data samples and indicates that effectiveness to the consumer so that the training data used for (re)training may be optimized.

The 3GPP management system should have capability to report the effectiveness of the different training data instances. The effectiveness of training data can be per data instance or as statistics summary.

### 6.M.3	Requirements for ML training Data Management

Table 6.M.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-TRAIN_EFF-01: | The MLT MnS Producer should have the capability enabling an authorized consumer to configure an ML training function to report the effectiveness of data used for model training. | Reporting Training data effectiveness (clause 6.M.2.x) |
| REQ-TRAIN_EFF-02: | The MLT MnS Producer should have the capability enabling an authorized consumer to activate the ML training function /Entity to label the effectiveness of data used for training. | Training data effectiveness analytics (clause 6.M.2.x) |

End of Modification