| 3GPP TS 28.204 V0.4.0 (2023-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Network slice-specific authentication and authorization charging in the 5G System (5GS); Stage 2 (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	8

3.3	Abbreviations	8

4	Architecture considerations	9

4.1	High-level 5G System architecture	9

4.1.1	Non-roaming reference architecture	9

4.2	Network Slice-Specific Authentication and Authorization converged charging architecture	9

4.2.1	Non-roaming	9

5.1	Network Slice-Specific Authentication and Authorization charging principles	11

5.1.1	General	11

5.1.2	Requirements	11

5.1.3	Charging information	11

5.2	Network Slice-Specific Authentication and Authorization charging converged charging scenarios	12

5.2.1	Basic principles	12

5.2.1.1	General	12

5.2.1.2	Applicable Triggers	12

5.2.1.2.1	General	12

5.2.2	Message flows	14

5.2.2.1	General	14

5.2.2.2	Network slice-specific authentication and authorization charging	14

5.2.2.2.1	General	14

5.2.2.2.2	Network slice-specific authentication and authorization – AMF - PEC	14

5.2.2.2.3	Network slice-specific authentication and authorization – NSSAAF - PEC	15

5.2.2.2.4	Network slice-specific authentication and authorization – NSSAAF - IEC	15

5.2.2.2.5	Network slice-specific authentication and authorization – NSSAAF - ECUR	16

5.2.2.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization charging	18

5.2.2.3.1	General	18

5.2.2.3.1	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - AMF - PEC	18

5.2.2.3.2	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - PEC	18

5.2.2.3.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - IEC	19

5.2.2.3.4	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - ECUR	20

5.2.2.4	AAA Server triggered Network Slice-Specific Authorization Revocation charging	22

5.2.2.4.1	General	22

5.2.2.4.2	AAA Server triggered Network Slice-Specific Authorization Revocation - AMF - PEC	22

5.2.2.4.3	AAA Server triggered Network Slice-Specific Authorization Revocation- NSSAAF - PEC	22

5.2.2.4.4	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - IEC	23

5.2.2.4.5	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - ECUR	24

5.2.3	CDR generation	26

5.2.3.1	Introduction	26

5.2.3.2	Triggers for CHF CDR	26

5.2.3.2.1	General	26

5.2.3.2.2	Triggers for CHF CDR generation	26

5.2.3.2.3	Triggers for CHF CDR opening	26

5.2.3.2.4	Triggers for CHF CDR closure	26

5.2.4	Ga record transfer flows	26

5.2.5	Bnssaa CDR file transfer	26

6	Definition of charging information	26

6.1	Data description for Network slice-specific authentication and authorization charging	26

6.1.1	Message contents	26

6.1.1.1	General	26

6.1.1.2	Charging Data Request message	28

6.1.1.3	Charging data response message	28

6.2	Network slice-specific authentication and authorization charging specific parameters	29

6.2.1	Definition of Network slice-specific authentication and authorization charging information	29

6.2.1.1	General	29

6.2.1.2	Definition of Network slice-specific authentication and authorization charging information	29

Annex <A> (informative): Change history	30

# Foreword

This Technical Specification| has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the Converged Charging description for network slice-specific authentication and authorization charging in the 5G System (5GS) based on Network Slice-Specific Authentication and Authorization Function (NSSAAF) of 5GS architecture and procedures specified in 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4].

The scope is the Network Slice-Specific Authentication and Authorization as specified in TS 23.502 [4] with a AAA Server (AAA-S).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[3]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 32.256: "Telecommunication management; Charging management; 5G connection and mobility domain charging; stage 2".

[6]	3GPP TS 32.290: "Telecommunication management; Charging management; 5G system; Services, operations and procedures of charging using Service Based Interface (SBI)".

[7]	3GPP TS 32.291: "Telecommunication management; Charging management; 5G system; Charging service, stage 3".

[8]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[9]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[10]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Record (CDR) file format and transfer".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5G System: 3GPP system consisting of 5G Access Network (AN), 5G Core Network and UE.

Network Slice: A logical network that provides specific network capabilities and network characteristics.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Bnssaa	Reference point for the CDR file transfer from the NSSAAF CGF to the BD.

Ga	Reference point for CDR transfer between a CDF and the CGF.

Nchf	Service based interface exhibited by CHF.

N103	Reference point between NSSAAF and the CHF.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

3GPP	3rd Generation Partnership Project

5GS	5G System

AAA 	Authentication Authorization Accounting

AAA-S	AAA Server

AMF	Access and Mobility Management Function

CDR	Charging Data Record

CGF	Charging Gateway Function

CHF	CHarging Function

EAP	Extensible Authentication Protocol

ECUR	Event Charging with Unit Reservation

GPSI	Generic Public Subscription Identifier

IEC	Immediate Event Charging

NSSAA	Network Slice-Specific Authentication and Authorization

NSSAAF	Network Slice-Specific Authentication and Authorization Function

PDU	Protocol Data Unit

PEC	Post Event Charging

S-NSSAI	Single Network Slice Selection Assistance Information

SUPI	Subscription Permanent Identifier

UE	User Equipment

# 4	Architecture considerations

## 4.1	High-level 5G System architecture

### 4.1.1	Non-roaming reference architecture

Figure 4.1.1-1 shows the Non-roaming 5G System high level architecture in the service-based representation, as defined in 3GPP TS 23.501 [3], with Network Slice-Specific Authentication and Authorization (NSSAAF):

Figure 4.1.1-1: Non-Roaming 5G System architecture

## 4.2	Network Slice-Specific Authentication and Authorization converged charging architecture

### 4.2.1	Non-roaming

Architectural options for Network Slice-Specific Authentication and Authorization converged charging in service-based representation are depicted in figure 4.2.1-1.

Figure 4.2.1-1: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture

Architectural options of figure 4.2.1-1 apply to any Network Slice-Specific Authentication and Authorization converged charging architecture of this clause.

AMF is part of this architecture for the purpose of Network Slice-Specific Authentication and Authorization converged charging. For AMF 5G connection and mobility converged charging see TS 32.256 [5].

Details on the interfaces and functions can be found in 3GPP TS 32.240 [2] for the general architecture components, Ga is described in clause W and Bnssaa in clause Z of this document, and Nchf is described in 3GPP TS 32.290 [6].

Figure 4.2.1-2 shows the Network Slice-Specific Authentication and Authorization converged charging architecture in reference point representation for non-roaming:

Figure 4.2.1-2: Non-Roaming Network Slice-Specific Authentication and Authorization converged charging architecture - reference point representation

# 5	Network Slice-Specific Authentication and Authorization charging principles and scenarios

## 5.1	Network Slice-Specific Authentication and Authorization charging principles

### 5.1.1	General

The charging functions specified for Network Slice-Specific Authentication and Authorization charging, are based on Network Slice-Specific Authentication and Authorization with a AAA Server (AAA-S) functionality as specified in TS 23.501 [3], supported by:

-	NSSAAF,	and

-	optionaly AMF.

The network slice is identified by a S-NSSAI.

For AMF the present specification only covers Network Slice-Specific Authentication and Authorization charging. For AMF 5G connection and mobility charging see TS 32.256 [5].

### 5.1.2	Requirements

The following are high-level charging requirements specific to Network Slice-Specific Authentication and Authorization charging:

-	The NSSAAF shall support converged charging using service based interface.

-	The NSSAAF shall support converged charging for Network Slice-Specific Authentication and Authorization procedure per S-NSSAI per UE.

-	The AMF shall support converged charging for Network Slice-Specific Authentication and Authorization procedure per S-NSSAI per UE.

### 5.1.3	Charging information

The charging information for Network Slice-Specific Authentication and Authorization charging are:

-	GPSI;

-	SUPI;

-	S-NSSAI;

- 	AAA-S address.

## 5.2	Network Slice-Specific Authentication and Authorization charging converged charging scenarios

### 5.2.1	Basic principles

#### 5.2.1.1	General

Network Slice-Specific Authentication and Authorization converged charging, may be performed by the NSSAAF and AMF interacting with the Charging Function (CHF) using Nchf specified in 3GPP TS 32.290 [6] and 3GPP TS 32.291 [7]. In order to provide the data required for the management activities outlined in 3GPP TS 32.240 [2], the NSSAAF and AMF shall be able to perform converged charging for each of the following:

-	Charging information related to Network Slice-Specific Authentication and Authorization procedure per UE per S-NSSAI.

-	Charging information related to the "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" procedure per UE per S-NSSAI.

-	Charging information related to the "AAA Server triggered Network Slice-Specific Authorization Revocation" procedure per UE per S-NSSAI.

The NSSAAF and AMF shall be able to perform converged charging by interacting with the CHF, for charging data related to Network Slice-Specific Authentication and Authorization, AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization and AAA Server triggered Slice-Specific Authorization Revocation procedures.

The Charging Data Request and Charging Data Response are exchanged between the NSSAAF and the CHF based on IEC, PEC, or ECUR scenarios as specified in TS 32.290 [6]. The Charging Data Request and Charging Data Response are exchanged between the AMF and the CHF, based on PEC scenario. IEC, PEC and ECUR scenarios are specified in TS 32.290 [6].

The Charging Data Request is issued by the NSSAAF towards the CHF, and by the AMF towards the CHF, when certain conditions (chargeable events) are met.

The contents and purpose of each charging event that triggers interaction with CHF, as well as the chargeable events that trigger them, are described in the following clauses.

A detailed formal description of the converged charging parameters defined in the present document is to be found in 3GPP TS 32.291 [7].

A detailed formal description of the CDR parameters defined in the present document is to be found in 3GPP TS 32.298 [8].

#### 5.2.1.2	Applicable Triggers

##### 5.2.1.2.1	General

When a charging event is issued towards the CHF, it includes details such as Subscriber identifier (e.g. SUPI).

Each trigger condition (i.e. chargeable event) defined for the Network Slice-Specific Authentication and Authorization converged charging functionality, is specified with the associated behaviour when they are met.

Table 5.2.1.2.1-1 summarizes the set of default trigger conditions and their category which shall be supported by the NSSAAF when charging is active for the corresponding Network Slice-Specific Authentication and Authorization functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from NSSAAF towards the CHF.

Table 5.2.1.2.1-1: Default Trigger conditions in NSSAAF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Network Slice-Specific Authentication and Authorization request | - | Immediate | Not Applicable | Yes | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| Network Slice-Specific Authentication and Authorization completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| AAA-S Network Slice-Specific Re-auth request | - | Immediate | Not Applicable | Yes | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| AAA-S Network Slice-Specific Re-auth completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |
| AAA-S Network Slice-Specific Revocation request | - | Immediate | Not Applicable | Yes | IEC: Charging Data Request [Event] ECUR: Charging Data Request [Initial] |
| AAA-S Network Slice-Specific Revocation completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] ECUR: Charging Data Request [Termination] |

Table 5.2.1.2.1-2 summarizes the set of default trigger conditions and their category which shall be supported by the AMF when charging is active for the corresponding Network Slice-Specific Authentication and Authorization functionality. For "immediate report" category, the table also provides the corresponding Charging Data Request message sent from AMF towards the CHF.

Table 5.2.1.2.1-2: Extended Default Trigger conditions in AMF

| Trigger Conditions | Trigger level | Default category | CHF allowed to change category | CHF allowed to enable and disable | Message when "immediate reporting" category |
| --- | --- | --- | --- | --- | --- |
| Network Slice-Specific Authentication and Authorization completed | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] |
| AAA-S Network Slice-Specific Re-auth Notification | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] |
| AAA-S Network Slice-Specific Revocation Notification | - | Immediate | Not Applicable | Yes | PEC: Charging Data Request [Event] |

NOTE: If the same triggers are enabled in the NSSAAF and the AMF, the respective CHF CDRs will be associated to the same procedure.

### 5.2.2	Message flows

#### 5.2.2.1	General

The flows in the present document specify the interaction between the NSSAAF and the CHF and between the AMF and the CHF, for Network slice-specific authentication and authorization converged charging functionality, in different scenarios, based on 3GPP TS 23.501 [3] and 3GPP TS 23.502 [4] procedures and flows.

This interaction is based on Charging Data Request /Response specified in 3GPP TS 32.290 [6], exchanged between the NSSAAF embedding the CTF and the CHF, and between the AMF embedding the CTF and the CHF.

If both NSSAAF and AMF generate CHF CDRs, for the same procedure, they will be the same.

The following scenarios are supported by NSSAAF:

- 	PEC;

-	IEC;

-	ECUR.

The PEC scenario is supported by AMF.

As a general principle, the steps in the figures for the message flows below correspond to the steps of figures in 3GPP TS 23.502 [4], which is the reference. The present document specifies the charging specific extension part.

#### 5.2.2.2	Network slice-specific authentication and authorization charging

##### 5.2.2.2.1	General

The subclauses below describe Network slice-specific authentication and authorization charging message flows based on figure 4.2.9.2-1 of 3GPP TS 23.502 [4].

##### 5.2.2.2.2	Network slice-specific authentication and authorization – AMF - PEC

The following figure 5.2.2.2.2-1 describes a Network slice-specific authentication and authorization charging in PEC scenario for AMF:

Figure 5.2.2.2.2-1: Network slice-specific authentication and authorization – AMF - PEC

Steps 1 to 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

18ch-a: NSSAA procedure in AMF completed:  AMF sends Charging Data Request [Event] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

18ch-b: The CHF creates a CDR.

18ch-c: CHF provides response to AMF.

##### 5.2.2.2.3	Network slice-specific authentication and authorization – NSSAAF - PEC

The following figure 5.2.2.2.3-1 describes a Network slice-specific authentication and authorization charging in PEC scenario for NSSAAF:

Figure 5.2.2.2.3-1: Network slice-specific authentication and authorization - PEC

Steps 1 to 17: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

17ch-a: NSSAAF procedure with AAA-S for NSSAA is completed:  NSSAAF sends Charging Data Request [Event] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

17ch-b: The CHF creates a CDR.

17ch-c: CHF provides response to NSSAAF.

Step 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

##### 5.2.2.2.4	Network slice-specific authentication and authorization – NSSAAF - IEC

The following figure 5.2.2.2.4-1 describes a Network slice-specific authentication and authorization charging in IEC scenario for NSSAAF :

Figure 5.2.2.2.4-1: Network slice-specific authentication and authorization – NSSAAF - IEC

Steps 1 to 4 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure, AMF sends the EAP Identity Response to the NSSAAF.

4ch-a: NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

4ch-b: Account, Rating, control  by the CHF.

4ch-c: The CHF creates a CDR.

4ch-d: CHF provides response to NSSAAF.

Steps 5 to 18 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

##### 5.2.2.2.5	Network slice-specific authentication and authorization – NSSAAF - ECUR

The following figure 5.2.2.2.5-1 describes a Network slice-specific authentication and authorization charging in ECUR scenario for NSSAAF:

Figure 5.2.2.2.5-1: Network slice-specific authentication and authorization – NSSAAF - ECUR

Steps 1 to 4 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure, AMF sends the EAP Identity Response to the NSSAAF.

4ch-a: NSSAAF sends Charging Data Request [Initial] to CHF with GPSI and S-NSSAI.

4ch-b: Account, Rating, control  by the CHF be granted authorization for NSSAA.

4ch-c: The CHF opens a CDR.

4ch-d: CHF provides response to NSSAAF.

Steps 5 to 17 per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

17ch-a: NSSAAF procedure with AAA-S is completed:  NSSAAF sends Charging Data Request [Terminationt] to CHF with EAP-Success/Failure, GPSI and S-NSSAI.

17ch-b: Account, Rating, control by the CHF for NSSAA.

17ch-c: The CHF closes the CDR.

17ch-d: CHF provides response to NSSAAF.

Step 18: per 3GPP TS 23.502 [4] Figure 4.2.9.2-1 Network Slice-Specific Authentication and Authorization procedure.

#### 5.2.2.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization charging

##### 5.2.2.3.1	General

The subclauses below describe "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization"  charging message flows based on figure 4.2.9.3-1 of 3GPP TS 23.502 [4].

##### 5.2.2.3.1	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - AMF - PEC

The following figure 5.2.2.3.1-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in PEC scenario for AMF:

Figure 5.2.2.3.1-1: AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure - AMF - PEC

Steps 1 to 4: per 3GPP TS 23.502 [4] Figure 4.2.9.3-1 AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure.

4ch-a: AAA-S Network Slice-Specific Re-authentication and Re-authorization request received in AMF :  AMF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

4ch-b: The CHF creates a CDR.

4ch-c: CHF provides response to AMF.

##### 5.2.2.3.2	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - PEC

The following figure 5.2.2.3.2-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in PEC scenario for NSSAAF:

Figure 5.2.2.3.2-1: AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure - NSSAAF - PEC

Steps 1 to 3c: per 3GPP TS 23.502 [4] Figure 4.2.9.3-1 AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure.

3cch-a: AAA-S Network Slice-Specific Re-authentication and Re-authorization completed in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

3cch-b: The CHF creates a CDR.

3cch-c: CHF provides response to NSSAAF.

##### 5.2.2.3.3	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - IEC

The following figure 5.2.2.3.3-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in IEC scenario for NSSAAF:

Figure 5.2.2.3.3-1: AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure - NSSAAF - IEC

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.3-1 AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure.

2ch-a: AAA-S Network Slice-Specific Re-authentication and Re-authorization request received in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF creates a CDR.

2ch-d: CHF provides response to NSSAAF.

Steps 3 to 5: per 3GPP TS 23.502 [4] Figure 4.2.9.3-1 AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure.

##### 5.2.2.3.4	AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization - NSSAAF - ECUR

The following figure 5.2.2.3.4-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in ECUR scenario for NSSAAF:

Figure 5.2.2.3.4-1: AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure - NSSAAF - ECUR

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.3-1 AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure.

2ch-a: AAA-S Network Slice-Specific Re-authentication and Re-authorization request received in NSSAAF :  NSSAAF sends Charging Data Request [Initial] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF opens a CDR.

2ch-d: CHF provides response to NSSAAF.

3cch-a: AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure is completed:  NSSAAF sends Charging Data Request [Termination] to CHF with GPSI and S-NSSAI.

3cch-b: Account, Rating, control by the CHF.

3cch-c: The CHF closes the CDR.

3cch-d: CHF provides response to NSSAAF.

#### 5.2.2.4	AAA Server triggered Network Slice-Specific Authorization Revocation charging

##### 5.2.2.4.1	General

The subclauses below describe "AAA Server triggered Network Slice-Specific Authorization Revocation" charging message flows based on figure 4.2.9.4-1 of 3GPP TS 23.502 [4].

##### 5.2.2.4.2	AAA Server triggered Network Slice-Specific Authorization Revocation - AMF - PEC

The following figure 5.2.2.4.2-1 describes a "AAA Server triggered Network Slice-Specific Authorization Revocation" charging in PEC scenario for AMF:

Figure 5.2.2.4.2-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - AMF - PEC

Steps 1 to 4: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

4ch-a: AAA-S Network Slice-Specific Authentication revocation Notification received in AMF :  AMF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

4ch-b: The CHF creates a CDR.

4ch-c: CHF provides response to AMF.

##### 5.2.2.4.3	AAA Server triggered Network Slice-Specific Authorization Revocation- NSSAAF - PEC

The following figure 5.2.2.4.3-1 describes a "AAA Server triggered Network Slice-Specific Authentication revocation" charging in PEC scenario for NSSAAF:

Figure 5.2.2.4.3-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - PEC

Steps 1 to 3c: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

3cch-a: AAA-S Network Slice-Specific Authorization Revocation completed in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

3cch-b: The CHF creates a CDR.

3cch-c: CHF provides response to NSSAAF.

##### 5.2.2.4.4	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - IEC

The following figure 5.2.2.4.4-1 describes a "AAA Server triggered Network Slice-Specific Authorization Revocation" charging in IEC scenario for NSSAAF:

Figure 5.2.2.4.4-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - IEC

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

2ch-a: AAA-S Network Slice-Specific Authorization Revocation request received in NSSAAF :  NSSAAF sends Charging Data Request [Event] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF creates a CDR.

2ch-d: CHF provides response to NSSAAF.

Steps 3 to 5: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

##### 5.2.2.4.5	AAA Server triggered Network Slice-Specific Authorization Revocation - NSSAAF - ECUR

The following figure 5.2.2.4.5-1 describes a "AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization" charging in ECUR scenario for NSSAAF:

Figure 5.2.2.4.5-1: AAA Server triggered Network Slice-Specific Authorization Revocation procedure - NSSAAF - ECUR

Steps 1 to 2: per 3GPP TS 23.502 [4] Figure 4.2.9.4-1 AAA Server triggered Network Slice-Specific Authorization Revocation procedure.

2ch-a: AAA-S Network Slice-Specific Authorization Revocation request received in NSSAAF :  NSSAAF sends Charging Data Request [Initial] to CHF with GPSI and S-NSSAI.

2ch-b: Account, Rating, control  by the CHF.

2ch-c: The CHF opens a CDR.

2ch-d: CHF provides response to NSSAAF.

3cch-a: AAA Server triggered Network Slice-Specific Authorization Revocation procedure is completed:  NSSAAF sends Charging Data Request [Termination] to CHF with GPSI and S-NSSAI.

3cch-b: Account, Rating, control by the CHF.

3cch-c: The CHF closes the CDR.

3cch-d: CHF provides response to NSSAAF.

### 5.2.3	CDR generation

#### 5.2.3.1	Introduction

The CHF CDRs for Network Slice-Specific Authentication and Authorization charging are generated by the CHF to collect charging information that they subsequently transfer to the Charging Gateway Function (CGF).

The following clauses describe in detail the conditions for generating the CHF CDR, which shall be supported by the CHF.

#### 5.2.3.2	Triggers for CHF CDR

##### 5.2.3.2.1	General

A Network Slice-Specific Authentication and Authorization charging CHF CDR is used to collect charging information related to Network Slice-Specific Authentication and Authorization chargeable events for PEC, IEC and ECUR scenarios.

##### 5.2.3.2.2	Triggers for CHF CDR generation

A CHF CDR shall be generated by the CHF for each received Charging Data Request [Event].

##### 5.2.3.2.3	Triggers for CHF CDR opening

A CHF CDR shall be opened when the CHF receives Charging Data Request [Initial].

##### 5.2.3.2.4	Triggers for CHF CDR closure

The CHF CDR shall be closed when the CHF receives Charging Data Request [Termination].

### 5.2.4	Ga record transfer flows

Details of the Ga protocol application are specified in 3GPP TS 32.295 [9].

### 5.2.5	Bnssaa CDR file transfer

Details of the Bnssaa protocol application are specified in 3GPP TS 32.297 [10].

# 6	Definition of charging information

## 6.1	Data description for Network slice-specific authentication and authorization charging

### 6.1.1	Message contents

#### 6.1.1.1	General

The Charging Data Request and Charging Data Response are specified in subclause 5.1.2.2.1 of 3GPP TS 32.290 [6].

Table 6.1.1.1-1 describes the use of these messages for Network slice-specific authentication and authorization charging.

Table 6.1.1.1-1: Network slice-specific authentication and authorization charging messages reference table

| Message | Source | Destination |
| --- | --- | --- |
| Charging Data Request | NSSAAF, AMF | CHF |
| Charging Data Response | CHF | NSSAAF, AMF |

The following clauses describe the different fields used in the Charging Data messages and the category in the tables is used according to the charging data configuration defined in clause 5.4 of 3GPP TS 32.240 [2].

#### 6.1.1.2	Charging Data Request message

Table 6.1.1.2-1 illustrates the basic structure of a Charging Data Request message from the NSSAAF and AMF, as used for network slice-specific authentication and authorization.

Table 6.1.1.2-1: Charging Data Request message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6] |
| Subscriber Identifier | OC | This field contains the identification of the individual subscriber in the PLMN i.e. SUPI. |
| NF Consumer Identification | M | Described in 3GPP TS 32.290 [6] and holds the identifier of the NSACF |
| NF Functionality | M | Described in 3GPP TS 32.290 [6]. |
| NF Name | OC | Described in 3GPP TS 32.290 [6]. |
| NF Address | OC | Described in 3GPP TS 32.290 [6]. |
| NF PLMN ID | OC | Described in 3GPP TS 32.290 [6]. |
| Charging Identifier | OM | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | M | Described in 3GPP TS 32.290 [6]. |
| Retransmission Indicator | - | This field is not applicable. |
| One-time Event | OC | Described in 3GPP TS 32.290 [6]. |
| One-time Event Type | OC | Described in 3GPP TS 32.290 [6]. |
| Notify URI | - | This field is not applicable. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Service Specification Information | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | OC | Described in 3GPP TS 32.290 [6] and holds the Network slice-specific authentication and authorization specific triggers described in clause 5.2.1. |
| Multiple Unit Usage | OM | Described in 3GPP TS 32.290 [6]. |
| Rating Group | M | Described in 3GPP TS 32.290 [6]. |
| Requested Unit | OC | Described in 3GPP TS 32.290 [6]. |
| Used Unit Container | - | This field is not applicable. |
| User identification | M | This field holds the user identification of the individual subscriber, i.e. Generic Public Subscription Identifier (GPSI). |
| S NSSAI | M | This field holds the Single Network Slice Selection Assistance Information identifying the network slice. |
| NSSAA Charging Information | C | This field holds NSSAA specific information described in clause 6.x.1.2 |

#### 6.1.1.3	Charging data response message

Table 6.1.1.3-1 illustrates the basic structure of a Charging Data Response message from the CHF to the NSSAAF and AMF as used for network slice-specific authentication and authorization.

Table 6.1.1.3-1: Charging Data Response message contents

| Information Element | Converged Charging Category | Description |
| --- | --- | --- |
| Session Identifier | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Timestamp | M | Described in 3GPP TS 32.290 [6]. |
| Invocation Result | OC | Described in 3GPP TS 32.290 [6]. |
| Invocation Sequence Number | OM | Described in 3GPP TS 32.290 [6]. |
| Session Failover | OC | Described in 3GPP TS 32.290 [6]. |
| Supported Features | OC | Described in 3GPP TS 32.290 [6]. |
| Triggers | - | This field is not applicable. |
| Multiple Unit Information | OC | Described in 3GPP TS 32.290 [6]. |
| Result Code | OC | Described in 3GPP TS 32.290 [6]. |
| Rating Group | OM | Described in 3GPP TS 32.290 [6]. |
| Granted Unit | - | This field is not applicable. |
| Validity Time | OC | Described in 3GPP TS 32.290 [6]. |
| Final Unit Indication | - | This field is not applicable. |
| Time Quota Threshold | - | This field is not applicable. |
| Volume Quota Threshold | - | This field is not applicable. |
| Unit Quota Threshold | - | This field is not applicable. |
| Quota Holding Time | - | This field is not applicable. |
| Triggers | - | This field is not applicable. |

## 6.2	Network slice-specific authentication and authorization charging specific parameters

### 6.2.1	Definition of Network slice-specific authentication and authorization charging information

#### 6.2.1.1	General

The Charging Information parameter used for Network slice-specific authentication and authorization charging is provided in the following clauses.

#### 6.2.1.2	Definition of Network slice-specific authentication and authorization charging information

Specific charging information used for Network slice-specific authentication and authorization charging is provided within the NSSAA Charging Information.

The detailed structure of the NSSAA Charging Information can be found in table 6.2.1.2-1.

Table 6.2.1.2-1: Structure of NSSAA Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| NSSAA message type | M | This field holds the message type of the NSSAA procedure |
| AAA P Address | OC | This field holds the AAA-P Address when available. |
| AAA S Address | OC | This field holds the AAA-S Address when available. |
| EAP ID Response | OC | This field holds the EAP ID Response message from the UE. |
| EAP auth status | OC | This field holds the result of EAP authentication procedure. |
| AMF Identifier | OC | This field holds the AMF identifier of the AMF serving the UE |

Annex <A> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-04 | SA5#148e |  |  |  |  | Initial skeleton | 0.0.0 |
| 2023-04 | SA5#148e | S5-233220 S5-233222 S5-233223 S5-233656 |  |  |  | Introduction of the Reference Introduction of the Terms Introduction of the Abbreviations Introduction of the Scope | 0.1.0 |
| 2023-05 | SA5#149 | S5-234473 S5-234474 |  |  |  | Introduce architecture Introduce charging principles | 0.2.0 |
| 2023-06 | SA5#150 | S5-235766 S5-235767 S5-235768 S5-235783 S5-235784 |  |  |  | Introduce charging scenarios principles Introduce applicable triggers Introduce PEC message flow Introduce IEC message flow Introduce ECUR message flow | 0.3.0 |
| 2023-10 | SA5#151 | S5-236289 S5-236291 S5-236292 S5-236293 S5-236904 S5-236295 S5-236905 S5-236297 |  |  |  | Introduce Reference Point for NSSAAF Addition of applicable triggers Introduction of AAA-S Re-authentication and Re-authorization flows Introduction of AAA-S triggered NS-Specific Autho. Revocation flows Introduce CDR generation Introduce Ga record and CDR file transfer Introduce definition of charging information Introduce NSSAA specific charging information | 0.4.0 |