3GPP TSG-SA5 Meeting #151	S5-236959

Xiamen, China, 9 - 13 October 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.552 | CR | 0474 | rev | 1 | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update measurements related to coordination among multiple NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | MANWDAF |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Measurements related to the coordination between multiple NWDAFs may be applicable for both analytics and training service operations. Currently, the specified measurements related to the coordination between multiple NWDAFs does not differentiate between analytics service subscriptions/notifications and training service subscriptions/notifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the measurements related to the coordination between multiple NWDAFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The confusion between analytics service subscriptions/notifications and training service subscriptions/notifications persists for the measurements related to the coordination between multiple NWDAFs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.18.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | S5-236959 is the revision of S5-236658 |  |  |  |  |  |  |  |  |

Start of First change

### 5.18.2	Measurements related to the coordination between multiple NWDAFs

#### 5.18.2.X	Number of analytics service subscription from Aggregator NWDAF

a)	This measurement provides the number of analytics service subscriptions generated by the Aggregator NWDAF.

b)	CC

c)	On sending the analytics service subscription by the Aggregator NWDAF to other NWDAFs to subscribe analytics information for analytics aggregation (See TS 23.288 [2]), each subscription is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.AnalyticsSerSubGenerated, and

DANS.AnalyticsSerSubGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.AnalyticsSerSubGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.2.X	Number of analytics service requests from Aggregator NWDAF

a)	This measurement provides the number of analytics service requests generated by the Aggregator NWDAF.

b)	CC

c)	On sending the analytics service request by the Aggregator NWDAF to other NWDAFs to request analytics information for analytics aggregation (See TS 23.288 [2]), each request is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.AnalyticsSerReqGenerated, and

DANS.AnalyticsSerReqGenerated.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.AnalyticsSerReqGenerated.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.2.X	Number of analytics service notifications received by the Aggregator NWDAF

a)	This measurement provides the number of analytics service notifications received by the Aggregator NWDAF.

b)	CC

c)	On receipt by the Aggregator NWDAF of the notification corresponding to the analytics service subscription from Aggregator NWDAF (See TS 23.288 [2]), each transmitted notification is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [X]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.AnalyticsSerNotificationReceived, and

DANS.AnalyticsSerNotificationReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.AnalyticsSerNotificationReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

#### 5.18.2.X	Number of analytics service responses received by the Aggregator NWDAF

a)	This measurement provides the number of analytics service responses received by the Aggregator NWDAF.

b)	CC

c)	On receipt by the Aggregator NWDAFs of the response corresponding to the analytics service request from Aggregator NWDAF (See TS 23.288 [2]), each received response is added to the corresponding counter. The measurement can be split into subcounters per Analytics ID (see clause 5.1.6.2.42 and 5.1.6.3.4 in TS 29.520 [58]) and subcounters per S-NSSAI.

d)	An integer value

e)	The measurement name has the form DANS.AnalyticsSerResponseReceived, and

DANS.AnalyticsSerResponseReceived.AnalyticsID, where AnalyticsID identifies different analytics, and

DANS.AnalyticsSerResponseReceived.SNSSAI, where SNSSAI identifies the S-NSSAI.

f)	NWDAFFunction

g)	Valid for packet switched traffic

h)	5GS

End of Changes