3GPP TSG-SA5 Meeting #151 	S5-236973

Xiamen, China, 9 - 13 October 2023

# Source:	Huawei

# Title:	Update charging principles

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the updated charging principles of TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

This contribution proposes updated charging principles of TS 32.282.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

5.1	Time sensitive networking charging principles

5.1.1	General

The charging functions specified for the TSN charging, are based on functionalities supported by TSN specified in TS 23.501 [11]:

-	5GS as TSN bridge, as specified in clause 5.28 in TS 23.501 [11].

-	Enablers of TSC and TS service, as specified in clause 5.27 in TS 23.501 [11].

The charging functions specified for the TSN charging are:

-	TSN 5GS bridge configuration and management charging.

-	TSC and TS service enabler charging.

-	TSC traffic charging.

5.1.2	Requirements

The following are high-level charging requirements specific to TSN domain, derived from requirements in TS 22.261 [10], TS 22.104 [9], TS 23.501 [11], TS 23.502 [12] and TS 23.503 [13].

-	The TSN AF shall support converged charging.

-	The TSN AF shall collect and report charging information for TSN 5GS bridge configuration and management.

-	The TSCTSF shall support converged charging.

-	The TSCTSF shall collect and report charging information for TSC and TS service enabler.

The requirements for SMF and NEF to support TSN charging are specified in the respective specifications, i.e. TS 32.255 [3] and TS 32.254 [2].

5.1.3	TSN charging information

Editor’s note: the charging information is FFS.

5.1.4	CHF selection

The CHF selection by the TSN AF may be based on the following and with this priority order (highest to lowest):

-	NRF based discovery.

-	TSN AF locally provisioned charging characteristics.

The CHF selection by the TSCTSF may be based on the following and with this priority order (highest to lowest):

-	NRF based discovery.

-	TSCTSF locally provisioned charging characteristics.

The priority order for CHF selection may be changed by operator.

End of change