3GPP TSG-SA5 Meeting #152 	S5-237690

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | TS 28.104 | CR | Draft CR | rev | <Rev#> | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 InputToDraftCR TS 28.104 Corrections for resource related analysis |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph2 |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In clause 7.2.x.1.2, the description of recommended actions for physical resource optimization is missing. In clause 8.4.x.1.2.3, it is unclear and inaccurate for the definition of the attribute of “recommendedActions”, since “scaling out” is always used to describe virtual resource optimization actions. In clause 8.4.x.1.1.3, the Support qualifier of highVRUsageNFs are missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add description of recommended actions for physical resource optimization in clause 7.2.x.1.2. - Correct the definition of the attribute of “recommendedActions” in clause 8.4.x.1.2.3. - Add Support qualifier of highVRUsageNFs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is unclear how to perform the recommendedActions for physical resource utilization analysis. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.x.1.2, 8.4.x.1.1.3, 8.4.x.1.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st modified section

### 7.2.x	Resource related analytics

#### 7.2.x.1	NF resource utilization analysis

##### 7.2.x.1.1	Description

This MDA capability is for analysis of resource utilization of 3GPP NFs.

##### 7.2.x.1.2	Use case

The 3GPP system is a resource limited system, no matter whether the NF is working on virtualized resources or physical resources.

Resource shortage would affect the QoS and potentially impact users’ quality of experience (QoE), e.g., by lowering the users’ data throughput, prolonging the users’ data delay, raising the rejections for the establishment of new connections (e.g., RRC connection), sessions (e.g., PDU session) and resources (e.g., QoS flows, DRBs, etc.) and increasing the drops of the existing connections, sessions, and resources. This may also consequently lead to risking or failing SLAs.

On the other hand, resource excess would cause wastage that leads to additional CapEx and OpEx.

Therefore, it is imperative to ensure optimum and efficient resource utilization for the NFs.

The resource utilization of an NF is heavily dependent on load or traffic patterns, which could vary in different coverage areas (e.g., business area, entertainment area, and residential area) and in different time periods (weekdays and time of the day). It is desirable that the spare resource of the low-usage areas can be allocated to the busy areas.It is expected that MDA can perform an analysis of the resource utilization for physical resources or virtualized resources for the 3GPP NFs (in a specific domain or cross domains) to indicate the resource usage patterns in the past and predict the resource usage trend for some time periods in the future. The physical resources to be analyzed may include hardware resources (e.g., CPU), DL and UL PRBs (for gNB), etc., while the virtualized resources to be analyzed may include virtual CPU, virtual memory, virtual disk, etc.It is also very useful that MDA correlates the resource analytics across 3GPP NFs and provides recommendations that can be utlized to efficiently orchestrate the resources among NFs between the low usage and high usage areas for some time periods. The recommended actions could be for example to increase the capacity of gNB to optimize allocation of the physical resources or to schedule the "scale in" and "scale out" of VNFs via ETSI MANO system to optimize the allocation of the virtualized resources.

##### 7.2.x.1.3	Requirements

Table 7.2.x.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-RES_UTI_ANA-01 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with low usage of physical resources (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-02 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with high usage of physical resources (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-03 | MDA capability for resource utilization analysis shall include providing the prediction of physical resource usage for a 3GPP NF (see Note 1). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-04 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with low usage of virtualized resources (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-05 | MDA capability for resource utilization analysis shall include identifying the 3GPP NFs with high usage of virtualized resources (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-06 | MDA capability for resource utilization analysis shall include providing the prediction of virtualized resource usage for a 3GPP NF (see Note 2). | NF resource utilization analysis |
| REQ-RES_UTI_ANA-07 | MDA capability for resource utilization analysis shall include providing recommended actions to manage and orchestrate one or more 3GPP NFs. e.g. to orchestrate the resource allocation or load balancing for one or multiple 3GPP NFs. | NF resource utilization analysis |
| NOTE 1: The requirement is valid only if the subject 3GPP NF uses physical resources. NOTE 2: The requirement is valid only if the subject 3GPP NF uses virtualized resources. |  |  |

Next modified section

### 8.4.x	Resource related analytics

#### 8.4.x.1	NF resource utilization analysis

##### 8.4.x.1.1	Virtualized resource utilization analysis

###### 8.4.x.1.1.1	MDA type

The MDA type for virtualized resource utilization analysis is: ResourceAnalytics.virtualizedResourceUtilizationAnalysisNF.

###### 8.4.x.1.1.2	Enabling data

The enabling data for virtualized resource utilization analysis are provided in table 8.4.x.1.1.2-1.

For general information about enabling data, see clause 8.2.1.

Note: 	The MDA output of ETSI NFV MANO should be reused as much as possible for virtualized resource analysis for 3GPP NFs, by mapping from the ETSI MANO VNF/VNFCs to 3GPP NFs.

Table 8.4.x.1.1.2-1: Enabling data for virtualized resource utilization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | VR (including Virtual CPU, Virtual Memory, and Virtual Disk) usage of NF | VR usage of NF (clause 5.7.1 of TS 28.552 [4]) |
|  | Connection Point data volumes of NF | Connection data volumes of NF (clause 5.7.2 of TS 28.552 [4]) |
|  | N3 interface data volume | N3 interface related measurements (clause 5.4.1 of TS 28.552 [4]) |
|  | N4 interface session establishments | N4 session establishments (clause 5.4.3.1 of TS 28.552 [4]) |
|  | N6 interface link usage | N6 related measurements (clause 5.4.2 of TS 28.552 [4]) |
|  | N9 interface data volume | GTP Data Packets and volume on N9 interface (clause 5.4.4.2 of TS 28.552 [4]) |
|  | Number of PDU sessions | Number of PDU sessions (mean) (clause 5.3.1.1 of TS 28.552 [4]) |
|  | Number of QoS flows | Mean number of QoS flows (clause 5.3.2.1.7 of TS 28.552 [4]) |
| Configuration data | The NRMs of the analyzed NFs | The NRMs defined in TS 28.622 [19] and TS 28.541 [15]. |
| Network Data Analytics | Analysis data from the control plane produced by NWDAF including NF load, observed/predicted service experience, user plane performance, and slice load level analytics. | Analytics data from NWDAF in TS 23.288 [10] including e.g. NF load analytics (clause 6.5), observed/ predicted service experience related network data analytics (clause 6.4), analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14), and Slice load level related network data analytics (clause 6.3). |

###### 8.4.x.1.1.3	Analytics output

The specific information elements of the analytics output for virtualized resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.1.3-1.

Table 8.4.x.1.1.3-1: Analytics output for virtualized resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowVRUsageNFs | The NFs with low virtualized resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highVRUsageNFs | The NFs with high virtualized resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedVRUsageForNFs | The predicted virtualized resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may be (but not limited to): - scale in a list of NFs; - scale out a list of NFs. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: it is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

##### 8.4.x.1.2	Physical resource utilization analysis

###### 8.4.x.1.2.1	MDA type

The MDA type for physcial resource utilization analysis is: ResourceAnalytics.PhyiscalResourceUtilizationAnalysisNF.

###### 8.4.x.1.2.2	Enabling data

The enabling data for physical resource utilization analysis are provided in table 8.4.x.1.2.2-1.

For general information about enabling data, see clause 8.2.1.

Table 8.4.x.1.2.2-1: Enabling data for physical resource utilization analysis

| Data category | Description | References |
| --- | --- | --- |
| Performance measurements | Radio resource utilization | Radio resource utilization (clause 5.1.1.2 of TS 28.552 [4]) |
|  | RRC connection number | RRC connection number (clause 5.1.1.4 of TS 28.552 [4]) |
|  | Mean number of PDU sessions in NR cell | Mean number of PDU sessions being allocated (clause 5.1.1.5.4 of TS 28.552 [4]) |
|  | Mean number of DRBs in NR cell | Mean number of DRBs being allocated ( clause 5.1.1.10.9 of TS 28.552 [4]) |
|  | QoS flow release in NR cell | QoS flow release (clause 5.1.1.13.1 of TS 28.552 [4]) |
|  | Number of Active UEs | Number of Active UEs (clause 5.1.1.23 of TS 28.552 [4]) |
|  | PDCP Data Volume | PDCP Data Volume (clause 5.1.2.1 and 5.1.3.6 of TS 28.552 [4]) |
| Geographical data | The geographical information (longitude, latitude, altitude) of the deployed RAN (NG-RAN and E-UTRAN). | The geographical information (longitude, latitude, altitude) information (see the peeParametersList attribute of the ManagedFunction IOC in TS 28.622 [19]). |
| Configuration data | The NRMs of the analyzed gNB-CUs, and gNB-DUs | The GNBCUCPFunction, GNBCUUPFunction and GNBDUFunction defined in TS 28.622 [19] and TS 28.541 [15]. |
| Network Data Analytics | Analysis data from the control plane produced by NWDAF including NF load, observed/predicted service experience, user plane performance, and slice load level analytics. | Analytics data from NWDAF in TS 23.288 [10] including e.g. NF load analytics (clause 6.5), observed/predicted service experience related network data analytics (clause 6.4), analytics for user plane performance (i.e. average/maximum traffic rate, average/maximum packet delay, average packet loss rate in clause 6.14), and Slice load level related network data analytics (clause 6.3). |

###### 8.4.x.1.2.3	Analytics output

The specific information elements of the analytics output for physical resource utilization analysis, in addition to the common information elements of the analytics outputs (see clause 8.3), are provided in table 8.4.x.1.2.3-1.

Table 8.4.x.1.2.3-1: Analytics output for physical resource utilization analysis

| Information element | Definition | Support qualifier | Properties |
| --- | --- | --- | --- |
| lowPRUsageNFs | The NFs with low physical resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| highPRUsageNFs | The NFs with high physical resource usage (see Note 1) during some time periods in the past. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| predictedPRUsageForNFs | The predicted physical resource usage for NFs during some time periods in the future. | M | type: ResourceUsageNF multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| recommendedActions | The recommended actions to orchestrate the resource allocation for NFs. The recommended action may include (but not limited to) increasing the capacity of gNB. | M | type: RecommendedAction multiplicity: * isOrdered: False isUnique: True defaultValue: None isNullable: False |
| NOTE 1: it is up to the MDA MnS producer to decide the thresholds for low and high usage. |  |  |  |

End of modified sections