# 3GPP TSG-SA5 Meeting #152	S5-237903

# Chicago, US,  13-17, November 2023

# Source:	Nokia

# Title:	Rel-18 pCR TS 28.318 Cell Availability performance measurement definition

# Document for:	Approval

Agenda Item:	6.6.4.3

# 1	Decision/action requested

The group is asked to discuss and approve the proposals.

# 2	References

[Y]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[Z]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

# 3	Rationale

This pCR defines the cell availability performance measurement based on the values of the operational state and administrative state attributes of the cells.

# 4	Detailed proposal

It is proposed to agree to the following text to clause 7.1.1 in TS 28.318.

Begin Change

#### 7.1.1.X Cell Availability

##### 7.1.1.X.1 Cell AvailabilityNRCellDU

a) This measurement provides the amount of time that the NRCellDU part of the cell is available to provide services to UEs within a given measurement window (Twin).  It is computed by the NRCellDU as the arithmetic sum of the amount of time that the NRCellDU is unlocked and enabled to serve UEs in each measurement window Twin.

b) cc

c) This measurement is obtained as: CellAvailNRCellDU = T(OSTNRCellDU=Enabled)+ T(ASTNRCellDU=Unlocked)Twin*100, where CellAvailNRCellDU is the availability of the NRCellDU, Twin is the measurement window, T(OSTNRCellDU=Enabled) is the total amount of time that the operational state attribute(OST) of the NRCellDU is enabled in a given Twin, and T(ASTNRCellDU=Unlocked) is the total amount of time that the administrative state attribute(AST) of the NRCellDU is unlocked in a given Twin.

d) Two values: The CellAvailNRCellDU from 0 to 100 and the corresponding Twin.

e) CellAvailNRCellDU

f) NRCellDU

g) Valid for packet switched traffic

h) 5GS

##### 7.1.1.X.2 Cell AvailabilityNRCellCU

a) This measurement provides the amount of time that the NRCellCU part of the cell is available and providing services within a given measurement window (Twin).  It is computed by the NRCellCU as the arithmetic sum of the amount of time that the NRCellCU is unlocked and enabled in each measurement window Twin.

b) cc

c) This measurement is obtained as: CellAvailNRCellCU = T(OSTNRCellCU=Enabled)+ T(ASTNRCellCU=Unlocked)Twin*100, where CellAvailNRCellCU is the availability of the NRCellCU, Twin is the measurement window, T(OSTNRCellCU=Enabled) is the total amount of time that the operational state attribute(OST) of the NRCellCU is enabled in a given Twin, and T(ASTNRCellCU=Unlocked) is the total amount of time that the administrative state attribute(AST) of the NRCellCU is unlocked in a given Twin.

d) Two values: The CellAvailNRCellDU value from 0 to 100 and the corresponding Twin.

e) CellAvailNRCellCU

f) NRCellCU

g) Valid for packet switched traffic

h) 5GS

End Change