# 3GPP TSG SA WG5 Meeting #152                 	           S5-238010

# Chicago, US, 13-17 November 2023

# Source:	Huawei

# Title:	Define the structure of TSN Charging Information

# Document for:	Approval

Agenda Item:	7.4.6

1	Decision/action requested

Agree the structure of TSN Charging Information in TS 32.282.

2	References

[1] 		3GPP TS 32.282: "Charging management; Time-Sensitive Networking (TSN) charging".

3	Rationale

Define the TSN charging specific parameters, i.e. the TSN charging information.

4	Detailed proposal

Propose to incorporate the following change into the TS 32.282 [1].

First change

6.2	Time sensitive networking charging specific parameters

6.2.1	Definition of time sensitive networking charging information

6.2.1.1	General

The Charging Information parameter used for time sensitive networking charging is provided in the following clauses.

6.2.1.2	Definition of TSN Charging Information

Specific charging information for TSN charging is provided within the TSN Charging Information.

The detailed structure of the TSN Charging Information can be found in table 6.2.1.2-1.

Table 6.2.1.2-1: Structure of TSN Charging Information

| Information Element | Category | Description |
| --- | --- | --- |
| DNN | OC | This field holds data network name the AF belongs to. |
| S-NSSAI | OC | This field holds network slice information the AF belongs to. |
| Internal Group Identifier | OC | This field holds a group of UE represented by Internal Group Identifier(s) that the AF request is targeted for. |
| External Individual Id List | OC | This field holds one or more individual UE(s) that the AF request is targeted for. |
| 5GS Bridge Information | OC | This parameter holds the information for TSN 5GS bridge, defined in clause 6.2.1.3. |
| TSN QoS Information | OC | This parameter holds the information for TSN QoS, defined in clause 6.2.1.4. |
| TSC Assistance Information | OC | This parameter holds the information for TSC Assistance, defined in clause 6.2.1.5. |
| Time Synchronization Information | OC | This parameter holds the information for Time Synchronization, defined in clause 6.2.1.6. |

End of change