3GPP TSG-SA5 Meeting #152 	S5-238123

Chicago,US, 13-17 November 2023

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.105 | CR | Draft CR | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 Input to Draft CR 28.105 Modification on ML training initiated by producer |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile Com. Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | AIML_MGT |  |  |  |  |  | Date: |  |  | 2023-11-14 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | With respect to the REQ- ML_TRAIN-FUN-05, we found there is lack of description in the related clause 6.2.1.2.2 to elabarate the relationship between the threshold and re-training. This Draft CR is to supply those contents referring to TR 28.908, clause 5.1.4.2.1, with slight modifications. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The explaination related to threshold in requirements REQ- ML_TRAIN-FUN-05 has been added in clause 6.2.1.2.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No description related to threshold of subsequent requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision of S5-237927 |  |  |  |  |  |  |  |  |

First modified section

6.2	ML training phase

6.2.1	ML model training

6.2.1.2	Use cases

##### 6.2.1.2.2	ML training initiated by producer

The ML training or re-training may be initiated by the MLT MnS producer, for instance as a result of performance evaluation of the ML entity or based on feedback or new training data received from the consumer, or when new training data which are not from the consumer describing the new network status/events become available.

Therefore, there is a need to monitor the performance and/or the KPIs of the ML entity and use the thresholds that the MLT MnS consumer configured for the MLT MnS producer to trigger the training or re-training.

When the MLT MnS producer decides to start the ML training, the producer performs the followings:

-	selects the training data;

-	trains the ML entity using the selected training data;

-	provides the training results (including the identifier of the initially trained ML entity or the version number of the re-trained ML entity, training performance, etc.) to the MLT MnS consumer(s) who have subscribed to receive the ML training results.

6.2.1.3	Requirements for ML training

Table 6.2.1.3-1

| Requirement label | Description | Related use case(s) |
| --- | --- | --- |
| REQ-ML_TRAIN-FUN-01 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to request ML training. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-02 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the data sources containing the candidate training data for ML training. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-03 | The MLT MnS producer shall have a capability allowing the authorized MLT MnS consumer to specify the inference type of the ML entity to be trained. | ML training requested by consumer (clause 6.2.1.2.1) |
| REQ- ML_TRAIN-FUN-04 | The MLT MnS producer shall have a capability to provide the training result to the MLT MnS consumer. | ML training requested by consumer (clause 6.2.1.2.1), ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-05 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to configure the thresholds of the performance measurements and/or KPIs to trigger the re-training of an ML entity. (See Note) | ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-06 | The MLT MnS producer shall have a capability to provide the version number of the ML entity and the time when it is generated by ML re-training to the authorized MLT MnS consumer. | ML training requested by consumer (clause 6.2.1.2.1), /ML training initiated by producer (clause 6.2.1.2.2) |
| REQ- ML_TRAIN-FUN-07 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage the training process, including starting, suspending, or resuming the training process, and configuring the ML context for ML training. | ML training requested by consumer (clause 6.2.1.2.1), ML training initiated by producer (clause 6.2.1.2.2), ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-08 | The MLT MnS producer should have a capability to provide the grouping of ML entities to an authorized MLT MnS consumer to enable coordinated inference. | ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-09 | The MLT MnS producer should have a capability to allow an authorized MLT MnS consumer to request joint training of a group of ML entities. | ML entity joint training (clause 6.2.1.2.6) |
| REQ- ML_TRAIN-FUN-10 | The MLT MnS producer should have a capability to jointly train a group of ML entities and provide the training results to an authorized consumer. | ML entity joint training (clause 6.2.1.2.6) |
| REQ-ML_SELECT-01 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to discover the characteristics of available ML entitiesincluding the contexts under which each of the models associated with the entities were trained. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-02 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to select an ML entity. | ML models and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-03 | The MLT MnS producer shall have a capability to enable an authorized MLT MnS consumer to request for a model to be trained to satisfy the consumer's expectations. | ML training requested by consumer (clause 6.2.2.1), ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request for information and be informed about the available alternative ML entities of differing complexity and performance. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-05 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to request one of the known or available alternative models of differing complexity and performance to be used for inference. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_SELECT-06 | The 3GPP management system shall have a capability to provide a selected ML entity to the authorized MLT MnS consumer. | ML model and ML entity selection (clause 6.2.2.3) |
| REQ-ML_TRAIN- MGT-01 | The MLT MnS producer shall have a capability allowing an authorized consumer to manage and configure one or more requests for the specific ML training, e.g. to modify the characteristics of the request or to delete a request. | ML training requested by consumer (clause 6.2.2.1),Managing ML Training Processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-02 | The MLT MnS producer shall have a capability allowing an authorized MLT MnS consumer to manage and configure one or more training processes, e.g. to start, suspend or restart the training; or to adjust the training conditions and/or characteristics. | ML training requested by consumer (clause 6.2.2.1), Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-03 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer (e.g. the function/entity different from the function that generated a request for ML training) to request for a report on the outcomes of a specific training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-04 | 3GPP management system shall have a capability to enable an authorized MLT MnS consumer to define the reporting characteristics related to a specific training request or training instance. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_TRAIN- MGT-05 | 3GPP management system shall have a capability to enable the MLT function to report to any authorized MLT MnS consumer about specific ML training process and/or report about the outcomes of any such ML training process. | Managing ML training processes (clause 6.2.2.4) |
| REQ-ML_ERROR-01 | The 3GPP management system shall enable an authorized consumer of data services (e.g. an MLT function) to request from a producer of data services a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-02 | The 3GPP management system shall enable an authorized consumer of AI/ML decisions (e.g. a controller) to request ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by an AI/ML-inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-03 | The 3GPP management system shall enable a producer of data services (e.g. a gNB) to provide to an authorized consumer (e.g. an MLT function) a Value Quality Score of the data, which is the numerical value that represents the dependability/quality of a given observation and measurement type. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_ERROR-04 | The 3GPP management system shall enable a producer of ML decisions (e.g. an AI/ML inference function) to provide to an authorized consumer of ML decisions (e.g. a controller) an AI/ML decision confidence score which is the numerical value that represents the dependability/quality of a given decision generated by the inference function. | Handling errors in data and ML decisions (clause 6.2.2.5) |
| REQ-ML_VLD-01 | The MLT MnS producer should have a capability to validate the ML entities during the ML training process and report the performance of the ML entities on both the training data and validation data to the authorized consumer. | ML entity validation performance reporting (clause 6.2.1.2.7) |
| REQ-ML_VLD-02 | The MLT MnS producer should have a capability to report the ratio (in terms of quantity of data samples) of the training data and validation data used during the ML training and validation process. | ML entity validation performance reporting (clause 6.2.1.2.7) |
| NOTE: The performance measurements and KPIs are specific to each type (i.e., the inference type that the ML entity supports) of ML entity. |  |  |

End of modifications